Description and Data Collection

on Biofuel Technologies

Prepared For:

Danish Energy Agency

Amaliegade 44

1256 København, Denmark

Prepared By

(S&T)2 Consultants Inc.

11657 Summit Crescent

Delta, BC

Canada, V4E 2Z2

Date: April 1, 2018

Executive Summary

This biofuel technology catalogue presents technologies for generation and conversion of energy carriers. In particular it includes mostly fuels made from biomass and mostly liquid fuels although there is one exception to each characteristic. Methanol produced from electricity and “waste” carbon dioxide is a clean fuel rather than a biofuel and SNG (Synthetic Natural Gas) is a gaseous fuel.

The main purpose of the catalogue is to provide generalized data for analysis of energy systems, including economic scenario models and high-level energy planning.

The report includes both qualitative and quantitative information on each of the technologies considered. A consistent format is used for each technology but it must be remembered that the technologies are at different stages of development and thus the quality of some of the quantitative information is not the same for all technologies.

Most of the process are characterised by multiple inputs and multiple outputs, which include for example different fuels/feedstocks, electricity and process heat (recoverable or lost).

Upstream and downstream processes for the technologies are not included - the datasheets do not provide information on prices for fuels, environmental impact from fuel procurement, or the economic consequences of the substitution of fossil fuels with liquid fuels produced from biomass.

Ten technologies have been identified by the Danish Energy Agency for inclusion in the report. Some of the technologies can have multiple feedstocks so the feedstocks are combined when they behave similarly in the technology. For example, rapeseed oil and soybean oil are considered together as vegetable oil FAME (Fatty Acid Methyl Esters) but are separate from UCO (Used Cooking Oil) and animal fat FAME.

Table ES- 1Technologies Included

| Technology description to be made | Data tables to be made |
| --- | --- |
| Vegetable Oil FAME | Rapeseed and soybean methyl esters |
| UCO and Animal Fat FAME | Waste oil and animal fat methyl esters |
| HVO | Any feedstock |
| Jet Fuel from HVO process | Any feedstock |
| 1st Generation Ethanol | Wheat and corn |
| Pyrolysis oil | Fast Pyrolysis of biomass |
| Cellulosic Ethanol (2nd gen) | Straw |
| Gasification of biomass | Jet and diesel fuel from relevant biomass |
| Methanol from gasification of biomass | Methanol from relevant biomass |
| Methanol from electrofuel process | CO2 and H2 (considering the collection of CO2 from relevant source (atmosphere or power plant stack), not the production of H2) |
| SNG (no description of specific H production) | Methanation of CO2 in biogas (excluding H production) |
| Hydrothermal Liquefaction | Pyrolysis oil from relevant biomass |

Table of Contents

Executive Summary	i

Table of Contents	iii

List of Tables	x

List of Figures	xi

Glossary	xiii

1.	Introduction	1

1.1	Scope of Work	1

1.2	General Assumptions	2

1.3	Definitions	2

1.4	Technology Maturity	2

1.4.1	Learning Curves	3

1.5	Capital Costs	5

1.5.1	Capital Cost Approaches	5

1.5.2	Capital Cost Adjustments	7

1.6	Operating Costs	9

1.6.1	Fixed Operating and Maintenance Costs	10

1.6.2	Variable Operating and Maintenance Costs	10

1.7	Excess Heat	10

1.8	Plant Life	10

1.9	References	10

2.	Vegetable Oil FAME	12

2.1	Qualitative Description	12

2.1.1	Contact Information	12

2.1.2	Brief Technology Description	12

2.1.3	Input	13

2.1.4	Output	13

2.1.5	Energy Balance	13

2.1.6	Typical Capacities	14

2.1.7	Regulation Ability	14

2.1.8	Space Requirements	14

2.1.9	Advantages/Disadvantages	14

2.1.10	Environment	15

2.1.11	Research and Development Perspective	15

2.1.12	Examples of Market Standard Technology	15

2.1.13	Predication of Performance and Cost	15

2.1.14	Uncertainty	16

2.1.15	Additional Remarks	16

2.2	Quantitative Description	16

2.2.1	Energy/Technical Data	17

2.2.1.1	Typical Plant Size	17

2.2.1.2	Input and Output	17

2.2.1.3	Forced and Planned Outage	17

2.2.1.4	Technical Lifetime	18

2.2.1.5	Construction Time	18

2.2.2	Financial Data	18

2.2.2.1	Investment Costs	18

2.2.2.2	Operating and Maintenance Costs	18

2.2.2.3	Start-up Costs	19

2.2.3	Technology Specific Data	19

2.3	Summary	19

2.4	References	21

3.	UCO and Animal Fat FAME	22

3.1	Qualitative Description	22

3.1.1	Contact Information	22

3.1.2	Brief Technology Description	22

3.1.3	Input	23

3.1.4	Output	23

3.1.5	Energy Balance	23

3.1.6	Typical Capacities	23

3.1.7	Regulation Ability	23

3.1.8	Space Requirements	24

3.1.9	Advantages/Disadvantages	24

3.1.10	Environment	24

3.1.11	Research and Development Perspective	24

3.1.12	Examples of Market Standard Technology	25

3.1.13	Predication of Performance and Cost	25

3.1.14	Uncertainty	25

3.1.15	Additional Remarks	25

3.2	Quantitative Description	25

3.2.1	Energy/Technical Data	26

3.2.1.1	Typical Plant Size	26

3.2.1.2	Input and Output	26

3.2.1.3	Forced and Planned Outage	27

3.2.1.4	Technical Lifetime	27

3.2.1.5	Construction Time	27

3.2.2	Financial Data	27

3.2.2.1	Investment Costs	27

3.2.2.2	Operating and Maintenance Costs	27

3.2.2.3	Start-up Costs	28

3.2.3	Technology Specific Data	28

3.2.4	Summary	28

3.3	References	30

4.	Hydrogenated Vegetable Oil	31

4.1	Qualitative Description	31

4.1.1	Contact Information	31

4.1.2	Brief Technology Description	31

4.1.3	Input	31

4.1.4	Output	32

4.1.5	Energy Balance	32

4.1.6	Typical Capacities	32

4.1.7	Regulation Ability	32

4.1.8	Space Requirements	32

4.1.9	Advantages/Disadvantages	33

4.1.10	Environment	33

4.1.11	Research and Development Perspective	33

4.1.12	Examples of Market Standard Technology	33

4.1.13	Predication of Performance and Cost	34

4.1.14	Uncertainty	34

4.1.15	Additional Remarks	34

4.2	Quantitative Description	35

4.2.1	Energy/Technical Data	35

4.2.1.1	Typical Plant Size	35

4.2.1.2	Input and Output	35

4.2.1.3	Forced and Planned Outage	36

4.2.1.4	Technical Lifetime	36

4.2.1.5	Construction Time	36

4.2.2	Financial Data	36

4.2.2.1	Investment Costs	37

4.2.2.2	Operating and Maintenance Costs	37

4.2.2.3	Start-up Costs	37

4.2.3	Technology Specific Data	37

4.2.4	Summary	38

4.3	References	39

5.	HVO Jet Fuel	41

5.1	Qualitative Description	41

5.1.1	Contact Information	41

5.1.2	Brief Technology Description	41

5.1.3	Input	42

5.1.4	Output	42

5.1.5	Energy Balance	42

5.1.6	Typical Capacities	43

5.1.7	Regulation Ability	44

5.1.8	Space Requirements	44

5.1.9	Advantages/Disadvantages	44

5.1.10	Environment	44

5.1.11	Research and Development Perspective	44

5.1.12	Examples of Market Standard Technology	44

5.1.13	Predication of Performance and Cost	45

5.1.14	Uncertainty	45

5.1.15	Additional Remarks	45

5.2	Quantitative Description	46

5.2.1	Energy/Technical Data	46

5.2.1.1	Typical Plant Size	46

5.2.1.2	Input and Output	46

5.2.1.3	Forced and Planned Outage	46

5.2.1.4	Technical Lifetime	46

5.2.1.5	Construction Time	46

5.2.2	Financial Data	47

5.2.2.1	Investment Costs	47

5.2.2.2	Operating and Maintenance Costs	47

5.2.2.3	Start-up Costs	47

5.2.3	Technology Specific Data	47

5.2.4	Summary	47

5.3	References	49

6.	1st Generation Ethanol	50

6.1	Qualitative Description	50

6.1.1	Contact Information	50

6.1.2	Brief Technology Description	50

6.1.3	Input	50

6.1.4	Output	51

6.1.5	Energy Balance	51

6.1.6	Typical Capacities	51

6.1.7	Regulation Ability	52

6.1.8	Space Requirements	52

6.1.9	Advantages/Disadvantages	52

6.1.10	Environment	52

6.1.11	Research and Development Perspective	52

6.1.12	Examples of Market Standard Technology	52

6.1.13	Predication of Performance and Cost	53

6.1.14	Uncertainty	53

6.1.15	Additional Remarks	53

6.2	Quantitative Description	54

6.2.1	Energy/Technical Data	54

6.2.1.1	Typical Plant Size	54

6.2.1.2	Input and Output	55

6.2.1.3	Forced and Planned Outage	56

6.2.1.4	Technical Lifetime	56

6.2.1.5	Construction Time	56

6.2.2	Financial Data	56

6.2.2.1	Investment Costs	56

6.2.2.2	Operating and Maintenance Costs	57

6.2.2.3	Start-up Costs	57

6.2.3	Technology Specific Data	57

6.3	Summary	58

6.4	References	59

7.	Pyrolysis Oils	60

7.1	Qualitative Description	60

7.1.1	Contact Information	60

7.1.2	Brief Technology Description	60

7.1.3	Input	61

7.1.4	Output	61

7.1.5	Energy Balance	61

7.1.6	Typical Capacities	62

7.1.7	Regulation Ability	62

7.1.8	Space Requirements	63

7.1.9	Advantages/Disadvantages	63

7.1.10	Environment	64

7.1.11	Research and Development Perspective	64

7.1.12	Examples of Market Standard Technology	64

7.1.13	Predication of Performance and Cost	64

7.1.14	Uncertainty	65

7.1.15	Additional Remarks	65

7.2	Quantitative Description	65

7.2.1	Energy/Technical Data	65

7.2.1.1	Typical Plant Size	65

7.2.1.2	Input and Output	65

7.2.1.3	Forced and Planned Outage	66

7.2.1.4	Technical Lifetime	66

7.2.1.5	Construction Time	66

7.2.2	Financial Data	66

7.2.2.1	Investment Costs	66

7.2.2.2	Operating and Maintenance Costs	66

7.2.2.3	Start-up Costs	67

7.2.3	Technology Specific Data	67

7.3	Summary	67

7.4	References	68

8.	Cellulosic Ethanol	70

8.1	Qualitative Description	70

8.1.1	Contact Information	70

8.1.2	Brief Technology Description	70

8.1.3	Input	71

8.1.4	Output	71

8.1.5	Energy Balance	71

8.1.6	Typical Capacities	73

8.1.7	Regulation Ability	73

8.1.8	Space Requirements	73

8.1.9	Advantages/Disadvantages	74

8.1.10	Environment	74

8.1.11	Research and Development Perspective	74

8.1.12	Examples of Market Standard Technology	74

8.1.13	Predication of Performance and Cost	75

8.1.14	Uncertainty	75

8.1.15	Additional Remarks	75

8.2	Quantitative Description	75

8.2.1	Energy/Technical Data	76

8.2.1.1	Typical Plant Size	76

8.2.1.2	Input and Output	76

8.2.1.3	Forced and Planned Outage	77

8.2.1.4	Technical Lifetime	78

8.2.1.5	Construction Time	78

8.2.2	Financial Data	78

8.2.2.1	Investment Costs	78

8.2.2.2	Operating and Maintenance Costs	78

8.2.2.3	Start-up Costs	79

8.2.3	Technology Specific Data	79

8.3	Summary	79

8.4	References	80

9.	Biomass Gasification for Diesel and Jet Fuel	82

9.1	Qualitative Description	82

9.1.1	Contact Information	82

9.1.2	Brief Technology Description	82

9.1.3	Input	82

9.1.4	Output	83

9.1.5	Energy Balance	83

9.1.6	Typical Capacities	84

9.1.7	Regulation Ability	84

9.1.8	Space Requirements	84

9.1.9	Advantages/Disadvantages	85

9.1.10	Environment	85

9.1.11	Research and Development Perspective	85

9.1.12	Examples of Market Standard Technology	86

9.1.13	Predication of Performance and Cost	86

9.1.14	Uncertainty	86

9.1.15	Additional Remarks	86

9.2	Quantitative Description	87

9.2.1	Energy/Technical Data	87

9.2.1.1	Typical Plant Size	88

9.2.1.2	Input and Output	88

9.2.1.3	Forced and Planned Outage	88

9.2.1.4	Technical Lifetime	88

9.2.1.5	Construction Time	88

9.2.2	Financial Data	88

9.2.2.1	Investment Costs	89

9.2.2.2	Operating and Maintenance Costs	89

9.2.2.3	Start-up Costs	89

9.2.3	Technology Specific Data	89

9.3	Summary	89

9.4	References	90

10.	Methanol from Biomass Gasification	93

10.1	Qualitative Description	93

10.1.1	Contact Information	93

10.1.2	Brief Technology Description	93

10.1.3	Input	94

10.1.4	Output	94

10.1.5	Energy Balance	94

10.1.6	Typical Capacities	96

10.1.7	Regulation Ability	96

10.1.8	Space Requirements	96

10.1.9	Advantages/Disadvantages	96

10.1.10	Environment	97

10.1.11	Research and Development Perspective	97

10.1.12	Examples of Market Standard Technology	97

10.1.13	Predication of Performance and Cost	98

10.1.14	Uncertainty	98

10.1.15	Additional Remarks	98

10.2	Quantitative Description	98

10.2.1	Energy/Technical Data	98

10.2.1.1	Typical Plant Size	98

10.2.1.2	Input and Output	99

10.2.1.3	Forced and Planned Outage	99

10.2.1.1	Technical Lifetime	99

10.2.1.2	Construction Time	99

10.2.2	Financial Data	99

10.2.2.1	Investment Costs	99

10.2.2.2	Operating and Maintenance Costs	99

10.2.2.3	Start-up Costs	99

10.2.3	Technology Specific Data	100

10.3	Summary	100

10.4	References	101

11.	Methanol from Power	102

11.1	Qualitative Description	102

11.1.1	Contact Information	102

11.1.2	Brief Technology Description	102

11.1.3	Input	103

11.1.4	Output	103

11.1.5	Energy Balance	103

11.1.6	Typical Capacities	103

11.1.7	Regulation Ability	104

11.1.8	Space Requirements	104

11.1.9	Advantages/Disadvantages	104

11.1.10	Environment	104

11.1.11	Research and Development Perspective	105

11.1.12	Examples of Market Standard Technology	105

11.1.13	Predication of Performance and Cost	105

11.1.14	Uncertainty	105

11.1.15	Additional Remarks	105

11.2	Quantitative Description	106

11.2.1	Energy/Technical Data	106

11.2.1.1	Typical Plant Size	106

11.2.1.2	Input and Output	106

11.2.1.3	Forced and Planned Outage	106

11.2.1.4	Technical Lifetime	106

11.2.1.5	Construction Time	107

11.2.2	Financial Data	107

11.2.2.1	Investment Costs	107

11.2.2.2	Operating and Maintenance Costs	107

11.2.2.3	Start-up Costs	107

11.2.3	Technology Specific Data	107

11.3	Summary	107

11.4	References	109

12.	SNG	110

12.1	Qualitative Description	110

12.2	Contact Information	110

12.2.1	Brief Technology Description	110

12.2.2	Input and Output	110

12.2.3	Energy Balance	110

12.2.4	Typical Capacities	111

12.2.5	Regulation Ability	111

12.2.6	Space Requirements	111

12.2.7	Advantages/Disadvantages	112

12.2.8	Environment	112

12.2.9	Research and Development Perspective	112

12.2.10	Examples of Market Standard Technology	112

12.2.11	Predication of Performance and Cost	113

12.2.12	Uncertainty	113

12.2.13	Additional Remarks	113

12.3	Quantitative Description	115

12.3.1	Energy/Technical Data	115

12.3.1.1	Typical Plant Size	115

12.3.1.2	Input and Output	116

12.3.1.3	Forced and Planned Outage	116

12.3.1.4	Technical Lifetime	116

12.3.1.5	Construction Time	116

12.3.2	Financial Data	116

12.3.2.1	Investment Costs	116

12.3.2.2	Operating and Maintenance Costs	116

12.3.2.3	Start-up Costs	117

12.3.3	Technology Specific Data	117

12.4	Summary	117

12.5	References	119

13.	Hydrothermal Liquefaction	120

13.1	Qualitative Description	120

13.1.1	Contact Information	120

13.1.2	Brief Technology Description	120

13.1.3	Input	121

13.1.4	Output	121

13.1.5	Energy Balance	121

13.1.6	Typical Capacities	122

13.1.7	Regulation Ability	122

13.1.8	Space Requirements	122

13.1.9	Advantages/Disadvantages	122

13.1.10	Environment	123

13.1.11	Research and Development Perspective	123

13.1.12	Examples of Market Standard Technology	123

13.1.13	Predication of Performance and Cost	123

13.1.14	Uncertainty	123

13.1.15	Additional Remarks	124

13.2	Quantitative Description	124

13.2.1	Energy/Technical Data	125

13.2.1.1	Typical Plant Size	125

13.2.1.2	Input and Output	125

13.2.1.3	Forced and Planned Outage	126

13.2.1.4	Technical Lifetime	126

13.2.1.5	Construction Time	126

13.2.2	Financial Data	126

13.2.2.1	Investment Costs	126

13.2.2.2	Operating and Maintenance Costs	127

13.2.2.3	Start-up Costs	127

13.2.3	Technology Specific Data	127

13.3	Summary	128

13.4	References	129

List of Tables

Table 1-1	Technologies Included	1

Table 1-2	Types of Cost Estimates	6

Table 1-3	Cost Growth Factors	7

Table 1-4	CEPCI	7

Table 1-5	Capital Cost Location Factors	8

Table 1-6	Energy Costs 2015	9

Table 2-1	Mass and Energy Flows	17

Table 2-2	Production Costs Vegetable oil FAME	19

Table 2-3	Vegetable Oil Biodiesel Properties	19

Table 2-4	Vegetable Oil FAME Datasheet	20

Table 3-1	Mass and Energy Flows	26

Table 3-2	UCO Biodiesel Operating Costs	28

Table 3-3	UCO and Animal Fat Biodiesel Properties	28

Table 3-4	UCO and Animal Fat FAME Datasheet	29

Table 4-1	Mass and Energy Flows HVO	35

Table 4-2	HVO Operating Costs	37

Table 4-3	HVO Properties	38

Table 4-4	HVO Datasheet	38

Table 5-1	Mass and Energy Flows HVO Jet	46

Table 5-2	HVO Jet Properties	47

Table 5-3	HVO Jet Fuel Datasheet	48

Table 6-1	European Ethanol Plants	55

Table 6-2	Corn Ethanol Inputs and Outputs	55

Table 6-3	Capital Costs	57

Table 6-4	Production Costs 1st Generation Ethanol	57

Table 6-5	Ethanol Properties	57

Table 6-6	1st Generation Ethanol Datasheet	58

Table 13-1	Typical Plant Sized	65

Table 13-2	Typical Input and Output	66

Table 13-3	Fast Pyrolysis Bio-Oil Properties	67

Table 13-4	Fast Pyrolysis Bio-oil Datasheet	68

Table 7-1	Cellulosic Ethanol Plants	75

Table 7-2	Cellulosic Ethanol Inputs and Outputs	77

Table 7-3	Capital Costs	78

Table 7-4	Operating and Maintenance Costs	79

Table 7-5	Cellulosic Ethanol Datasheet	80

Table 8-1	European Biomass Gasification Plants	84

Table 8-2	Woody Biomass Production and Consumption for Energy	85

Table 8-3	Product Distributions – Shell SMDS	87

Table 8-4	Inputs and Outputs	88

Table 8-5	Operating and Maintenance Costs	89

Table 8-6	Typical Fuel Properties	89

Table 8-7	Biomass Gasification Datasheet	90

Table 9-1	Inputs and Outputs	99

Table 9-2	Methanol Properties	100

Table 9-3	Bio Methanol Datasheet	100

Table 10-1	Electrolysis Systems	105

Table 10-2	Mass and Energy Flows	106

Table 10-3	Capital Costs	107

Table 10-4	Methanol from Power Datasheet	108

Table 11-1	SNG Inputs and Outputs	116

Table 11-2	SNG Datasheet	118

Table 12-1 	Hydrotreated HTL Properties	124

Table 12-2	Typical Plant Sized	125

Table 12-3	Typical Input and Output	125

Table 12-4	Capital Cost Comparison	126

Table 12-5	Operating and Maintenance Costs	127

Table 12-6	Operating and Maintenance Costs	127

Table 12-7	HTL Bio-Oil Properties	128

Table 12-8	Hydrothermal Liquefaction Datasheet	128

List of Figures

Figure 1-1	Technology Development Phases	3

Figure 1-2 	Photovoltaic Experience Curve	4

Figure 1-3	Distribution of Progress Ratios for 108 Case Studies in the Manufacturing Sector	5

Figure 1-4	Learning Curves with Various Learning Rates	6

Figure 1-5	Impact of Starch Ethanol Plant Size on Capital Costs	8

Figure 1-6	Impact of FAME Plant Size on Capital Costs	9

Figure 2-1	Biodiesel Production Process	13

Figure 2-2	Vegetable Oil FAME Energy Balance	14

Figure 2-3	World Biodiesel Production	16

Figure 2-4	Basic Biodiesel Reactions	17

Figure 3-1	Two Step Biodiesel Production Process	22

Figure 3-2	UCO and Animal Fat FAME Energy Balance	23

Figure 3-3	Daka FAME Plant	24

Figure 3-4	Esterification Reaction	26

Figure 4-1	HVO Production Process	31

Figure 4-2	HVO Energy Balance	32

Figure 4-3	HVO Selling Prices vs Biodiesel and Fossil Diesel	34

Figure 4-4	HVO Chemistry	35

Figure 4-5	Quarterly Performance – Neste HVO	36

Figure 5-1	HVO Diesel vs Jet Process	41

Figure 5-2	HVO Jet Fuel Energy Balance	42

Figure 5-3	Comparison of Product Yields	43

Figure 5-4	Alternative HVO Jet Energy Balance	43

Figure 5-5	Renewable Diesel Distillation Curves	45

Figure 6-1	1st Generation Ethanol Production Process	50

Figure 6-2	1st Generation Ethanol Energy Balance	51

Figure 6-3	World Starch Ethanol Production	53

Figure 13-1	Fast Pyrolysis Process Flow	61

Figure 13-2	Fast Pyrolysis Oil Energy Balance	62

Figure 13-3	Typical product Yields vs. Temperature	63

Figure 7-1	Cellulosic Ethanol Process	71

Figure 7-2	Cellulosic Ethanol Energy Balance	72

Figure 7-3	Cellulosic Ethanol Energy Balance with Chemicals	72

Figure 7-4	Inbicon Cellulosic Ethanol Energy Balance	73

Figure 7-5	Cellulosic Ethanol Production Rates	77

Figure 8-1	Biomass to Diesel and Jet Process	82

Figure 8-2	Biomass to Diesel and Jet Energy Balance	83

Figure 8-3	Selectivity vs. Yield	87

Figure 9-1	Biomass to Methanol Process	93

Figure 9-2	Bio Methanol Energy Balance	94

Figure 9-3	GreenSynFuels Traditional Methanol Plant	95

Figure 9-4	Optimized Biomass to Methanol Plant	95

Figure 9-5	Hybrid Biomass to Methanol Plant	96

Figure 10-1	Methanol to Power Process	102

Figure 10-2	Methanol from Power Energy Balance	103

Figure 11-1	SNG Process	110

Figure 11-2	SNG Energy Balance	111

Figure 11-3	SNG Methanation Plant	114

Figure 11-4	Haldor Topsøe SNG Pilot Plant	115

Figure 11-5	Gas Composition Prior to Final Clean-up	117

Figure 12-1	Hydrothermal Liquefaction Process Flow	121

Figure 12-2	Hydrothermal Liquefaction Energy Balance	122

Figure 12-3	Major Chemical Reactions in the Steeper Energy Process	125

Glossary

Bbls		Barrels (159 litres)

BTL		Biomass to Liquids

CEPCI		Chemical Engineering Plant Cost Index

CH4		Methane

CO		Carbon monoxide

CO2		Carbon dioxide

cP		Centipoise

DDG		Distillers’ Dried Grains

DME		Dimethyl Ether

EU		European Union

FAME     	Fatty Acid Methyl Ester

FFA		Free Fatty Acid

FT		Fischer Tropsch

GJ		Giga Joule (1,000 MJ)

GTL		Gas to Liquids

H2		Hydrogen

H2O		Water

Ha		Hectare

HFO		Heavy Fuel Oil

HHV		Higher heating value (also known as gross heating value)

HTL		Hydrothermal Liquefaction

HVO		Hydrogenated Vegetable Oil

km		Kilometre

kW		Kilowatt

kWh		Kilowatt hour

LGE		Litres Gasoline Equivalent

LHV		Lower heating value (also known as net heating value)

Lpy		Litre per year

M2		Square metre

MeOH		Methanol

MJ		Mega Joule

MW		Mega watt

MWh		Megawatt hour

NREL		National Renewable Energy Laboratory

odt		Oven dry tonnes

PJ		Peta Joule (1,000,000 GJ)

SNG		Synthetic Natural Gas

tpd		Tonne per day

tph		Tonne  per hour

tpy		Tonne per year

UCO		Used Cooking Oil

Introduction

This biofuel technology catalogue presents technologies for generation and conversion of energy carriers. In particular it includes mostly fuels made from biomass and mostly liquid fuels although there is one exception to each characteristic. Methanol produced from electricity and “waste” carbon dioxide is a clean fuel rather than a biofuel and SNG (Synthetic Natural Gas) is a gaseous fuel.

The main purpose of the catalogue is to provide generalized data for analysis of energy systems, including economic scenario models and high-level energy planning.

The report includes both qualitative and quantitative information on each of the technologies considered. A consistent format is used for each technology but it must be remembered that the technologies are at different stages of development and thus the quality of some of the quantitative information is not the same for all technologies.

Most of the process are characterised by multiple inputs and multiple outputs, which include for example different fuels/feedstocks, electricity and process heat (recoverable or lost).

Upstream and downstream processes for the technologies are not included - the datasheets do not provide information on prices for fuels, environmental impact from fuel procurement, or the economic consequences of the substitution of fossil fuels with liquid fuels produced from biomass.

Scope of Work

Ten technologies have been identified by the Danish Energy Agency for inclusion in the report. Some of the technologies can have multiple feedstocks so the feedstocks are combined when they behave similarly in the technology. For example, rapeseed oil and soybean oil are considered together as vegetable oil FAME (Fatty Acid Methyl Esters) but are separate from UCO (Used Cooking Oil) and animal fat FAME.

Table 1-1	Technologies Included

| Technology description to be made | Data tables to be made |
| --- | --- |
| Vegetable Oil FAME | Rapeseed and soybean methyl esters |
| UCO and Animal Fat FAME | Waste oil and animal fat methyl esters |
| HVO | Any feedstock |
| Jet Fuel from HVO process | Any feedstock |
| 1st Generation Ethanol | Wheat and corn |
| Pyrolysis oil | Fast Pyrolysis of biomass |
| Cellulosic Ethanol (2nd gen) | Straw |
| Gasification of biomass | Jet and diesel fuel from relevant biomass |
| Methanol from gasification of biomass | Methanol from relevant biomass |
| Methanol from electrofuel process | CO2 and H2 (considering the collection of CO2 from relevant source (atmosphere or power plant stack), not the production of H2) |
| SNG (no description of specific H production) | Methanation of CO2 in biogas (excluding H production) |
| Hydrothermal Liquefaction | Pyrolysis oil from relevant biomass |

The first six technologies are commercial processes that are operated in Europe and other parts of the world. The last 6 technologies are not yet commercially operated. Cellulosic ethanol, methanol from the electro fuel process, and SNG have large scale demonstration projects in operation and are generating interest from commercial operators. The other three processes have been demonstrated at the pilot scale.

General Assumptions

The data presented in this catalogue is based on some general assumptions, mainly with regards to the utilization and start-ups of plants and technologies.

On the one hand, plants for biofuel production and production/upgrade of biogas and syngas are assumed to be designed and operated on a continuous basis along the year, except for maintenance and outages. Therefore, they feature a high number of full load hours (around 8,000 h/y) and a reduced number of start-ups (5 or less per year).

On the other hand, electrolysers are assumed to be designed and operated for approximately 4,000 full load hours annually. In particular, use the advantage of lower power prices by producing e.g. in hours of high renewable energy production (similarly to heat pumps). The assumed number of start-ups and consequent shut-downs for electrolysers, unless otherwise stated, is 50 per year. Several of the technologies, power to methanol, and methanation might use an electrolyzer and while electrolyzers can be follow a load; the ability of the methane and methanol synthesis stages has not yet been proven in commercial applications.

Definitions

A number of definitions and conventions have been used in the analysis. These include:

The latent heat of vaporization is the heat absorbed when a substance changes phase from liquid to gas.

The lower heating value (also known as net calorific value) of a fuel is defined as the amount of heat released by combusting a specified quantity (initially at 25°C) and returning the temperature of the combustion products to 150°C, which assumes the latent heat of vaporization of water in the reaction products is not recovered. The LHV are the useful calorific values in boiler combustion plants and are frequently used in Europe. All energy data in the report is presented on a LHV basis unless otherwise noted.

Using the LHV for efficiency definition, a condensing boiler can achieve a thermal efficiency of more than 100%, because the process recovers part of the heat of vaporization.

The higher heating value (also known as gross calorific value or gross energy) of a fuel is defined as the amount of heat released by a specified quantity (initially at 25°C) once it is combusted and the products have returned to a temperature of 25°C, which takes into account the latent heat of vaporization of water in the combustion products.

When using HHV for thermal efficiency definition, the thermodynamic limit of 100% cannot be exceeded.

Technology Maturity

The potential for improving technologies is linked to the level of technological maturity. The technologies are categorized within one of the following four levels of technological maturity.

Category 1. Technologies that are still in the research and development phase. The uncertainty related to price and performance today and in the future is highly significant (e.g. wave energy converters, solid oxide fuel cells).

Category 2. Technologies in the pioneer phase. The technology has been proven to work through demonstration facilities or semi-commercial plants. Due to the limited application, the price and performance is still attached with high uncertainty, since development and customization is still needed. The technology still has a significant development potential (e.g. gasification of biomass).

Category 3. Commercial technologies with moderate deployment. The price and performance of the technology today is well known. These technologies are deemed to have a certain development potential and therefore there is a considerable level of uncertainty related to future price and performance (e.g. offshore wind turbines)

Category 4. Commercial technologies, with large deployment. The price and performance of the technology today is well known and normally only incremental improvements would be expected. Therefore, the future price and performance may also be projected with a relatively high level of certainty (e.g. coal power, gas turbine).

The capital and production costs can be expected to decline as a technology moves from Category 1 to 4 as shown in the following figure.

Figure 1-1	Technology Development Phases

Learning Curves

There is actually a significant body of knowledge related to the rate that costs decline as production experience is gained. The learning curve phenomenon was first reported by Wright in the literature in 1936 [1]. The phenomenon which Wright observed was that as the quantity of units manufactured doubles, the number of direct labor hours' it takes to produce an individual unit decreases at a uniform rate. The uniform rate (i.e., 90 percent, 80 percent, 70 percent, etc.) of learning is peculiar to the manufacturing process being observed.

Since that first observation there has been an accumulation of empirical evidence that deploying new technologies in competitive markets leads to technology learning, in which the cost of using a new technology falls and its technical performance improves as sales and operational experience accumulate. Experience and learning curves, which summarise the paths of falling technology costs and improving technical performance respectively, provide a robust and simple tool for analysing technology learning.

The shape of the curves indicates that improvements follow a simple power law. This implies that relative improvements in price and technical performance remain the same over each doubling of cumulative sales or operational experience. As an example, the following figure shows that the prices of photovoltaic modules declined by more than 20 percent as each doubling of sales occurred during the period between 1976 and 1992 [2]. Furthermore, the relationship remains the same over three orders of magnitude of sales.

Figure 1-2 	Photovoltaic Experience Curve

The experience curve is described mathematically as:

Price at year t = P0 * X -E

where:

P0 	=	the price at one unit of cumulative production.

X 	=	the cumulative production of energy, sales, or a similar surrogate for the experience gained with the technology in year t.

E 	=	the experience parameter which characterizes the slope of the trend line when plotted on a log-log scale.

Progress in the reduction in energy price as technology travels down the experience curve is commonly reported in terms of the progress ratio, or PR. The PR is the energy price after double the cumulative production, as a fraction of the starting price at any point on the line and is calculated from the experience parameter (E) using the equation: PR = 2-E.

The Progress Ratio is often presented as a percentage and in the PV case shown above; the progress ratio was 82%. It can also be reported as a fraction, such as 0.82 in this case. That is the costs would decline by 18% for every doubling in production. To be conservative we have used a 15% reduction in costs for a doubling of production for technologies where no other information is available.

The straight line captures a very important feature of the experience curve. Anywhere along the line, an increase by a fixed percentage of the cumulative production gives a consistent percentage reduction in price. This means that for technologies having the same progress ratio, the same absolute increase in installed capacity will yield a greater cost decrease for young technologies (i.e., they learn faster) than old technologies. This also means that the same absolute increase in cumulative production will have more a dramatic effect at the beginning of a technology’s deployment than it will later on. For well-established technology, such as oil refineries using conventional technology, the volume required to double cumulative sales may be of the order of 100 million bbls/day, so the experience effect will hardly be noticeable in stable markets.

There is a significant amount of information on experience curves in the literature for many different technologies. Figure 1-3 shows the distribution of Progress Ratios for 108 case studies for a range of different products in the manufacturing sector [2]. The average value of the progress ratio over these case studies was 82%. The consistency of the Progress Ratios over so many different technologies and products means that the approach can be used confidently, with some care, as a policy analysis tool for a range of technologies.

Figure 1-3	Distribution of Progress Ratios for 108 Case Studies in the Manufacturing Sector

Capital Costs

Capital Cost Approaches

The technologies reviewed in the report cover the full range of technology maturities, which can pose challenges when one technology is compared to another. There are three types of capital cost data available for the technologies, values from commercial experience, estimates for pioneering plants, and estimates based on the nth plant for new technologies. The definition of the nth plant is somewhat arbitrary, although it is often taken as the fifth or higher plant, although the actual number can vary with the expected learning rate [3]. The methodology for determining the nth plant cost estimate is usually developed by applying the learning curve concept described previously.

The difference between the cost of a pioneering plant and the nth plant varies with the learning rate. Technologies that have high learning rates will have larger differences in the costs. Furthermore, more plants may be required to be built before the nth plant estimate is reached as shown in the following figure [9].

Figure 1-4	Learning Curves with Various Learning Rates

The classification of the types of cost estimates found for the technologies reviewed in this report are summarized in the following table.

Table 1-2	Types of Cost Estimates

| Technology | Commercial Plant Data | Nth plant estimate | Pioneering Plant cost estimate |
| --- | --- | --- | --- |
| Vegetable Oil FAME | x |  |  |
| UCO and Animal Fat FAME | x |  |  |
| HVO | x |  |  |
| Jet Fuel from HVO process | x |  |  |
| 1st Generation Ethanol | x |  |  |
| Cellulosic Ethanol (2nd gen) |  | x | x |
| Gasification of biomass |  | x |  |
| Methanol from gasification of biomass |  | x | x |
| Methanol from electrofuel process |  | x | x |
| SNG (no description of specific H production) |  | x | x |
| Hydrothermal Liquefaction |  | x |  |

Pioneering plant cost estimates will be higher than the estimates for the nth plant but they can be used for the 2020 cost estimates. Nth plant cost estimates might be appropriate for costs for 2030 or 2050, depending on the rate of roll out of the technology. Commercial plant data is appropriate for the 2015 and 2020 cost estimates but there is the potential for reduced capital costs over time if more plants are built. The rate of cost reductions will depend on the rate of construction of new facilities.

De Jong et al [4] reported some “cost growth factors” for some of the emerging technologies. The cost growth factor is the ratio of the total capital investment of the nth plant to the total capital investment of the pioneer plant. They also reported the expected plant availability in year one, which has a large impact on the initial operating and maintenance costs. The results are shown in the following table.

Table 1-3	Cost Growth Factors

| Technology | Cost Growth Factor | Plant Availability in Year 1 |
| --- | --- | --- |
| Fischer Tropsch (biomass) | 0.45 | 32% |
| HTL (biomass) | 0.40 | 22% |
| Pyrolysis (biomass) | 0.37 | 22% |
| HVO (UCO) | 0.86 | 86% |

This information suggests that there will be significant room for reduction in capital costs for the biomass plants and only a modest reduction over time in the HVO plants.

The number of FAME and 1st generation ethanol plants expanded rapidly between 2000 and 2010, but few new plants have been built in the past five years. Unless there is significant new plant construction in the next decade there is likely very little capital cost reduction potential for these technologies by 2030.

Capital Cost Adjustments

Capital cost information in the literature can be presented with different system boundaries (what is included in the estimate and what is not), for different locations (construction costs are different in different locations), they can be based on plants that have been built and those that are still in the development stage, and for different years. Providing consistent information can be problematic across the technologies.

For commercial technologies the most recent available data on plant construction has been used. The data is adjusted where possible so that land costs are not included. Where the available data is for a plant expansion, the costs have been increased by 20 % to account for assumed synergies with the existing facility.

Capital costs are presented in 2015 Euros. To adjust the data to 2015 the Chemical Engineering Plant Cost Index (CEPCI) is used. The data for the past decade is shown in the following table [5]. Note that only 2009 had a lower index value than 2015. There has been relatively little change in capital costs since 2008.

Table 1-4	CEPCI

| Year | Index | Relative to 2015 |
| --- | --- | --- |
| 2015 | 556.8 | 1.000 |
| 2014 | 576.1 | 1.035 |
| 2013 | 567.3 | 1.019 |
| 2012 | 584.6 | 1.050 |
| 2011 | 585.7 | 1.052 |
| 2010 | 550.8 | 0.989 |
| 2009 | 521.9 | 0.937 |
| 2008 | 575.4 | 1.033 |

The CEPCI is valid for US construction costs. It is also necessary to have a location factor to adjust the capital costs. Location factors for the building sector are published annually by a number of organizations but much less information is available for location factors for the processing industries. In a report on the costs of carbon capture and storage costs [6] reported the relative materials, equipment and labour costs for various regions of the world. The ratios between the US and several other regions are shown in the following table. The factor used for the analysis is 60% equipment and 20% each for the materials and labour.

Table 1-5	Capital Cost Location Factors

| Location | Equipment | Materials | Labour | Used for analysis |
| --- | --- | --- | --- | --- |
| United States | 1.00 | 1.00 | 1.00 | 1.00 |
| Euro region (Germany) | 1.19 | 1.16 | 1.33 | 1.21 |
| Eastern Europe (Poland) | 1.01 | 0.81 | 0.79 | 0.93 |
| Brazil | 1.16 | 1.16 | 0.97 | 1.12 |
| China | 0.81 | 0.81 | 0.05 | 0.66 |
| India | 1.27 | 1.11 | 0.26 | 1.04 |

A conversion rate of 0.85 has been used to convert US dollars to Euros. Where capital cost data came from other than US locations, the average 2017 exchange rate to Euros has been used.

A factor of 1.2 has been used to convert plant expansion capital costs to greenfield capital costs. This is obviously a factor that will vary depending on the site but the factor should be representative of a typical expansion.

The information of capital costs may not be for the same size plant as had been chosen for the data sheets. While an exponential factor of 0.6 is often used for process industries to estimate the capital cost of a different size of plant, biomass projects usually have limitations on the sizes of certain units and can’t take full advantage of the economies of scale [7]. An analysis of the capital costs vs plant size for starch ethanol plants found that a factor of 0.8 is more appropriate as shown in the following figure.

Figure 1-5	Impact of Starch Ethanol Plant Size on Capital Costs

If the cost of a 100,000 tonne/year plant is $40 million then the estimated cost of a 200,000 tonne per year plant is calculated as

$40 million * (200.000/100,000)0.8 = $69.6 million

Biodiesel plants, where the feedstock is a liquid, exhibited a scaling factor more typical of a process industry as shown in the following figure.

Figure 1-6	Impact of FAME Plant Size on Capital Costs

The reported capital costs for the datasheets is calculated as the production of the various factors that are required to convert the data for the year, location, and size to the 2015 European plant size specified in the data sheets.

Operating Costs

Many of the technologies consume electricity and thermal energy in the production process. The cost of power and natural gas should be the same for any comparison of the technologies. Eurostat reports the electricity [8] and natural gas [9] prices for medium industrial customers by year and country. The 2015 data is shown in the following table. Energy costs are not included in either the fixed or variable operating costs.

Table 1-6	Energy Costs 2015

|  | Electricity | Natural gas |
| --- | --- | --- |
|  | Eur/kWh | Eur/GJ |
| EU 28 | 0.0877 | 9.19 |
| Denmark | 0.0609 | 7.67 |

The Danish Energy Agency maintains forecasts for future power and natural gas costs that can be applied to the analysis of the future economics of the technologies.

Fixed Operating and Maintenance Costs

The fixed share of Operating and Maintenance (O&M) is calculated as cost per plant size (€ per unit of energy per year). It includes all costs, which are independent of how the plant is operated, e.g. administration, operational staff, payments for O&M service agreements, network use of system charges, property tax, and insurance. Any necessary reinvestments to keep the plant operating within the scheduled lifetime are also included, whereas reinvestments to extend the life beyond the lifetime are excluded.

Variable Operating and Maintenance Costs

The variable O&M costs (€ per unit of energy) include consumption of auxiliary materials (water, lubricants, fuel additives), treatment and disposal of residuals, spare parts and output related repair and maintenance (however not costs covered by guarantees and insurances).

Excess Heat

All of the processes are exothermic and there is the potential to capture some of this heat for use in district heating systems. A low, medium and high rating is assigned to each technology to describe the quality of the excess heat. A low rating is applied to systems where the excess heat is available at temperatures of less than 70°C; a medium rating is applied to systems where the heat is available at temperatures between 70 and 150°C. A high rating is applied to systems where the heat is available at temperatures greater than 150°C.

Not all technologies quantify the availability of excess heat, best estimates are provided for those technologies.

Plant Life

It is common practice to depreciate process plants over a twenty year time frame. In practice, well designed and maintained plants can be operated for longer periods of time. Many corn ethanol plants in the US have passed the 20 year old threshold and the first FAME plant in the EU has also exceeded the 20 year threshold. The other technologies considered in this report do not yet have examples of commercial operations that have exceeded 20 years of operation but there is no reason to expect that they will not be designed so that they will operate for more than 20 years.

All of the technologies have therefore been assigned a 25 year value for the plant life.

References

- Wright, T. P. “Factors Affecting the Cost of Airplanes.” Journal of Aeronautical Sciences, Vol. 3, No. 4 (1936), pp. 122-128.

- IEA. 2000. Experience Curves for Energy Technology Policy, http://dx.doi.org/10.1787/9789264182165-en

- National Energy Technology Laboratory. 2013. Technology Learning Curve (FOAK to NOAK). https://www.netl.doe.gov/energy-analyses/pubs/QGESSSec8.pdf

- de Jong, S., Hoefnagels, R., Faaij, A., Slade, R., Mawhood, R. and Junginger, M. 2015. The feasibility of short-term production strategies for renewable jet fuels – a comprehensive techno-economic comparison. Biofuels, Bioprod. Bioref., 9: 778–800. https://www.doi.org/10.1002/bbb.1613

- Chemical Engineering magazine. 2017. Chemical Engineering Plant Cost Index. www.chemengonline.com/pci

- Global CCS Institute. 2011. Economic assessment of carbon capture and storage technologies: 2011 update. http://hub.globalccsinstitute.com/sites/default/files/publications/12786/economic-assessment-carbon-capture-and-storage-technologies-2011-update.pdf

- (S&T)2 Consultants Inc. 2004. Economic, Financial, Social Analysis and Public Policies for Fuel Ethanol. http://www.ghgenius.ca/reports/nrcanethanolphase1.pdf

- Eurostat Electricity Price. 2017. http://ec.europa.eu/eurostat/tgm/table.do?tab=table&init=1&plugin=1&language=en&pcode=ten00117

- Eurostat Natural Gas Price. 2017. http://ec.europa.eu/eurostat/tgm/table.do?tab=table&init=1&plugin=1&language=en&pcode=ten00118

Vegetable Oil FAME

Qualitative Description

Fatty Acid Methyl Esters (FAME) otherwise known as biodiesel can be produced from vegetable oils such as rapeseed and soy oil as well as from used cooking oil and recycled animal fats. This section discusses the production from vegetable oils and the next section of the report discusses the production from used cooking oil and animal fats as the feedstock free fatty acid (FFA) content has some impact on capital and operating costs.

Biodiesel production and use began in Europe in the early 1990’s and that region is still the leading biodiesel marketer and producer. Biodiesel production has since expanded to all regions of the world and there is some international trade in both biodiesel feedstocks and biodiesel.

The European Biodiesel Board reported that the FAME production capacity in Denmark in 2016 was 250,000 tonnes.

Contact Information

Don O’Connor – (S&T)2 Consultants Inc.

Brief Technology Description

The production of biodiesel, or methyl esters, is a well-known process. Vegetable oils are mixed with methanol in the presence of a catalyst at moderate pressure and temperatures to produce biodiesel and glycerine.  Since the methanol is not soluble in the oil, this reaction will proceed either exceedingly slowly or not at all, so heat, as well as catalysts (acid and/or base) are used to speed the reaction. Almost all biodiesel produced from virgin vegetable oils uses the base-catalyzed technique as it is the most economical process for treating virgin vegetable oils, requiring only low temperatures and pressures and producing over 98% conversion yield (provided the starting oil is low in moisture and free fatty acids).

After the vegetable oil and methanol react in the presence of a catalyst to produce the biodiesel, the product goes through a number of separation and purification steps to recover the excess methanol for re-use, isolate the glycerine for potential upgrading and ensue that the methyl ester meets the required specifications.

The general biodiesel production process is shown in the following figure.

Figure 2-1	Biodiesel Production Process

There are other processing schemes that can be used to produce FAME from vegetable oils including the use of heterogeneous catalysts, enzymes instead of chemical catalysts and a supercritical process. This catalyst-free method for transesterification uses supercritical methanol at high temperatures and pressures in a continuous process. In the supercritical state, the oil and methanol are in a single phase, and reaction occurs spontaneously and rapidly. The process can tolerate water in the feedstock; free fatty acids are converted to methyl esters instead of soap, so a wide variety of feedstocks can be used. Also the catalyst removal step is eliminated. High temperatures and pressures are required, but energy costs of production are similar or less than catalytic production routes. These alternative processes are not yet widely practiced.

Input

The primary inputs to the production process are the vegetable oil, methanol, electricity, some thermal energy, the catalyst, and some acids and bases to treat the feedstocks and finished products.

Output

The plants produce FAME, glycerine (of various qualities), and in some cases potassium salts that can be sold as fertilizer.

Energy Balance

The energy balance is shown graphically in the following figure. The external energy inputs, in the form of heat and power, are relatively small and the process is quite efficient. The sum of the inputs totals 100 MJ.

Figure 2-2	Vegetable Oil FAME Energy Balance

A small portion, about 5%, of the energy losses has the potential to be recovered as low level district heat [1].

Typical Capacities

Production capacities for individual plants can range from less than 10,000 tonnes per year to almost 600,000 tonnes per year (10 to 750 MW).

It was reported that in 2010, 46 FAME plants in Germany had a production capacity of almost 5 million tonnes per year [2]. The average plant size was 100,000 tonnes but more than 50% of the plant production capacity was found in 9 plants with more than 200,000 tonnes of capacity each.

Regulation Ability

FAME plants can be either batch or continuous processes. The time between batches regulates the total production from batch plants. Larger plants are generally continuous processes. The throughput can be regulated to a degree but process start-up is generally fast so regulating the days of operation is also a viable means to regulate production.

Space Requirements

A 100,000 tonne per year facility can be located on a site of less than 5 ha. This is equivalent to 0.04 ha/million litres of fuel, or 400 m2/MW. Factors that impact the required area include storage capacity of inputs and outputs and whether or not rail access is included at the facility.

Advantages/Disadvantages

FAME biodiesel is a relatively simple process that is well proven commercially. The capital costs are relatively low and the non-feedstock operating costs are also reasonable.

The feedstock costs are high compared to crude oil. The finished fuel has less desirable cold weather properties than fossil diesel fuel limiting its potential inclusion rates in diesel fuel in cold weather.

Environment

With low energy use the GHG emissions from the biodiesel plants are relatively minor. The GHG emissions of the methanol must also be included in the carbon footprint. Methanol and the process chemicals used have safety hazards associated with their use but these are well known and there are establishes procedure to accommodate their safe use.

The energy content of the biodiesel is 30 times the power and natural gas energy consumed by the plant.

Research and Development Perspective

Vegetable oil FAME is a Category 4 technology, a commercial technology with large scale deployment. The potential for significant improvements in capital and operating costs is limited. The price and performance of the technology today is well known and normally only incremental improvements would be expected. Therefore, the future price and performance may also be projected with a relatively high level of certainty.

Examples of Market Standard Technology

A number of companies provide biodiesel technology packages. Desmet Ballestra [3] is one such company.

Desmet Ballestra Group N.V. 
Fountain Plaza Office Park, Building 503
Belgicastraat 3
B - 1930 ZAVENTEM
Belgium

They have been in involved in over 85 plants around the world with capacity in excess of 12 million tonnes/year. Their plants have capacities from 50,000 to 300,000 tonnes per year.

Predication of Performance and Cost

Biodiesel production from vegetable oils is mature technology. Conversion performance in most plants approaches the theoretical maximum conversion rates. There is therefore limited development potential for the technology.

Worldwide production rates are about 30 billion litres per year with the cumulative production between 2000 and 2014 being 195 billion litres as shown in the following figure [4].

Figure 2-3	World Biodiesel Production

At current production rates the cumulative production since 2000 will double in six years. Berghout investigated the Progress ratio for the German Biodiesel industry between 1991 and 2004 and found that the limited data suggested a value of 0.97. This is a very low value and may be due to the fact that while the industry is new, the processing steps are based on mature chemical processes with limited potential for improvement. The cost reduction would also not apply to feedstock so the impact on production economics will be marginal.

Uncertainty

Given the mature status of the technology there are low levels of uncertainty associated with the data that is collected, but there will always be some uncertainty regarding future projections.

Additional Remarks

This technology is commercially practiced in Denmark today by Emmelev A/S. Denmark blends FAME and HVO at about 7% by energy content in diesel fuel [5]. The FAME could be from vegetable oil or from UCO.

Quantitative Description

The basic chemical reaction is depicted below. One hundred kilograms of a fat or oil is reacted with 10 kilograms of methanol in the presence of a catalyst to produce 10 kilograms of glycerine and 100 kilograms of methyl esters or biodiesel. The methanol is charged in excess to assist in quick conversion and recovered for reuse. The catalyst is usually sodium or potassium methoxide which has already been mixed with the methanol. R1, R2, and R3 indicate the fatty acid chains associated with an individual fat.

Figure 2-4	Basic Biodiesel Reactions

Energy/Technical Data

Quantitative data on the FAME vegetable oil process is presented below.

Typical Plant Size

While there has been a wide range of plants built in the past, large scale plants can enjoy some economies of scale. The one vegetable oil biodiesel plant in Denmark (Emmelev) has a production capacity of 100,000 tonnes per year [6]. This is the average plant size reported in Germany in 2010 and will be used for the data sheet.

Input and Output

The typical mass and energy balances for the primary materials and energy sources are shown in the following table. The data is from a 2016 Survey of vegetable oil biodiesel plants operating in the United States [7]. The data is from 12 plants with production capacities ranging from 40,000 to 330,000 tonnes per year. The plants operated at an average of 74% of capacity.

Table 2-1	Mass and Energy Flows

|  | Inputs | Outputs |
| --- | --- | --- |
| Vegetable oil | 0.893 kg |  |
| Methanol | 0.086 kg |  |
| FAME (one litre) |  | 0.88 kg |
| Glycerine |  | 0.09 kg |
| Electricity | 0.036 kWh |  |
| Natural gas | 0.93 MJ (LHV) |  |

The input and output data for future years assumes that the industry average moves towards the current best in class values.

Forced and Planned Outage

Biodiesel plants can operate continuously with limited downtime for planned maintenance. The NBB energy survey found that plants operated from about 40 to over 95% of production capacity in 2015.

The most likely reasons for forced outage are either a lack of feedstock or a lack of markets for the product.

Technical Lifetime

The first European plants were built in the 1990s and many of those are still operating more than 20 years later. Processing conditions are relatively mild with moderate pressures and temperatures for most processes. The corrosive nature of some of the reagents (methanol, acids and bases) are well understood and manageable.

Construction Time

Large scale plants should have a typical construction time of 12 to 18 months from the start of site preparation to the beginning of production.

Financial Data

Vegetable oil FAME plants are commercial technologies with capital and operating costs that are relatively well understood. There have been relatively few FAME plants constructed in recent years as the global industry is operating at less than full capacity.

Investment Costs

Most European vegetable oil FAME plants were built a decade ago. The most recent vegetable oil biodiesel plant that has been announced is a 200,000 tonne soybean plant to be built by Cargill in Kansas, USA. The plant will be built on the site of an existing soybean crushing facility and will replace an existing soy oil refinery. The capital cost is reported to be $90 million US$ [8].

Converting this cost to Europe, the location factor, exchange rate, and greenfield factor is applied as shown below.

US capital cost*greenfield factor*currency factor*Location factor

$90,000,000*1.2*0.85*1.21=€111 million

It is estimated that a 100,000 tonne per year plant would have a capital cost of €64 million. This is $0.66/litre, which is in the middle of the range reported by Irena [9].

Future investment costs are not expected to decline significantly as few new plants are being built. Berghout, with limited data, concluded that learning-by-upscaling to be the primary and learning-by doing the secondary driver behind reductions in specific investment costs over time for German biodiesel plants [10]. Since the same plant size has been used for the study period of 2015 to 2050 only a small reduction (~5%) in investment costs is projected by 2050.

Operating and Maintenance Costs

The production cost profile is dominated by the feedstock costs. Feedstock costs are highly variable and can increase or decrease in any given year due to the local supply and demand situation.

Iowa State University [11] maintains a biodiesel profitability model for a 100,000 tonne/year soy biodiesel plant. The 2016 data has been used and the power and natural gas costs have been adjusted to the Danish values reported in section 1.4. The capital cost has been adjusted to 64 million Euro to match the value above. The results for 2016 are shown in the following table.

Table 2-2	Production Costs Vegetable oil FAME

| Parameter | Costs (Eur/litre) |
| --- | --- |
| Natural gas | 0.01 |
| Methanol | 0.02 |
| Other Variable Costs | 0.06 |
| Fixed Costs | 0.02 |
| Depreciation and Interest | 0.06 |
| Total Costs ex feedstock | 0.17 |

The other variable costs include chemicals, power, water, repairs and maintenance, water, transportation and other costs. The fixed costs include depreciation, interest costs, labour and management, property taxes, insurance, and marketing and procurement costs.

Berghout found a very low Progress Ratio (97.4%) and a low R2 value (0.65) for the FAME processing costs, suggesting little potential for reductions in O&M costs over time.

The impact on production costs as the future industry moves towards the best practices of the existing plants is limited as gains in yield will impact the feedstock costs which are not included here; the natural gas, power and methanol costs which are a small portion of the operating costs.

Start-up Costs

The O&M costs stated in this catalogue includes start-up costs and takes into account a typical number of start-ups and shut-downs. Therefore, the start-up costs should not be specifically included in more general analyses.

Technology Specific Data

Some of the key fuel properties of biodiesel are shown in the following table.

Table 2-3	Vegetable Oil Biodiesel Properties

| Property | Value |
| --- | --- |
| Density, kg/m3 | 885 |
| LHV, MJ/kg | 37.2 |
| LHV, MJ/litre | 32.7 |
| Oxygen content | 11% |
| Cetane number | ~55 |
| Cloud point, C | ~-3 |

Summary

The quantitative data reported above is summarized in the following data sheet. This information is determined from the NBB operating data for vegetable oil feedstock plants and the operating costs estimated from the Iowa State model. The best biodiesel plants operate a close to theoretical yields and methanol consumption rates, and with low energy consumption values. The 2015 data is based on the 2015 NBB data, it has been assumed that by 2050 the industry moves to have an average performance the same as the best plant in 2015. The 2020 and 2030 values are estimated from the transition from the 2015 to 2050 values.

Table 2-4	Vegetable Oil FAME Datasheet

| Technology | Vegetable Oil FAME |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2030 | 2050 | Uncertainty (2020) |  | Uncertainty (2050) |  | Note | Ref |
|  |  |  |  |  | Lower | Upper | Lower | Upper |  |  |
| Energy/technical data |  |  |  |  |  |  |  |  |  |  |
| Typical total plant size 1,000 t biodiesel/year | 100 | 100 | 100 | 100 |  |  |  |  | A | 1 |
| Typical total plant size MW biodiesel/year | 125 | 125 | 125 | 125 |  |  |  |  |  |  |
| Inputs |  |  |  |  |  |  |  |  |  |  |
| Veg Oil Consumption, MWh/MWh Total Input | 0.925 | 0.926 | 0.929 | 0.933 | 99% | 101% | 99% | 101% | E,I | 4 |
| Methanol Consumption, MWh/MWh Total Input | 0.046 | 0.046 | 0.046 | 0.046 | 93% | 107% | 93% | 107% | B,E,I | 4 |
| Electricity Consumption, MWh/MWh Total Input | 0.004 | 0.004 | 0.003 | 0.002 | 50% | 150% | 75% | 125% | E,I | 4 |
| Natural Gas Consumption, MWh/MWh Total Input | 0.025 | 0.024 | 0.022 | 0.018 | 50% | 150% | 75% | 125% | E,I | 4 |
| Outputs |  |  |  |  |  |  |  |  |  |  |
| Biodiesel Output, MWh/MWh Total Input | 0.903 | 0.903 | 0.903 | 0.903 | 99% | 101% | 99% | 101% | F | 4 |
| Glycerine Output, MWh/MWh Total Input | 0.039 | 0.039 | 0.039 | 0.039 | 99% | 101% | 99% | 101% | F | 4 |
| Forced outage (%) | 0 | 0 | 0 | 0 |  |  |  |  | H |  |
| Planned outage (weeks per year) | 2 |  |  |  |  |  |  |  |  |  |
| Technical lifetime (years) | 25 |  |  |  |  |  |  |  |  |  |
| Construction time (years) | 1.5 |  |  |  |  |  |  |  |  |  |
| Financial data |  |  |  |  |  |  |  |  |  |  |
| Specific investment (€ /l biodiesel) | 0.66 | 0.65 | 0.64 | 0.63 | 0.5 | 0.8 | 0.5 | 0.8 | C, J |  |
| - equipment (%) | 75 | 75 | 75 | 75 |  |  |  |  | G |  |
| - installation (%) | 25 | 25 | 25 | 25 |  |  |  |  | G |  |
| Fixed O&M (€ /l biodiesel) | 0.02 | 0.02 | 0.02 | 0.02 | 90% | 110% | 90% | 110% | D, K |  |
| Variable O&M (€ /l biodiesel) | 0.05 | 0.05 | 0.05 | 0.05 | 90% | 110% | 90% | 110% | D, K |  |
| Startup (€ /l biodiesel) | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Technology specific data |  |  |  |  |  |  |  |  |  |  |

Notes:

A. The plant size is assumed based on the plants sizes of existing vegetable oil based bio-diesel plants in EU.

B. Energy input from supporting chemicals of minor amounts is not considered.

C. The total capital cost include total installed cost (total direct costs) and all indirect costs such as engineering, construction, contractor’s fee, contingency and working capital.

D. O&M costs, costs for main raw materials are not included

E. 2050 values are based on industry best plant in 2015. 2020 and 2030 values interpolated between 2015 and 2050 values.

F. Total output excludes the heat loss.

G. Estimated from Iowa State biodiesel profitability file.

H. Best plants operate at greater than 95% of capacity.

I. The uncertainty is one standard deviation from the average from the NBB survey.

J. Range of capital cost is +/- 25% of mean.

K. Operating cost is +/- 10% of mean.

References

- Personal Communication. 2018. T Jensen, Emmelev A/S

- International Institute for Sustainable Development. 2012. Biofuels – At What Cost? Mandating ethanol and biodiesel consumption in Germany. https://www.iisd.org/gsi/sites/default/files/bf_awc_germany.pdf

- Desmet Ballestra. 2015. Biodiesel. http://www.desmetballestra.com/images/PDF-Brochures-2014/Biodiesel2015.pdf

- US Energy Information Administration. 2015. International Energy Statistics. Biofuels Production. http://www.eia.gov/cfapps/ipdbproject/IEDIndex3.cfm?tid=79&pid=79&aid=1

- Energy Statistics 2015. https://ens.dk/sites/ens.dk/files/Statistik/energy_statistics_2015.pdf

- IEA Bioenergy Task 39. Newsletter #39. Denmark Feature. http://task39.sites.olt.ubc.ca/files/2015/06/IEA-Bioenergy-Task-39-Newsletter-Issue-39-May-2015-final.pdf

- Chen, R., Qin, Z., Han, J., Wang, M., Taheripour, F., Tyner, W., O'Connor, D., Duffield, J. 2018. Life cycle energy and greenhouse gas emission effects of biodiesel in the United States with induced land use change impacts. Bioresource Technology, Volume 251, 2018, Pages 249-258, ISSN 0960-8524, https://doi.org/10.1016/j.biortech.2017.12.031.

- Cargill. 2017. Cargill to build state of- the-art biodiesel plant in Wichita, Kansas. https://www.cargill.com/2017/cargill-to-build-state-of-the-art-biodiesel-plant-in-wichita-ks

- IRENA. 2013. Road Transport: The Cost of Renewable Solutions. http://www.irena.org/-/media/Files/IRENA/Agency/Publication/2013/Road_Transport.ashx

- Berghout, Niels. (2008). Technological learning in the German biodiesel industry: An experience curve approach to quantify reductions in production costs, energy use and greenhouse gas emissions. http://task39.sites.olt.ubc.ca/files/2013/05/Technological-learning-in-the-German-Biodiesel-Industry.pdf

- Iowa State University. 2017. Biodiesel Profitability. http://www.extension.iastate.edu/agdm/energy/xls/d1-15biodieselprofitability.xlsx

UCO and Animal Fat FAME

Qualitative Description

Used Cooking Oil (UCO) and animal fats can also be used to produce FAME. This section of the report discusses FAME production using these two feedstocks. The primary technical differentiation factor for these two feedstocks is the free fatty acid (FFA) content of the feedstocks. Free fatty acids can be produced when the lipids are heated, as they are when the cooking oils are used or when the animal carcass’ are rendered.

UCO and animal fats are generally less expensive feedstocks than vegetable oils. UCO is also treated as a waste material in most regulatory systems so it generates a better carbon intensity score and the volume is double counted in some systems. These attributes make it an attractive feedstock, more than outweighing some processing challenges described below.

Contact Information

Don O’Connor – (S&T)2 Consultants Inc.

Brief Technology Description

The base catalyzed transesterification process that produces FAME or biodiesel from vegetable oils is only effective with triglycerides. FFAs require an acid catalyzed esterification reaction for commercial production. This reaction is slower and involves an extra processing step. Some UCO and animal fat biodiesel producers accept the lower yield that results from not converting the FFAs to biodiesel and some use the two step process that converts both triglycerides and FFAs to biodiesel.

The two step process is shown below, the basic difference between this and the process used for vegetable oils is the initial esterification steps.

Figure 3-1	Two Step Biodiesel Production Process

Input

The primary inputs to the production process are the UCO or animal fat, methanol, electricity, some thermal energy, the catalyst(s), and some acids and bases to treat the feedstocks and finished products.

Output

The plants produce FAME, glycerine (of various qualities), in some cases potassium salts that can be sold as fertilizer, and unreacted FFAs.

Energy Balance

The average energy balance for the year 2015 from 15 US biodiesel plants processing mixed feedstocks is shown in the following figure [1]. Most of these plants do not do the two step process and accept the lower yield due to the higher FFA content of the feedstock.

Figure 3-2	UCO and Animal Fat FAME Energy Balance

The potential for recovering some heat for district heating is similar to that of the vegetable oil FAME process, about 5% of the heat loss.

Typical Capacities

The size of UCO and animal fat plants is slightly smaller than the size of vegetable oil plants. Production capacities of up to 150,000 tonnes per year (185 MW) have been built in Europe.

The Daka ecoMotion plant in Denmark has a capacity of 50,000 tonnes per year (60 MW). This plant uses the two step process of esterification followed by transesterification. The plant was designed by BDI in 2008.

Regulation Ability

FAME plants can be either batch or continuous processes. The time between batches regulates the total production from batch plants. Larger plants are generally continuous processes. The throughput can be regulated to a degree but process start-up is generally fast so regulating the days of operation is also a viable means to regulate production.

Space Requirements

The Daka plant in Denmark sits on 2.1 ha of land. This is 0.038 ha/million litres or 340 m2/MW of biodiesel.

All of the materials in and out of the plant come by truck. The plant is shown in the following figure. The actual storage and processing equipment coves less than 50% of the site.

Figure 3-3	Daka FAME Plant

Advantages/Disadvantages

UCO and animal fats have many of the same advantages and disadvantages as vegetable oil FAME when compared to petroleum diesel fuel.

When UCO and animal fat FAME is compared to vegetable oil FAME the feedstock costs are usually lower but capital and operating costs are slightly higher. The cold weather properties of UCO and animal fat FAME are usually slightly less attractive than the vegetable oil FAME.

Environment

With low energy use the emissions from the biodiesel plants are relatively minor. Methanol and the process chemicals used have safety hazards associated with their use but these are well known and there are establishes procedure to accommodate their safe use.

The energy content of the UCO and animal fat biodiesel is about 20 times the power and natural gas energy consumed by the plant.

Research and Development Perspective

Used Cooking Oil (UCO) and animal fat FAME is a Category 4 technology, a commercial technology with large scale deployment. The price and performance of the technology today is well known and normally only incremental improvements would be expected. Therefore, the future price and performance may also be projected with a relatively high level of certainty.

The potential for significant improvements in capital and operating costs is limited. There is work being undertaken on making the use of lower quality feedstocks commercially attractive. These materials (e.g. brown grease) can have very high FFA levels and contain other contaminants such as sulphur and chlorine.

Examples of Market Standard Technology

One of the leaders with the multi-feedstock biodiesel process is the Austrian company, BDI Bioenergy International.

BDI - BioEnergy International AG

Parkring 18, 8074

Raaba-Grambach, Austria

They have built more than 30 biodiesel plants, mostly in Europe, since 1991.

Predication of Performance and Cost

Biodiesel production from UCO and animal fats is relatively mature technology. Conversion performance in the best plants that use the two step process approaches the theoretical maximum conversion rates. There is therefore limited development potential for the technology.

Berghout [2] investigated the Progress Ratio for the German Biodiesel industry between 1991 and 2004 and found that the limited data suggested a value of 0.97. This is a very low value and may be due to the fact that while the industry is new, the processing steps are based on mature chemical processes with limited potential for improvement. The cost reduction would also not apply to feedstock so the impact on production economics will be marginal.

Uncertainty

Given the mature status of the technology there are relatively low levels of uncertainty associated with the data that is collected. One area of potential uncertainty is the proportion of plants that practice esterification in the United States (where the plant data comes from) vs the EU.

Additional Remarks

This technology is commercially practiced at the Daka plant in Denmark. Europe is an importer of used cooking oil as demand for the product exceeds the quantity supplied in Europe [2]. It is also reported that that there are limited opportunities to increase the collection rate of UCO in Europe [3], resulting in about 60% of the UCO consumed in Europe is imported into the EU.

Quantitative Description

Additional quantitative information on the production of UCO or animal fat FAME is presented in the following sections.

The esterification reaction that is practiced with some technologies prior to the transesterification process is shown below. In this reaction there is no glycerine produced but there is some water produced.

Figure 3-4	Esterification Reaction

Energy/Technical Data

Quantitative data on the UCO animal fat FAME process is presented below.

Typical Plant Size

The plants can range in size from 5,000 to over 100,000 tonnes per year of capacity. Older plants generally have lower production rates as market access for the production would have been an issue when they were built.

A plant of 50,000 tonne/year is used for the data sheet. This is in the middle of the range of plants built and is the same size as the existing Danish plant.

Input and Output

The typical mass and energy balances for the primary materials and energy sources are shown in the following table. The data is from a 2016 Survey of vegetable oil biodiesel plants operating in the United States [1]. The data is from 15 plants with production capacities ranging from 3,000 to 200,000 tonnes per year. The plants operated at an average of 73% of capacity. The primary feedstocks for these plants were animal fats, distillers’ corn oil (extracted from the stillage of corn ethanol plants), and used cooking oil. About 10% of the feed for these mixed feedstock plants was vegetable oils.

Table 3-1	Mass and Energy Flows

|  | Inputs | Outputs |
| --- | --- | --- |
| Vegetable oil | 0.977kg |  |
| Methanol | 0.087 kg |  |
| FAME (one litre) |  | 0.88 kg |
| Glycerine |  | 0.09 kg |
| Electricity | 0.08 kWh |  |
| Natural gas | 1.71 MJ (LHV) |  |

The FAME yield is lower, indicating that many of the plants do not practice the esterification step and the energy use is about double that of the vegetable oil plants. The energy use would be higher if more of the plants were batch processes rather than continuous operations.

Forced and Planned Outage

Biodiesel plants can operate continuously with limited downtime for planned maintenance. The NBB energy survey found that plants operated from about 40 to over 97% of production capacity in 2015.

The most likely reasons for forced outage are either a lack of feedstock or a lack of markets for the product.

Technical Lifetime

The first European plants were built in the 1990s and many of those are still operating more than 20 years later. Processing conditions are relatively mild with moderate pressures and temperatures for most processes. The corrosive nature of some of the reagents (methanol, acids and bases) are well understood and manageable.

Construction Time

Large scale plants should have a typical construction time of 12 to 18 months from the start of site preparation to the beginning of production.

Financial Data

Information on capital and operating costs are presented below.

Investment Costs

Argent Energy built a 75,000 tonne per year multi-feedstock facility near Chester, UK in 2015-2016. The reported cost was £75 million (€100 million) [4]. This is considerably more expensive than the recent cost estimate for a soybean biodiesel plant in the United States. The plant has significant pretreatment facilities and can reportedly process 100% FFA feedstocks. Given the extra processing required for the plants processing higher FFA feedstocks some increase in price is expected but this €1.17/litre, almost double the soy oil cost.

The future capital costs have been reduced by about 25% by 2050. The rationale for the larger percent cost reduction than a vegetable oil FAME plant is that a greater yield improvement is expected as the industry moves towards the best in class performance, due to the higher capital costs there should be more room for improvement, and there is greater emphasis being place on the use of “waste” feedstocks by many governments.

Operating and Maintenance Costs

There is no public source of economic data for UCO or animal fat biodiesel plants like the Iowa State information on soybean biodiesel plants. Given the higher energy requirements, the higher capital costs, the potentially lower yields the operating costs should be higher than they are for a vegetable oil plant.

A German biodiesel Petrotec, a UCO biodiesel producer, used to supply some financial information to their shareholders. The last information available is for 2014 [5], before they were purchased by REG Inc. from the United States. The plant has a capacity of 185,000 tonnes per year and operated at 75% capacity in 2014. The information that can be developed from the annual report is shown in the following table.

Table 3-2	UCO Biodiesel Operating Costs

| Category | Euro /litre |
| --- | --- |
| Services | 0.05 |
| Labour | 0.05 |
| Operating Costs | 0.04 |
| Marketing Costs | 0.03 |
| Admin Costs | 0.01 |
| Legal Costs | 0.01 |
| Other Expenses | 0.01 |
| Interest | 0.01 |
| Total | 0.21 |

The fixed costs (labour, marketing, admin, legal, other, interest) total €0.13/litre. There is no breakdown of feedstock costs from other supplies in the published information. The vegetable oil plant has variable operating costs of €0.05/litre, the chemical costs will be higher for the UCO facility, it is estimated that the variable operating costs of €0.07/litre.

As with the vegetable oil FAME plants only limited improvements in the fixed and variable production costs are forecast due to the low Progress ratio found by Berghout.

Start-up Costs

The O&M costs stated in this catalogue includes start-up costs and takes into account a typical number of start-ups and shut-downs. Therefore, the start-up costs should not be specifically included in more general analyses.

Technology Specific Data

Table 3-3	UCO and Animal Fat Biodiesel Properties

| Property | Value |
| --- | --- |
| Density, kg/m3 | 885 |
| LHV, MJ/kg | 37.2 |
| LHV, MJ/litre | 32.7 |
| Oxygen content | 11% |
| Cetane number | 50-60 |
| Cloud Point, C | 0-10 C |

Summary

The quantitative information collected for a UCO biodiesel facility is shown in the following table. This information is determined from the NBB operating data for mixed feedstock plants and the operating costs estimated from the Petrotec annual report. The best biodiesel plants operate a close to theoretical yields and methanol consumption rates, and with low energy consumption values. The 2015 data is based on the 2015 NBB data; it has been assumed that by 2050 the industry moves to have an average performance the same as the best plant in 2015. The 2020 and 2030 values are estimated from the transition from the 2015 to 2050 values.

Table 3-4	UCO and Animal Fat FAME Datasheet

| Technology | UCO and Animal Fat FAME |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2030 | 2050 | Uncertainty (2020) |  | Uncertainty (2050) |  | Note | Ref |
|  |  |  |  |  | Lower | Upper | Lower | Upper |  |  |
| Energy/technical data |  |  |  |  |  |  |  |  |  |  |
| Typical total plant size 1,000 t biodiesel/year | 50 | 50 | 50 | 50 |  |  |  |  | A | 1 |
| Typical total plant size MW biodiesel/year | 60 | 60 | 60 | 60 |  |  |  |  |  |  |
| Inputs |  |  |  |  |  |  |  |  |  |  |
| Veg Oil Consumption, MWh/MWh Total Input | 0.906 | 0.909 | 0.912 | 0.912 | 99% | 101% | 99% | 101% | E,I | 1 |
| Methanol Consumption, MWh/MWh Total Input | 0.044 | 0.044 | 0.044 | 0.044 | 93% | 107% | 93% | 107% | B,E,I | 1 |
| Electricity Consumption, MWh/MWh Total Input | 0.007 | 0.006 | 0.005 | 0.004 | 50% | 150% | 50% | 150% | E,I | 1 |
| Natural Gas Consumption, MWh/MWh Total Input | 0.044 | 0.040 | 0.036 | 0.032 | 50% | 50% | 50% | 50% | E,I | 1 |
| Outputs |  |  |  |  |  |  |  |  |  |  |
| Biodiesel Output, MWH/MWh Total Input | 0.84 | 085 | 0.86 | 0.87 | 99% | 101% | 99% | 101% | F | 1 |
| Glycerine Output, MWH/MWh Total Input | 0.04 | 0.04 | 0.04 | 0.04 | 99% | 101% | 99% | 101% | F | 1 |
| Forced outage (%) | 0 | 0 | 0 | 0 |  |  |  |  | H |  |
| Planned outage (weeks per year) | 2 |  |  |  |  |  |  |  |  |  |
| Technical lifetime (years) | 25 |  |  |  |  |  |  |  |  |  |
| Construction time (years) | 1.5 |  |  |  |  |  |  |  |  |  |
| Financial data |  |  |  |  |  |  |  |  |  |  |
| Specific investment (€ /l biodiesel) | 1.17 | 1.1 | 1.0 | 0.9 | 0.8 | 1.2 | 0.8 | 1.0 | C, J | 3 |
| - equipment (%) | 75 | 75 | 75 | 75 |  |  |  |  | G |  |
| -installation (%) | 25 | 25 | 25 | 25 |  |  |  |  | G |  |
| Fixed O&M (€ /l biodiesel) | 0.13 | 0.13 | 0.13 | 0.13 | 90% | 110% | 90% | 110% | D, K | 4 |
| Variable O&M (€ /l biodiesel) | 0.07 | 0.07 | 0.07 | 0.07 | 90% | 110% | 90% | 110% | D, K | 4 |
| Startup (€ /l biodiesel) | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Technology specific data |  |  |  |  |  |  |  |  |  |  |

Notes:

A. The plant size is assumed based on the plants sizes of existing UCO based bio-diesel plants in EU.

B. Energy input from supporting chemicals of minor amounts is not considered.

C. The total capital cost include total installed cost (total direct costs) and all indirect costs such as engineering, construction, contractor’s fee, contingency and working capital.

D. O&M costs only, costs for main raw materials are not included

E. 2050 values are based on industry best plant in 2015. 2020 and 2030 values interpolated between 2015 and 2050 values.

F. Total output excludes the heat loss.

G. Estimated from Iowa State biodiesel profitability file.

H. Best plants operate at greater than 95% of capacity.

I. The uncertainty is one standard deviation from the average from the NBB survey.

J. Range of capital cost is +2/- 30% of mean for 2015.

K. Operating cost is +/- 10% of mean.

References

- Chen, R., Qin, Z., Han, J., Wang, M., Taheripour, F., Tyner, W., O’Connor, D., Duffield, J. 2018. Life cycle energy and greenhouse gas emission effects of biodiesel in the United States with induced land use change impacts. Bioresource Technology, Volume 251, 2018, Pages 249-258, ISSN 0960-8524, https://doi.org/10.1016/j.biortech.2017.12.031.

- Berghout, Niels. (2008). Technological learning in the German biodiesel industry: An experience curve approach to quantify reductions in production costs, energy use and greenhouse gas emissions. http://task39.sites.olt.ubc.ca/files/2013/05/Technological-learning-in-the-German-Biodiesel-Industry.pdf

- Greenea. 2016. Waste-based feedstock and biofuels market in Europe. https://www.greenea.com/wp-content/uploads/2016/11/Argus-2016.pdf

- The Chester Chronicle. May 19, 2016. Ellesmere Port's new £75m waste chip oil plant will power cars. http://www.chesterchronicle.co.uk/news/chester-cheshire-news/ellesmere-ports-new-75m-waste-11352514

- Petrotec 2014 Annual Report. http://www.equitystory.com/Download/Companies/petrotec/Annual%20Reports/DE000PET1111-JA-2014-EQ-E-00.pdf

Hydrogenated Vegetable Oil

Qualitative Description

Hydrogenated vegetable oil (HVO) plants use the same feedstocks that are used for FAME plants including used cooking oils and animal fats, however rather than reacting the feedstock with methanol the feedstock is reacted with hydrogen in the presence of a catalyst. The operating temperatures and pressures are higher than in FAME plants.

There are small differences in hydrogen demand between the different feedstocks that are used but the variation is generally less than what is seen between process developers and between plants so only one data sheet is presented.

The resulting product is oxygen free and has better fossil diesel blending properties. The product is generally isomerized to improve the cold weather properties of the fuel.

Contact Information

Don O’Connor – (S&T)2 Consultants Inc.

Brief Technology Description

HVO is a mixture of straight chain and branched paraffins – the simplest type of hydrocarbon molecules from the point of view of clean and complete combustion. Typical carbon numbers are C15 to C18. Paraffins exist also in fossil diesel fuels which additionally contain significant amounts of aromatics and naphthenics. Aromatics are not favorable for clean combustion. HVO is practically free of aromatics and its composition is quite similar to GTL and BTL diesel fuels made by Fischer Tropsch synthesis from natural gas and gasified biomass.

The general HVO production steps are shown in the following figure.

Figure 4-1	HVO Production Process

Input

The primary inputs to the production process are the fats and oils and hydrogen, along with some electricity, thermal energy, and chemicals.

Output

The outputs from the process are the HVO, some fuel gas (a mixture of hydrogen and light hydrocarbons), propane, and a light liquid stream that can be used as fuel or as a renewable gasoline blending component.

Energy Balance

High quality public information on the energy balance of the HVO process is very limited. The following figure shows the typical energy balance based on a composite of information in the public domain [1] [2] [3]. The sum of the inputs totals 100 MJ.

Figure 4-2	HVO Energy Balance

There is the potential to recover about 15% of the heat loss as high temperature heat for district heating applications.

Typical Capacities

Stand-alone HVO plants range in size from 150 million litres/year (AltAir, USA) to move than 1,000 million litres per year (Neste Singapore and Rotterdam) (170 to 1,125 MW).

Regulation Ability

Very little information is available on the turn down ratios of the process. It is a process with a high operating temperature and pressure and thus will likely have limited capacity to regulate throughputs. The existing HVO plants are generally running at capacity.

Space Requirements

The Neste plants in Singapore and Rotterdam and the Diamond Green Diesel and REG plants in Louisiana are greenfield plants that have some integration with nearby utility suppliers. The space required for the plants are in the 0.02 to 0.025 ha/million litres of production range (220 m2/MW).

Advantages/Disadvantages

HVO has a number of advantages over biodiesel as a fuel and as a production process. The process is not restricted by the free fatty acid content of the feedstock and most plants process a wide range of feedstocks from vegetable oils to UCO and animal fats.

HVO can be blended with petroleum diesel fuel without any limit or labelling requirements because it is a hydrocarbon. HVO has a higher cetane value than biodiesel and can be produced with better cold weather properties as a result of the secondary isomerization step in the process.

The fuel is sulphur free and has a very low aromatics content (although this can cause issues with some engine seals).

The market price of HVO is higher than that of biodiesel and of petroleum diesel.

Environment

The process has a low ratio of energy consumed to fuel produced (~0.02) excluding the hydrogen but drops to about 0.15 when the hydrogen energy is considered.

The hydrotreating catalysts have a long life and can be re-generated. There is limited waste generated as part of the process, although this can depend on the feedstock and the need to pre-treat the feedstock to remove minor contaminants.

Research and Development Perspective

Hydrogenated vegetable oil (HVO) is a Category 4 technology, a commercial technology with large scale deployment. While there are fewer HVO facilities in the world compared to FAME facilities, the basic processing technology have been practiced for many years in petroleum refineries. The price and performance of the technology today is well known and normally only incremental improvements would be expected. Therefore, the future price and performance may also be projected with a relatively high level of certainty.

Examples of Market Standard Technology

The leading commercial supplier of the technology is Honeywell UOP. Their corporate office is:

UOP World Corporate Offices
25 E. Algonquin Road (Bldg A)
P.O. Box 5017
Des Plaines, IL 60017-5017
Toll Free: +1 (800) 877-6184
Phone: +1 (847) 391-2000

They have a European office at

Noorderlaan 147
B-2030 Antwerp, Belgium
Phone: +32-3-540-9911

Haldor Topsøe offers their HydroFlex™ technology. Topsøe has been researching and developing innovative solutions for renewable fuel production since 2004. They claim that many licensed HydroFlex™ units are in full operation today.

Haldor Topsøe A/S 
Haldor Topsøes Allé 1
DK-2800
Kgs. Lyngby
Denmark

Neste and REG have developed their own technology and have not sold it to any other producers at this time.

Predication of Performance and Cost

The production of HVO from triglycerides is a commercial technology that is deployed at scale. Only small incremental improvements on performance can be expected in the future. The current price and performance information presented has a relatively high level of certainty, although with a limited number of participants in the sector much of the information is considered confidential by the producers.

Future capital costs can be expected to improve as the plant size increases, however the future inputs and outputs are expected to be very stable due to the high levels of performance of the existing facilities. These assumptions are confirmed by the work of de Jong [4] who determined that the technology had a high capital growth factor indication a small difference between the capital costs of the pioneering plants and the nth plant.

Uncertainty

There are relatively low levels of uncertainty with respect to this pathway as it is commercially practice but a number of companies throughout the world. With exception of Neste, these companies provide little information on their operations.

Additional Remarks

As noted above Neste does provide some information on the performance of their renewable products division as part of their quarterly financial reporting [5]. The reported HVO revenue is their world-wide average value. The following figure is developed from those reports and benchmark price data on diesel fuel and biodiesel reported by the US Energy Information Administration [6] and Iowa State University [7].

Figure 4-3	HVO Selling Prices vs Biodiesel and Fossil Diesel

RD, biodiesel and ULSD represents selling prices for HVO, FAME biodiesel and ultra low sulphur diesel, respectively.

Quantitative Description

The quantitative data is presented below for the technology. The HVO chemistry is shown in the following figure [8].

Figure 4-4	HVO Chemistry

The oxygen from the triglyceride feedstock is removed as CO2 and some as H2O. Some of the feedstock is also converted to propane and because there is some cracking of the chains a portion of the liquid product is in the jet fuel or gasoline pool boiling range.

Energy/Technical Data

The quantitative data is presented below for the technology.

Typical Plant Size

There is almost an order of magnitude difference between the output of the smallest plant and the largest plants. It is assumed that the typical plant size will be in the middle of the range at 500,000 tonne per year of primary product (640 million litres per year).

Input and Output

The input and output data shown in the following table is the same information used for the energy balance shown in Figure 4-2. It is typical data and not representative of any specific plant. The output is one litre of HVO.

Table 4-1	Mass and Energy Flows HVO

|  | Inputs | Outputs |
| --- | --- | --- |
| Feedstock | 0.97 kg |  |
| Hydrogen | 0.035 kg |  |
| Natural gas | 0.27 MJ |  |
| Electricity | 0.085 kWh |  |
| Fuel gas |  | 0.03 kg |
| LPG |  | 0.025 kg |
| Naphtha |  | 0.06 kg |
| HVO (one litre) |  | 0.77 kg |

Forced and Planned Outage

Neste report their quarterly sale rates. Since the last quarter of 2011, the plants in Finland, Singapore, and Rotterdam have been operational with an original nameplate capacity of 500,000 tonne per quarter. Neste now claims a quarterly production capacity of 650,000 tonnes. The sales performance is shown in the following figure and it is assumed that the production rates closely follow the sales rates. Since 2013 the plants have operated above the original nameplate capacity indicating minimal unplanned outages.

Figure 4-5	Quarterly Performance – Neste HVO

Neste report that the plants undergo a major turnaround lasting 8 weeks every four to five years. This is equivalent to 15 days per year.

Technical Lifetime

Plant lifetimes should exceed 25 years, similar to what would be expected in a petroleum refinery.

Construction Time

Construction time of 24 to 30 months should be expected. This is longer than a FAME plant but the HVO plants are generally larger and have process vessels that operate at higher pressures and temperatures with an expectation of longer fabrication periods.

Financial Data

Financial data from public data sources is presented below. Where primary public data is not available two peer reviewed techno-economic analyses have been used [9] [10].

Investment Costs

Diamond Green Diesel is undergoing a 330,000 tonne/year expansion in Louisiana. The reported cost is $190 million (US) [10]. They reported that this was about 50% of the cost of a greenfield plant.

Converting this cost to Europe, the location factor, exchange rate, and greenfield factor is applied as shown below.

US capital cost*greenfield factor*currency factor*Location factor

$190,000,000*2.0*0.85*1.21=€390 million

It is estimated that a 500,000 tonne per year plant would have a capital cost of €550 million using a scaling factor of 0.8. This €0.86/litre.

The EU Sub Group on Advanced Biofuels [12] reported that the capital costs for a 500,000 tonne/year plants would be between 192 and 577 million Euros. The Diamond Green data would be at the upper end of the range.

A one million tonne per year plant is expected to cost 20.6 times (1.5) that of the 500,000 tonne per year plant or 0.65 €/litre. This value is used for the 2050 capital cost. The same approach is used for the 750,000 tonne plant in 2030.

Operating and Maintenance Costs

Neste publishes some information on their production costs in their quarterly financial reports. The available information for 2016 is shown in the following table.

Table 4-2	HVO Operating Costs

| Item | Value |  |
| --- | --- | --- |
|  | €/tonne | €/litre |
| Revenue, | 1,211 | 0.93 |
| Variable production costs | 117 | 0.09 |
| Fixed Costs | 58 | 0.04 |
| Depreciation | 50 | 0.04 |
| EBIT | 21 | 0.02 |
| Implied Feedstock costs | 876 | 0.67 |

Operating costs are not expected to change over time due to the high efficiency level of the current technology. The variable operating costs are reduced by 0.01 €/litre to account for the energy costs that would be included in the reported variable costs.

Start-up Costs

The start-up costs are included in the Neste financial information.

Technology Specific Data

The physical properties of HVO are shown in the following table [13].

Table 4-3	HVO Properties

| Property | Value |
| --- | --- |
| Density, kg/m3 | 770 |
| LHV, MJ/kg | 44.1 |
| LHV, MJ/litre | 34.4 |
| Oxygen content | 0 |
| Cetane number | >70 |
| Cloud point, C | As low as -40 °C |

Summary

The available data is summarized in the following table.

Table 4-4	HVO Datasheet

| Technology | HVO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2030 | 2050 | Uncertainty (2020) |  | Uncertainty (2050) |  | Note | Ref |
|  |  |  |  |  | Lower | Upper | Lower | Upper |  |  |
| Energy/technical data |  |  |  |  |  |  |  |  |  |  |
| Typical total plant size 1,000 t HVO/year | 500 | 500 | 750 | 1000 | 50% | 200% | 50% | 100% | A, B |  |
| Typical total plant size MW HVO | 675 | 675 | 850 | 1125 | 50% | 200% | 50% | 100% | A, B |  |
| Inputs |  |  |  |  |  |  |  |  |  |  |
| Feedstock Consumption, MWh/MWh Total Inputs | 0.881 | 0.881 | 0.881 | 0.881 | 99% | 101% | 99% | 101% | C | 3,4,11 |
| Hydrogen Consumption, MWh/MWh Total Inputs | 0.105 | 0.105 | 0.105 | 0.105 | 93% | 107% | 93% | 107% | C, D | 3,4,11 |
| Electricity Consumption, MWh/MWh Total Inputs | 0.008 | 0.008 | 0.008 | 0.008 | 50% | 150% | 50% | 150% | C | 3,4,11 |
| Natural Gas Consumption, MWh/MWh Total Inputs | 0.007 | 0.007 | 0.007 | 0.007 | 50% | 50% | 50% | 50% | C | 3,4,11 |
| Outputs |  |  |  |  |  |  |  |  |  |  |
| HVO Output, MWh/MWh Total Input | 0.85 | 0.85 | 0.85 | 0.85 | 99% | 101% | 99% | 101% | C, H |  |
| Co-products Output, MWh/MWh Total Input | 0.13 | 0.13 | 0.13 | 0.13 | 99% | 101% | 99% | 101% | C, H |  |
| Forced outage (%) | 0 | 0 | 0 | 0 |  |  |  |  | I |  |
| Planned outage (weeks per year) | 2 |  |  |  |  |  |  |  |  | 7 |
| Technical lifetime (years) | 25 |  |  |  |  |  |  |  |  |  |
| Construction time (years) | 2.0 |  |  |  |  |  |  |  |  |  |
| Financial data |  |  |  |  |  |  |  |  |  |  |
| Specific investment (€ /l HVO) | 0.86 | 0.85 | 0.72 | 0.65 | 0.5 | 0.8 | 0.5 | 0.8 | E, J | 10, 12 |
| - equipment (%) | 75 | 75 | 75 | 75 |  |  |  |  |  |  |
| -installation (%) | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| Fixed O&M (€ /l HVO) | 0.04 | 0.04 | 0.04 | 0.04 | 90% | 110% | 90% | 110% | F, K | 7 |
| Variable O&M (€ /l HVO) | 0.08 | 0.08 | 0.08 | 0.08 | 90% | 110% | 90% | 110% | F, K | 7 |
| Startup (€ /l HVO) | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Technology specific data |  |  |  |  |  |  |  |  |  |  |

Notes:

- The plant size is assumed based on the plants sizes of existing HVO plants.

- The plant size uncertainty represents the current range in the market.

- Performance is typical based on public data and confidential data reviewed by (S&T)2 Consultants Inc.

- Hydrogen consumption is also a function of the feedstock to a small degree.

- The total capital cost include total installed cost (total direct costs) and all indirect costs such as engineering, construction, contractor’s fee, contingency and working capital.

- O&M costs, costs for main raw materials are not included.

- There is limited potential for process improvements.

- Total output excludes the heat loss.

- Best plants operate at greater than 95% of capacity.

- Range of capital cost is +/- 25% of mean.

- Operating cost uncertainty is +/- 10% of mean.

References

- Neste. 2013. North American Tallow Pathway Description. https://www.arb.ca.gov/fuels/lcfs/2a2b/apps/nes-na-tallow-rpt-011414.pdf

- US EPA. 2017. EPA Approval for REG Geismar. https://www.epa.gov/sites/production/files/2017-05/documents/reg-geismar-deter-ltr-2017-04-13.pdf

- Kalnes, T et al. 2012. Green diesel production by hydrorefining renewable feedstocks. https://www.uop.com/wp-content/uploads/2012/12/UOP-Hydrorefining-Green-Diesel-Tech-Paper.pdf

- de Jong, S., Hoefnagels, R., Faaij, A., Slade, R., Mawhood, R. and Junginger, M. 2015. The feasibility of short-term production strategies for renewable jet fuels – a comprehensive techno-economic comparison. Biofuels, Bioprod. Bioref., 9: 778–800. https://www.doi.org/10.1002/bbb.1613

- Neste Financial Statements. https://www.neste.com/en/corporate-info/investors/financials

- US Energy Information Administration. New York Harbour ULSD Price. https://www.eia.gov/dnav/pet/pet_pri_spt_s1_d.htm

- Iowa State University. 2017. Biodiesel Profitability. http://www.extension.iastate.edu/agdm/energy/xls/d1-15biodieselprofitability.xlsx

- Rogelio Sotelo-Boyás, Fernando Trejo-Zárraga and Felipe de Jesús Hernández-Loyo (2012). Hydroconversion of Triglycerides into Green Liquid Fuels, Hydrogenation, Prof. Iyad Karamé (Ed.), InTech, DOI: 10.5772/48710. Available from: https://www.intechopen.com/books/hydrogenation/hydroconversion-of-triglycerides-into-green-liquid-fuels

- Tao, L., Milbrandt, A., Zhang, Y. and Wang, W. 2017. Techno-economic and resource analysis of hydroprocessed renewable jet fuel. Biotechnology for Biofuels 2017 10:261. https://doi.org/10.1186/s13068-017-0945-3

- Miller, P., Kumar, A. 2014. Techno-economic assessment of hydrogenation-derived renewable diesel production from canola and camelina, In Sustainable Energy Technologies and Assessments, Volume 6, 2014, Pages 105-115, ISSN 2213-1388, https://doi.org/10.1016/j.seta.2014.01.008

- Diamond Green Diesel. Press release. 2016. https://www.prnewswire.com/news-releases/diamond-green-diesel-to-be-expanded-to-275-million-gallons-annually-300248085.html

- Neste Renewable Diesel Handbook. https://www.neste.com/sites/default/files/attachments/neste_renewable_diesel_handbook.pdf

- Sub Group on Advanced Biofuels. 2017. Cost of Biofuel. http://platformduurzamebiobrandstoffen.nl/wp-content/uploads/2017/07/2017_SGAB_Cost-of-Biofuels.pdf

HVO Jet Fuel

Qualitative Description

This technology is very similar to the HVO renewable diesel process with one exception, in addition to the isomerization step there is a hydro-cracking step to produce products in the jet fuel boiling range rather than the diesel fuel range. This can be seen in the following figure which is a comparison of the UOP process diagrams for both technologies.

Figure 5-1	HVO Diesel vs Jet Process

There is one dedicated HVO jet facility in the world. It is the 130,000 tonne per year AltAir plant in Los Angeles, California. It employs the UOP process shown above. Detailed information on the AltAir operations is not publicly available and thus there is more reliance on secondary information in the description than there was for the previous section that considered the production of renewable diesel fuel.

This technology would be classified as being between the Pioneering and the early commercial phase.

Contact Information

Don O’Connor – (S&T)2 Consultants Inc.

Brief Technology Description

To meet the jet fuel specification, the produced bio-jet fuel has to have not only a high flash point, but also good cold flow properties. Therefore, it is required to hydro-isomerize and hydrocrack the normal paraffins produced from deoxygenation to a product with carbon chains ranging from C9 to C15. The hydrocracking stage results in some yield loss of jet and diesel fuel with increased production of naphtha and fuel gas.

The AltAir facility produces Renewable F-76 (a marine distillate fuel) and JP-5 (a military jet fuel) for the US military and Renewable Jet, Diesel, and Gasoline [1]. The company has not provided information on what percentage of each fuel is produced. The facility is using animal fats and used cooking oil as feedstocks. The technology should work for any of the same oils and fats used for renewable diesel fuels.

Compared to the HVO for diesel fuel described in the previous section, the yield of jet and diesel fuel is expected to be lower and the hydrogen demand higher when jet fuel is produced as a result of the additional hydrocracking step.

Input

The primary inputs to the production process are the fats and oils and hydrogen, along with some electricity, thermal energy, and chemicals.

Output

The outputs from the process are the jet fuel, some diesel fuel, some fuel gas (a mixture of hydrogen and light hydrocarbons), propane, and a light liquid stream that can be used as fuel or as a renewable gasoline blending component.

Energy Balance

Several of the analysis of HVO Jet fuel have used the GREET model data for the energy balance [2] [3]. Other studies have investigated the optimization of the process and reported significant process variability [4] [5].

The energy balance using the GREET information is shown in the following figure.

Figure 5-2	HVO Jet Fuel Energy Balance

As with the HVO technology in the previous chapter it is likely that 15% of the heat loss can be recovered as a high temperature stream that could be used for district heating.

The product yields for two feedstocks in the Tao paper [4], which are derived from Aspen modelling, are very different than the GREET product yields as shown in the following figure. Different product yields will impact the energy balance for the process.

Figure 5-3	Comparison of Product Yields

An earlier analysis of the process published in 2010 also showed much lower yields than is used in GREET [6]. This report was based on a theoretical analysis of the process. The energy balance from this work is shown in the following figure.

Figure 5-4	Alternative HVO Jet Energy Balance

Typical Capacities

Demand for biojet fuel is potentially much larger than the capacity of the single plant supplying the product. The one existing plant is also small compared to the HVO plants producing diesel fuel. There is no reason why the typical plant capacities won’t be similar to the HVO for diesel plants, 150 to 1,000 million litres per year.

Regulation Ability

Very little information is available on the turn down ratios of the process. It is a process with a high operating temperature pressure and thus will likely have limited capacity to regulate throughputs.

Space Requirements

The AltAir plant is a re-purposed oil refinery so the space requirements for this facility are not likely to be representative of greenfield plants.

The Neste HVO plants in Singapore and Rotterdam and the Diamond Green Diesel and REG plants in Louisiana are greenfield plants that have some integration with nearby utility suppliers. The space required for the plants are in the 0.02 to 0.025 ha/million litres of production range (220 m2/MW). This is likely to be the same space requirements of a HVO jet plant.

Advantages/Disadvantages

HVO Jet Fuel is the only commercial process for the production of renewable jet fuel. It is fully approved up to 50% blend level with fossil jet fuel. Most of the commercial biojet flights operating in the world today use fuel from this process.

Due to lower yields and higher energy and hydrogen consumption the production costs of HVO Jet will be higher than HVO diesel fuel, which are already higher than the fossil fuel prices.

Environment

Based on the information in the peer reviewed literature the jet fuel production is more energy intensive than the production of HVO diesel fuel.

The hydrotreating catalysts have a long life and can be re-generated. There is limited waste generated as part of the process, although this can depend on the feedstock and the need to pre-treat the feedstock to remove minor contaminants.

Research and Development Perspective

Hydrogenated vegetable oil (HVO) Jet Fuel has been classed as a Category 3 technology, a commercial technology with development potential. There is one production facility in operation that is dedicated to maximize jet fuel. These technologies are deemed to have a certain development potential such as opportunities for optimizing jet fuel production with different feedstocks or possibly feedstock blends and therefore there is a considerable level of uncertainty related to future price and performance. On the other hand the process steps that are used are all well know refinery process units which limits the potential improvements.

Examples of Market Standard Technology

The leading commercial supplier of the technology is Honeywell UOP. Their corporate office is:

UOP World Corporate Offices
25 E. Algonquin Road (Bldg A)
P.O. Box 5017
Des Plaines, IL 60017-5017
Toll Free: +1 (800) 877-6184
Phone: +1 (847) 391-2000

They have a European office at

Noorderlaan 147
B-2030 Antwerp, Belgium
Phone: +32-3-540-9911

Predication of Performance and Cost

The production of jet fuel from fats and oils is being practised commercially in one facility, however very little information on the actual performance is available in the public domain. Therefore it is uncertain where the technology sits on the development curve.

Uncertainty

There are relatively high levels of uncertainty with the performance parameters for this technology as the data in the public domain is not based on the performance of the one operating plant. That information is confidential. The information that is in the public domain from peer reviewed studies is sometimes contradictory especially with respect to product yields and energy consumption.

Additional Remarks

It is possible to produce some jet fuel from HVO plants that focus on the production of diesel fuel. There is a portion of the HVO that can be recovered by distillation that will meet the fuel specifications of the jet fuel. This is the “light” end of the diesel HVO and removing it makes the remaining product heavier. However, if there is a demand for diesel and jet fuel it is one way to produce HVO Jet without a loss of yield and large amounts of LPG or naphtha materials being produced. The distillation curves for three commercial renewable diesel fuels are compared to the typical upper and lower limit for jet fuel in the following figure. It can be seen that 10 to 20% of the HVO diesel fuel is within the jet fuel range. There is no quantitative public information on the flexibility of the operations to alter the diesel/jet fuel ratios.

Figure 5-5	Renewable Diesel Distillation Curves

Quantitative Description

There is very little actual data available on this process in the public domain. The information in the peer reviewed literature is often contradictory so much of the information presented below has a high degree of uncertainty.

Energy/Technical Data

The energy and technical data on the process is presented below.

Typical Plant Size

The output of the primary product will be 20 to 40% lower in a jet HVO plant compared to one that produced diesel fuel. The co-product output however will be higher. It is assumed that the typical plant size will be 400,000 tonnes, 80% of the typical diesel HVO plant reflecting the lower primary product yield.

Input and Output

The estimated input and output is shown in the following table. This is based on the GREET model parameters which were used for Figure 5-2. The output is one litre of HVO jet fuel.

Table 5-1	Mass and Energy Flows HVO Jet

|  | Inputs | Outputs |
| --- | --- | --- |
| Feedstock | 1.07 kg |  |
| Hydrogen | 0.035 kg |  |
| Natural gas | 6.06 MJ |  |
| Electricity | 0.048 kWh |  |
| LPG |  | 0.142 kg |
| Naphtha |  | 0.097 kg |
| HVO Jet (one litre) |  | 0.77 kg |

It has been assumed that no changes to the inputs and outputs are achieved over the present technology. The data in the data represents a very good mass balance with limited opportunity for improvement.

Forced and Planned Outage

HVO jet fuel plants should operate in a similar manner to HVO diesel fuel plants. Forced shutdowns should be minimal and longer turnaround should happen every four years with an annual rate of 15 days per year.

Technical Lifetime

Plant lifetimes should exceed 25years, similar to what would be expected in a petroleum refinery.

Construction Time

Construction time of 24 to 30 months should be expected. This is longer than a FAME plant but the HVO plants are generally larger and have process vessels that operate at higher pressures and temperatures with an expectation of longer fabrication periods.

Financial Data

The financial data has been estimated based on the available information for HVO plants and the process differences to produce jet fuel instead of diesel fuel.

Investment Costs

Investment costs are 25% higher than the HVO plant due to the lower yield of the primary product. It is estimated that a 400,000 tonne per year plant would have a capital cost of €550 million. This is the same cost but lower throughput than used for the HVO plant. This is €1.06/litre.

De Jong [7] reported a range of capital costs of €200 to 644 million for a 350,000 tpy HVO Jet plant. This is €0.57 to €1.84 per litre.

The future investment cost is lower due to the increased scale of the plants, the same approach is used as was used for the HVO plants, a scaling factor of 0.6 is applied to the ratio of the plant size in the future to the current plant size.

Operating and Maintenance Costs

The operating and maintenance costs will be allocated across the jet fuel and the co-products. It is assumed that they will be the same per litre as were determined for the HVO plant.

The fixed O&M costs are €0.04 per litre and the variable O&M costs are €0.08 per litre. These costs are held constant for the future years.

Start-up Costs

The start-up costs are included in the operating costs.

Technology Specific Data

The physical properties of HVO Jet are shown in the following table.

Table 5-2	HVO Jet Properties

| Property | Value |
| --- | --- |
| Density, kg/m3 | 770 |
| LHV, MJ/kg | 44.0 |
| LHV, MJ/litre | 34.4 |
| Oxygen content | 0 |
| Freezing point, C | -63 °C |

Summary

The quantitative data for HVO jet fuel is summarized in the following table.

Table 5-3	HVO Jet Fuel Datasheet

| Technology | HVO Jet Fuel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2030 | 2050 | Uncertainty (2020) |  | Uncertainty (2050) |  | Note | Ref |
|  |  |  |  |  | Lower | Upper | Lower | Upper |  |  |
| Energy/technical data |  |  |  |  |  |  |  |  |  |  |
| Typical total plant size 1,000 t HVO/year | 400 | 400 | 600 | 800 | 50% | 200% | 50% | 100% | A, B |  |
| Typical total plant size MW HVO | 540 | 540 | 810 | 1080 | 50% | 200% | 50% | 100% | A, B |  |
| Inputs |  |  |  |  |  |  |  |  |  |  |
| Feedstock Consumption, MWh/MWh Total Inputs | 0.779 | 0.779 | 0.779 | 0.779 | 90% | 110% | 90% | 110% | C, G | 2,3 |
| Hydrogen Consumption, MWh/MWh Total Inputs | 0.099 | 0.099 | 0.099 | 0.099 | 90% | 110% | 90% | 110% | C,D, G | 2,3 |
| Electricity Consumption, MWh/MWh Total Inputs | 0.003 | 0.003 | 0.003 | 0.003 | 90% | 110% | 90% | 110% | C, G | 2,3 |
| Natural Gas Consumption, MWh/MWh Total Inputs | 0.119 | 0.119 | 0.119 | 0.119 | 90% | 110% | 90% | 110% | C, G | 2,3 |
| Outputs |  |  |  |  |  |  |  |  |  |  |
| HVO Jet Output, GJ/GJ Total Input | 0.66 | 0.66 | 0.66 | 0.66 | 90% | 110% | 90% | 110% | C, H |  |
| Co-products Output, GJ/GJ Total Input | 0.16 | 0.16 | 0.16 | 0.16 | 90% | 110% | 90% | 110% | C, H |  |
| Forced outage (%) | 0 | 0 | 0 | 0 |  |  |  |  | I |  |
| Planned outage (weeks per year) | 2 |  |  |  |  |  |  |  |  |  |
| Technical lifetime (years) | 25 |  |  |  |  |  |  |  |  |  |
| Construction time (years) | 2.0 |  |  |  |  |  |  |  |  |  |
| Financial data |  |  |  |  |  |  |  |  |  |  |
| Specific investment (€ /l HVO Jet) | 1.07 | 1.07 | 0.91 | 0.80 | 0.5 | 0.8 | 0.5 | 0.8 | A, E, J |  |
| - equipment (%) | 75 | 75 | 75 | 75 |  |  |  |  |  |  |
| -installation (%) | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| Fixed O&M (€ /l HVO Jet and co-products) | 0.04 | 0.04 | 0.04 | 0.04 | 90% | 110% | 90% | 110% | F, K |  |
| Variable O&M (€ /l HVO Jet and co-products) | 0.08 | 0.08 | 0.08 | 0.08 | 90% | 110% | 90% | 110% | F, K |  |
| Startup (€ /l HVO Jet) | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Technology specific data |  |  |  |  |  |  |  |  |  |  |

Notes:

- The plant size is assumed based on the plants sizes of existing HVO plants. Jet production is 80% of HVO diesel production with the other 20% representing increased co-products.

- The plant size uncertainty represents the current range in the market.

- Performance is typical based similar performance to HVO diesel but with less primary product and more co-products.

- Hydrogen consumption is also a function of the feedstock to a small degree.

- The total capital cost include total installed cost (total direct costs) and all indirect costs such as engineering, construction, contractor’s fee, contingency and working capital.

- O&M costs, costs for main raw materials are not included.

- There is limited potential for process improvements.

- Total output excludes the heat loss.

- Best plants operate at greater than 95% of capacity.

- Range of capital cost is +/- 25% of mean.

- Operating cost uncertainty is +/- 10% of mean.

References

- AltAir. 2017. The Department of Energy: Wet and Gaseous Feedstocks: Barriers and Opportunities. https://energy.gov/sites/prod/files/2017/07/f35/BETO_2017WTE-Workshop_BryanSherbacow-AltAir.pdf

- Antonissen, K. 2016. Greenhouse gas performance of renewable jet fuel: a comparison of conversion pathways. https://dspace.library.uu.nl/bitstream/handle/1874/347665/MSc%20Thesis%20K.Y%20Antonissen%2c%20May%202016%20%281%29.pdf?sequence=2&isAllowed=y

- de Jong, S., Antonissen, K., Hoefnagels, R., Lonza, L., Wang, M., Faaij, A., and Junginger, M. 2017. Life-cycle analysis of greenhouse gas emissions from renewable jet fuel production. Biotechnology for Biofuels201710:64. https://doi.org/10.1186/s13068-017-0739-7

- Tao, L., Milbrandt, A., Zhang, Y., and Wang, W. 2017. Techno‑economic and resource analysis of hydroprocessed renewable jet fuel. Biotechnology for Biofuels. 10:261. https://doi.org/10.1186/s13068-017-0945-3

- Starck, L., Pidol, L., Jeuland, N., Chapus, T., Bogers, P., Bauldreay, J. 2016. Production of Hydroprocessed Esters and Fatty Acids (HEFA) - Optimization of Process Yield. Oil Gas Sci. Technol. - Rev. IFP Energies nouvelles. Volume 71, Number 1, January-February 2016. https://doi.org/10.2516/ogst/2014007

- Stratton et al. 2010. Life Cycle Greenhouse Gas Emissions from Alternative Jet Fuels. http://web.mit.edu/aeroastro/partner/reports/proj28/partner-proj28-2010-001.pdf

- de Jong, S., Hoefnagels, R., Faaij, A., Slade, R., Mawhood, R. and Junginger, M. 2015. The feasibility of short-term production strategies for renewable jet fuels – a comprehensive techno-economic comparison. Biofuels, Bioprod. Bioref., 9: 778–800. https://www.doi.org/10.1002/bbb.1613

1st Generation Ethanol

Qualitative Description

Figure 6-1	1st Generation Ethanol Production Process

Contact Information

Don O’Connor – (S&T)2 Consultants Inc.

Brief Technology Description

Cereal crops, with corn and wheat being the most common, are milled to reduce the particle size. The milled grain is mixed with water and a liquefying enzyme and heated to 90 to 100 °C to liquefy the starch in the grain. The slurry is cooled to 30 to 35 °C, a saccharification enzyme to convert the starch to fermentable sugars and yeast is added to ferment the sugars to ethanol and carbon dioxide. After about 48 hours the “beer” is sent to distillation columns to separate the ethanol and some of the water from the remaining solids. The ethanol stream has the remaining water removed in a molecular sieve or an anhydrous distillation column. The ethanol is suitable for fuel blending at this stage. The distillation bottom columns are centrifuge to separate some of the solids from the liquid. The solids can be sold or sent to a dryer. The liquid can have corn oil removed and is then concentrated through a vacuum evaporation system to produce a syrup which can also be sold or sent to the dryer. The distillers’ grains are used as a high protein animal feed.

Input

The primary inputs into the system are the corn or wheat feedstock, water, power and thermal energy. There are secondary inputs of enzymes, yeast, and acids and bases for pH adjustment.

Output

The primary outputs are ethanol and distillers’ grains. The distillers’ grains can be sold wet or dry, and either combined (where the solids and syrup are combined) or separately (where the solids and syrup streams are sold as individual products. Corn plants can also separate a small volume of corn oil which can be used for animal feed or as a FAME feedstock. The corn oil can have a much higher value when sold separately instead of as part of the distillers’ grains. The oil content of wheat is too low make the extraction practical.

Energy Balance

The energy balance of a 1st generation corn ethanol plant is shown in the following figure. This figure is based on an ethanol yield of 415 litres per tonne of corn. The ratio of the energy of ethanol to Distillers’ Dried Grains DDG will vary with the starch content of the feedstock. Wheat generally has a slightly lower ethanol output but higher DDG output. The natural gas and power requirements are typical values for a corn ethanol plant that dries all of the distillers’ grains. Wheat plants will use more energy than corn plants as more DDG is produced that must be dried and the evaporation systems are generally not as efficient as the viscosity of the syrup is higher for a given solids content.

Figure 6-2	1st Generation Ethanol Energy Balance

Most of the unit operations in the process are undertaken at relatively low temperatures so the heat losses are low quality. Most of the heat is lost either through a cooling tower or in the exhaust stack of the distillers’ grain dryer. A few plants have started to recover some of the latent heat of vapourization in the dryer exhaust and use the energy to reduce the purchased natural gas. The practice is not yet widespread.

Typical Capacities

In North America, the most recent new plants were built with 400 million litres of ethanol production capacity per year (320,000 tonnes ethanol per year, 280 MW). These plants generally had two production trains of 200 million litres per year each (160,000 tonnes ethanol per year).

Twenty-five years ago plants were typically 60 million litres per year. Over time capacities grew to take advantage of economies of scale.

Regulation Ability

Plants generally operate continuously with one or two shutdowns per year for maintenance. The process is biological and it is important to minimize the possibility of infections that can reduce yield and negatively impact the performance of equipment. This limits the potential for operations below about 80% of nameplate capacity. Lower production rates can only be achieved with more frequent start-up and shutdown cycles which increase the operating costs.

Space Requirements

Pannonia Ethanol in Hungary is a 430 million litre per year corn ethanol facility designed and built by the American company Fagen. It covers 22 ha on the banks of the Danube River. It was a greenfield plant.

The Vivergo Fuels 420 million litre per year wheat ethanol plant in the UK occupies 10 ha in an existing industrial complex. The site was chosen due to its existing infrastructure and utilities.

Space requirements will range from 0.024 to 0.052 ha/million litres (350 to 775 m2/M. Smaller plants may require more land per million litres than larger plants. The degree of integration with other infrastructure will also impact the land requirements.

Advantages/Disadvantages

1st generation ethanol is a well proven technology. It is the largest volume renewable fuel produced in the world today. Ethanol is a high octane fuel that can be used to increase the octane of the gasoline that it is blended with. This has the potential to reduce the GHG emissions of the petroleum refinery and increase the refinery energy efficiency, factors that are not often considered when the benefits of ethanol are discussed.

Ethanol does contain about 35% oxygen which limits the quantity that can be blended with gasoline. In Denmark, ethanol is blended at the 5% by volume level [1]. Europe, ethanol blended at the 10% by volume level is a relatively recent development. E10 is currently available in Belgium, Finland, France and Germany [2].

Environment

The lifecycle GHG emissions of 1st generation ethanol are about equally split between the feedstock production and fuel production [3]. Feedstock emissions are geographic specific and can vary significantly from country to country due to production practices.

Research and Development Perspective

First generation ethanol produced from corn and wheat is a category 4 technology; a commercial technology with large scale deployment. The price and performance of the technology today is well known and normally only incremental improvements over time would be expected. Therefore, the future price and performance may also be projected with a relatively high level of certainty.

Examples of Market Standard Technology

There are many suppliers of 1st generation ethanol technology. More than 50% of the US production capacity was designed by ICM Inc. They have also designed plants in Europe, Brazil, Argentina, and Africa.

ICM, Inc. 
P.O. Box 397
310 N. First Street
Colwich, KS 67030-0397

The German company GEA is one of the largest suppliers of process technology for the food industry and a wide range of other industries, including fuel ethanol. They also have expertise with respect to wheat starch production.

GEA Group Aktiengesellschaft

Peter-Müller-Str. 12

40468 Düsseldorf

Germany

Predication of Performance and Cost

The corn ethanol industry has been studied from the perspective of the experience curve [4] [5]. There have been significant improvements in the performance of the technology over time and the improvements are in line with the expectations based on the production growth of the industry.

Uncertainty

Given the level of maturity of the industry there is a low level of uncertainty associated with information collected on the technology.

Additional Remarks

The world starch ethanol production is shown in the following figure [6]. The United States is the dominant producer and that is almost all produced from corn. Production in Canada and Europe is a mixture of wheat and corn plants.

Figure 6-3	World Starch Ethanol Production

Production levels in the post 2010 period have not increased as rapidly as they did in the previous decade, as a result the rate of change in the technology has slowed down on a year over year basis since it takes longer to double the production in this decade than it did in the last decade.

Quantitative Description

There is a significant amount of quantitative information available for this technology since it is the largest renewable pathway in the world.

The process involves two chemical reactions. In the first, starch is hydrolyzed to glucose as shown below.

(C6H10O5)n + nH2O → nC6H12O6

Starch        Water      Glucose

The glucose is then fermented with yeast to ethanol and carbon dioxide.

C6H12O6        →     2C2H5OH    +      2CO2

Glucose     Ethanol     Carbon Dioxide

Energy/Technical Data

The energy and technical data for 1st generation ethanol plants is presented below.

Typical Plant Size

Plant sizes can range up to 400 million litres per year (300 MW), although plants of this size often have two production trains. The dryers for the plants are often the limiting size of the production train, although the size of individual fermenters can also be a limiting factor. Most plants operate with four fermenters and a 48 to 60 hour fermentation cycle so fermenters can be quite large. The average plant size in Europe is smaller than this.

In their 2014 report on the industry, ePure [7] reported the production capacity of beverage and fuel ethanol plants and the number of plants by country. The fuel ethanol production capacity was reported to be 7 billion litres. The fuel ethanol industry apparently operated at 65% of capacity in 2014. That information along with the fuel ethanol production reported by Eurostat for 2014 is shown in the following table. Not all of these plants are currently operating.

Table 6-1	European Ethanol Plants

| Country | Production Capacity | No. Plants | Avg Size | 2014 Fuel Production |
| --- | --- | --- | --- | --- |
|  | Million litres |  | Million Litres |  |
| France | 2,300 | 19 | 121 | 975 |
| Germany | 1,400 | 12 | 117 | 897 |
| United Kingdom | 900 | 5 | 180 | 519 |
| Poland | 750 | 14 | 54 | 181 |
| Spain | 600 | 5 | 120 | 486 |
| The Netherlands | 575 | 2 | 288 | 0 |
| Hungary | 520 | 3 | 173 | 372 |
| Belgium | 500 | 3 | 167 | 325 |
| Czech Republic | 350 | 6 | 58 | 132 |
| Italy | 300 | 6 | 50 | 1 |
| Sweden | 275 | 6 | 46 | 175 |
| Austria | 250 | 2 | 125 | 262 |
| Slovakia | 240 | 1 | 240 | 134 |
| Romania | 200 | 3 | 67 | 15 |
| Lithuania | 100 | 2 | 50 | 13 |
| Latvia | 50 | 2 | 25 | 0 |
| Bulgaria | 50 | 2 | 25 | 27 |
| Finland | 50 | 4 | 13 | 24 |
| Ireland | 40 | 1 | 40 | 0 |
| Denmark | 30 | 1 | 30 | 0 |
| Total | 9,480 | 99 | 96 | 4,538 |

ePure reported that 31% of the feedstock in 2016 was corn, 32% was wheat and the remainder was sugar based and other cereals [8]. Ethanol production from corn in 2016 was 1.61 billion litres and from wheat was 1.66 billion litres.

Input and Output

The primary input is the grain feedstock and the electric power and thermal energy. The primary output is the ethanol, the DDG for animal feed, and some plants capture the CO2 from the fermentation vessels for use in the industrial gas market. The inputs and outputs are shown in the following table.

Table 6-2	Corn Ethanol Inputs and Outputs

| Parameters | Input | Output |
| --- | --- | --- |
| Feedstock | 2.4 kg |  |
| Power | 0.20 kWh |  |
| Natural gas | 7 MJ |  |
| Ethanol |  | 1 litre |
| DDG |  | 0.72 kg |
| CO2 (if captured) |  | 0.75 kg |

The inputs and outputs for a wheat plant are slightly different since wheat usually has a lower starch content than corn. Feedstock consumed may be 2.6 kg/litre of ethanol and the DDG production may be 0.94 kg/litre. The energy consumption may be 9 to 10 MJ of natural gas per litre of ethanol. There can be variations in the feedstock starch contents from year to year and since wheat is a winter crop and corn is planted in the spring for a fall harvest, the ratio of the starch contents of corn and wheat can vary from year to year.

Future improvements can be expected in ethanol yield and energy use. In the US the ethanol yield has been increasing at about 0.2% per year. This is expected to continue. Natural gas energy use has been declining by 1.8% per year. This rate is expected to start to decline as the time to double production increases. It has been assumed that gas consumption declines by 20% over the next 30 years. Electric power consumption has shown little improvement in the past decade, which is not that surprising as the power consumption tends to be more a function of the original design rather than operating practices. No change in power production is forecast.

Forced and Planned Outage

Ethanol plants are generally designed to operate 350 days per year with 15 days for maintenance. Some plants take a maintenance break once per year and others will take two shorter breaks per year.

Since the process is a generally a combination of batch fermentation with intermediate storage tanks at the front and back ends of the plant, individual components can often be taken offline for cleaning or maintenance without impacting production rates. The industry generally has very high production utilization if the markets for the products are available. The European industry generally operates at lower rates of capacity utilization because of the smaller and less mature market than in North America.

Technical Lifetime

Plant lifetimes should exceed 25 years, as the US and EU starch ethanol industries have demonstrated.

Construction Time

Plants can generally be constructed in 18 to 24 months. There are no very high pressures or temperatures involved in the process which reduces the required lead time for process components.

Financial Data

There is more financial data available for the North America industry than there is for the European industry. Other than feedstock costs and selling prices the other categories of financial costs should be relatively similar for the two industries.

As with the FAME industry, Iowa State University maintains a financial model and monthly operating data for a 380 million litre corn ethanol plant located in Iowa [9].

Investment Costs

There have been relatively few 1st generation ethanol plants built since 2010. So the capital cost data is not as current as it used to be.

The Iowa State model has capital costs of $181 million dollars excluding working capital. There are two corn ethanol plants that are under construction in the United States [10] [11]. One will produce 300 million litres and has a reported cost of $150 million [12]. This included $10 million in working capital. The other has a capacity of 450 million litres and has a cost of $190 million [11].

Table 6-3	Capital Costs

|  | Iowa State | Ringneck Energy | Elite Octane |
| --- | --- | --- | --- |
| Capacity, million lpy | 380 | 300 | 450 |
| Capital cost, $ Million | 181 | 140 | 190 |
| $/annual litre | 0.48 | 0.47 | 0.42 |

The three cost estimates are quite close. Converting this data to a European value using the methodology provided in the Introduction, the capital cost for this large plant is estimated as follows;

$0.50/litre * 0.85 €/$ * 1.21 Europe factor = 0.51 €/litre.

Given the slow pace of new plant construction it is expected that the future capital costs are a function of the plant scale and a 0.8 scaling factor and not of technology improvements.

Operating and Maintenance Costs

The operating and maintenance costs in the Iowa State model are used as the basis for the data table. The 2016 data has been used and the power and natural gas costs have been adjusted to the Danish values reported in section 1.4. The capital cost has been adjusted to 206 million Euro to match the value above. The results for 2016 are shown in the following table.

Table 6-4	Production Costs 1st Generation Ethanol

| Parameter | Costs (Euro/litre) |
| --- | --- |
| Natural gas | 0.043 |
| Power | 0.012 |
| Other Variable | 0.049 |
| Fixed Costs | 0.048 |
| Total Costs ex feedstock | 0.152 |

Fixed operating costs are not expected to change over time but the variable costs decrease as the energy use improves.

Start-up Costs

Start-up costs are included in the previous table.

Technology Specific Data

The properties of ethanol are shown in the following table. The ethanol molecule contains about 35% oxygen which reduces the volumetric energy density of the fuel.

Table 6-5	Ethanol Properties

| Property | Value |
| --- | --- |
| Density, kg/m3 | 790 |
| LHV, MJ/kg | 26.9 |
| LHV, MJ/litre | 21.3 |
| Oxygen content | 35% |
| Blending Octane number | ~115 |
| Flash point, C | 13 |

Summary

The quantitative data for the 1st generation ethanol process are summarized in the following table.

Table 6-6	1st Generation Ethanol Datasheet

| Technology | 1st Generation Ethanol |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2030 | 2050 | Uncertainty (2020) |  | Uncertainty (2050) |  | Note | Ref |
|  |  |  |  |  | Lower | Upper | Lower | Upper |  |  |
| Energy/technical data |  |  |  |  |  |  |  |  |  |  |
| Ethanol/year, 1,000 tonnes/year | 320 | 320 | 400 | 500 | 50% | 120% | 60% | 100% | A, B | 11 |
| Ethanol/year, MW | 280 | 280 | 350 | 440 | 50% | 120% | 60% | 100% | A, B |  |
| Inputs |  |  |  |  |  |  |  |  |  |  |
| Feedstock Consumption, MWh/MWh Total Input | 0.83 | 0.83 | 0.84 | 0.85 | 98% | 105% | 98% | 102% | C |  |
| Electricity Consumption, MWh/MWh Total Input | 0.02 | 0.02 | 0.02 | 0.02 | 75% | 125% | 75% | 125% | D |  |
| Natural Gas Consumption, MWh/MWh Total Input | 0.15 | 0.15 | 0.14 | 0.13 | 75% | 125% | 75% | 125% | D |  |
| Outputs |  |  |  |  |  |  |  |  |  |  |
| Ethanol Output, MWh/MWh Total Inputs | 0.46 | 046 | 0.47 | 0.47 | 95% | 102% | 95% | 102% | E, F |  |
| DDG Output, MWh/MWh Total Input | 0.29 | 0.27 | 0.28 | 0.25 | 98% | 105% | 98% | 102% | E, F |  |
| Forced outage (%) | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Planned outage (weeks per year) | 2 |  |  |  |  |  |  |  |  |  |
| Technical lifetime (years) | 25 |  |  |  |  |  |  |  |  |  |
| Construction time (years) | 1.5 to 2.0 |  |  |  |  |  |  |  |  |  |
| Financial data |  |  |  |  |  |  |  |  |  |  |
| Specific investment (€ /l Ethanol) | 0.51 | 0.5 | 0.48 | 0.46 | 95% | 125% | 95% | 125% | I | 10 |
| - equipment (%) | 75% | 75% | 75% | 75% |  |  |  |  |  |  |
| -installation (%) | 25% | 25% | 25% | 25% |  |  |  |  |  |  |
| Fixed O&M (€ /l Ethanol) | 0.048 | 0.048 | 0.048 | 0.048 | 95% | 125% | 95% | 125% |  | 8 |
| Variable O&M (€ /l Ethanol) | 0.049 | 0049 | 0.049 | 0.049 | 90% | 110% | 90% | 110% | G, B | 8 |
| Startup (€ /l Ethanol) | 0 | 0 | 0 | 0 |  |  |  |  | H |  |
| Technology specific data |  |  |  |  |  |  |  |  |  |  |

Notes:

- The plant size is assumed based on the plants sizes of existing ethanol plants.

- The plant size uncertainty represents the current range in the market.

- The feedstock starch content impacts the yield. Ethanol yields in the United States have been slowly increasing from a combination of improved conversion efficiency and higher starch content of the feedstock.

- The upper and lower values are representative of the range between individual plants.

- Output excludes the low quality waste heat.

- Change over time is a function of increased ethanol yield, which also results in lower DDG yield as the mass must be conserved.

- Variable costs are reduced from improved energy efficiency.

- Start-up costs are included on variable costs.

- Capital costs can be site specific and process technology supplier dependent. The costs provided for in the table are from a technology developer with a reputation for low capital costs.

References

- Energy Statistics 2015. https://ens.dk/sites/ens.dk/files/Statistik/energy_statistics_2015.pdf

- ePure. 2017. Fuel Blends. http://epure.org/about-ethanol/fuel-market/fuel-blends/

- Biograce Version 4d without the extra 30% processing energy factor.

- Hettinga, W., Junginger, H., Dekker S., Hoogwijk, M., McAloon, A., Hicks, K. 2009. Understanding the reductions in US corn ethanol production costs: An experience curve approach. Energy Policy 37(2009)190–203. http://naldc.nal.usda.gov/download/22550/PDF

- IEA Bioenergy Task 39. 2009. Potential for Improving Carbon/Energy Balance of Bioethanol. http://task39.sites.olt.ubc.ca/files/2013/05/IEA_Bioenergy_Task-39_Improving_carbon_and_energy_balance.pdf

- US Energy Information Administration. 2015. International Energy Statistics.  Biofuels Production. http://www.eia.gov/cfapps/ipdbproject/IEDIndex3.cfm?tid=79&pid=79&aid=1

- ePure. 2014. State of the Industry Report 2014. http://epure.org/media/1137/state-of-the-industry-report-2014.pdf

- ePure. 2017. 2016 Industry Statistics.  http://epure.org/media/1610/2016-industry-statistics.pdf

- Iowa State University. 2017. Ethanol Profitability. http://www.extension.iastate.edu/agdm/energy/xls/d1-10ethanolprofitability.xlsx

- Capital Journal. Oct. 10, 2017.  Ringneck Energy pouring cement, building foundations for ethanol plant near Onida. http://www.capjournal.com/news/ringneck-energy-pouring-cement-building-foundations-for-ethanol-plant-near/article_4ee23aac-adec-11e7-80fb-a7814e744ba7.html

- Biofuel Journal. 2017. Third Quarter. Construction begins. Page 40-42.

- Ringneck Energy prospectus. 2016. http://ringneckenergy.com/wp-content/uploads/2016/12/2015_PPM_w_Exhibits.pdf

Pyrolysis Oils

  - Qualitative Description

Pyrolysis is the thermal decomposition of biomass occurring in the absence of oxygen. It is the fundamental chemical reaction that is the precursor of both the combustion and gasification processes. The products of biomass pyrolysis include biochar, bio-oil and gases including methane, hydrogen, carbon monoxide, and carbon dioxide.

A wide range of biomass feedstocks can be used in pyrolysis processes, however the pyrolysis process is very dependent on the moisture content of the feedstock, which should be around 10%.

Pyrolysis processes can be categorized as slow pyrolysis or fast pyrolysis. Fast pyrolysis is currently the most widely used pyrolysis system. Slow pyrolysis takes several hours to complete and results in biochar as the main product. On the other hand, fast pyrolysis yields 60% bio-oil and takes seconds for complete pyrolysis.

Pyrolysis can be performed at relatively small scale and at remote locations which enhance energy density of the biomass resource and reduce transport and handling costs.  Pyrolysis offers a flexible and attractive way of converting solid biomass into an easily stored and transported liquid, which can be successfully used for the production of heat, power and chemicals.

This section considers fast pyrolysis systems.

Contact Information

Don O’Connor – (S&T)2 Consultants Inc.

Brief Technology Description

Fast pyrolysis is a high temperature process in which biomass is rapidly heated in the absence of oxygen. As a result it decomposes to generate mostly vapours and aerosols and some charcoal. Liquid production requires very low vapour residence time to minimise secondary reactions of typically 1 s, although acceptable yields can be obtained at residence times of up to 5 s if the vapour temperature is kept below 400 °C. After cooling and condensation, a dark brown mobile liquid is formed which has a heating value about half that of conventional fuel oil. The basic process flow is shown in the following figure.

Figure 7-1	Fast Pyrolysis Process Flow

Input

The process inputs are dry biomass, some electricity is required to operate the process. The produced gas can be used to dry the feedstock or it could be used for power generation when dry feedstock is available.

Output

The primary output from the system is the bio-oil, and potentially some char and produced gas depending on the design of the system, operating conditions and the feedstocks used.

Energy Balance

The energy balance of the process is shown in the following figure [1]. This energy balance is based on laboratory scale operations and could change with feedstock and operating conditions. This system combusts the biochar that is produced to drive the reactions, producing just bio oil, steam and possibly electric power.

Figure 7-2	Fast Pyrolysis Oil Energy Balance

Typical Capacities

BTG Bioliquids operates a 5 T per hour (tph) fast pyrolysis system in Hengelo, The Netherlands [2]. The plant produces approximately 20 million litres per year of bio oil. In Canada, Ensyn is completing construction of a 8.3 tph plant in Port Cartier, Quebec [3]. Ensyn has several 16.7 tph projects in development around the world. The largest plants will produce approximately 75 million litres per year of bio oil.

Regulation Ability

The operating temperature of the reaction zone of the systems drives product yields and ratios of liquids to gas and char [4]. This is shown in the following figure.

Figure 7-3	Typical product Yields vs. Temperature

The regulation capacity of the systems is therefore limited. Operating outside of the optimal temperature zone will result in a loss of oil yield.

Space Requirements

Ensyn operates a 10 million litre per year facility in Renfrew Ontario. The plant is located on 6.4 ha of land. This is 0.64 ha/million litres. The BTG plant in Hengelo, which has a larger capacity occupies only 0.10 ha but has limited storage capacity for feedstock and finished products and is located in a less harsh climate.

Advantages/Disadvantages

The primary attractiveness of the technology is that biomass can be converted into a liquid fuel. The pyrolysis oil contains 30 to 50% oxygen and has a high water content and total acid number. It is not miscible in liquid hydrocarbons. It is used as a boiler fuel in North America and in Europe.

The energy density of the oil is higher than the biomass used to produce it and it can be more easily transported.

The characteristics of the oil can be improved by removing the oxygen either through hydrotreating (hydrodeoxygenation) or feeding the product into a fluid catalytic cracker or a hydrocracker at a petroleum refinery (decarboxylation).

Hydrodeoxygenation requires hydrogen but results in a higher yield. Single stage hydrotreating can be difficult to achieve with excess coking. Two stage hydrotreating and the use a co-solvent are options. Hydrogen consumption can be on the order of 15% by weight of the hydrotreated product.

Decarboxylation doesn’t require hydrogen but results in a lower liquid yield.

Environment

The GHG emission performance of the pyrolysis oil should be very good as most of the energy to drive the process comes from the biomass feedstock. The GHG emissions of the hydrotreated pyrolysis oil will be less attractive due to the hydrogen requirement but it will depend on the how the hydrogen is produced.

Research and Development Perspective

Fast pyrolysis of biomass to produce a bio-oil is a category 2 technology, a technology in the pioneer stage with limited applications at scale. There is significant uncertainty with respect to the performance and costs of the technology.

There is potential to improve yields and reduce costs as more experience with the technology is gained from the existing semi-commercial demonstration facilities and then the technology is scaled to commercial plants.

Examples of Market Standard Technology

Since this is a Category 2 technology there are no market standard technologies. One of the leading development companies with this technology is the Dutch company BTG Bioliquids. They have a commercial demonstration plant operating in The Netherlands.

BTG Bioliquids

Josink Esweg 34

7545 PN Enschede

The Netherlands

https://www.btg-btl.com/en

The other major technology supplier is the Canadian company, Ensyn Technologies Inc. They have formed a joint venture with Honeywell UOP and offer the technology through a company called Envergent Technologies. Envergent provides licensing, engineering services and equipment supply related to RTP biomass conversion equipment, with performance guarantees, to RFO production projects worldwide.  Under this joint venture, engineering of the RTP equipment is subcontracted to Honeywell UOP.

Honeywell UOP has also partnered with Ensyn in the commercial development of Refinery Co-processing opportunities.  Honeywell UOP is assisting Ensyn in interfacing with refiners and offers refineries delivery systems which allow the refiner to integrate Ensyn’s biocrude into their refineries.

Ensyn Technologies Inc.

Corporate Offices and Engineering

2 Gurdwara Road, Suite 210

Ottawa, Ontario K2E 1A2

Canada

http://www.ensyn.com/

Predication of Performance and Cost

Costs for the first of kind facilities are available and some information on the performance of the BTG facility is available.

Uncertainty

There is some uncertainty with respect to performance and economics, particularly related to scaling issues due to the stage of the development of the technology.

Additional Remarks

There is significant interest in this pathway from petroleum refiners. The potential of pyrolysis oils to be co-process in a petroleum refinery to produce gasoline and diesel blending components without oxygen is attractive to refiners who are mandated to lower the carbon intensity of the products used for transportation applications.

Quantitative Description

Biomass is a mixture of hemicellulose, cellulose, lignin and minor amounts of other organics which each pyrolyse or degrade at different rates and by different mechanisms and pathways. Lignin decomposes over a wider temperature range compared to cellulose and hemicellulose which rapidly degrade over narrower temperature ranges. The rate and extent of decomposition of each of these components depends on the process parameters of reactor (pyrolysis) temperature, biomass heating rate and pressure. The degree of secondary reaction (and hence the product yields) of the gas/vapour products depends on the time-temperature history to which they are subjected before collection, which includes the influence of the reactor configuration.

The essential features of a fast pyrolysis process are:

- Very high heat transfer rates which usually requires finely ground biomass.

- Reaction temperatures of about 500 C and short vapour residence times of less than 2 seconds

- Rapid cooling of the vapours to produce the bio-oil product.

Energy/Technical Data

Some quantitative information on the process is presented below.

Typical Plant Size

Ensyn plant sizes have been increasing as additional projects are developed. Their existing commercial plant in Ontario produces 10 million litres per year. The BTG plant has a capacity of about 20 million litres per year. The Ensyn Quebec project that is nearing completion will have a capacity of 40 million litres per year and projects in Brazil and the United States have capacities of 75 million litres per year. These different plant sizes are shown in the following table with different approximate equivalent metrics.

Table 7-1	Typical Plant Sized

| Volume Based, Million litres per year | Input Mass Based, tpy | Energy Output Based, GJ/day |
| --- | --- | --- |
| 10 | 20,000 | 650 |
| 20 | 40,000 | 1,300 |
| 40 | 80,000 | 2,600 |
| 75 | 150,000 | 5,200 |

Input and Output

The input and output for a 150,000 tpy plant is shown in the following table.

Table 7-2	Typical Input and Output

|  | Input | Output |
| --- | --- | --- |
| Wood, tonnes/year | 150,000 |  |
| Bio-oil, litres/year |  | 75,000,000 |

Forced and Planned Outage

The plants operate continuously. BTG have stated that they have almost reached the design value of 1900 hours/quarter of operating time [5].

Technical Lifetime

Plants of this type would normally be designed for at least a 20 year lifetime.

Construction Time

The Ensyn Quebec project started construction in June 2016 and is in the commission phase in the first quarter of 2018. Construction times of 18 to 24 months can be expected.

Financial Data

In addition to the public information on the capital costs for the recent BTG and Ensyn facilities there are several sources of detailed information [6] [7] [8] on capital costs and operating costs are in the public domain. These have been generated by independent third parties and not by the leading process developers.

Investment Costs

The BTG Hengelo facility had a project cost of €19 million [9]. The capital cost is essentially €1.0 per annual litre of production. The Ensyn Quebec Project had an announced capital cost of $103 million Canadian (€70 million) [10]. This is a cost of €1.75 per annual litre of production.

In 2010 Wright et al [6] develop the capital an operating costs for a fast pyrolysis system with hydrotreating. The information was detailed enough to be able to remove the hydrotreating costs. The plant processed 2000 tpd of biomass to 1 million litres of bio-oil per day. The nth plant capital cost excluding the hydrotreating was $250 million, or $0.25/litre. The nth plant capital cost was 31.5% of the pioneer plant cost.

Hu et al [7] presented the capital costs without upgrading of about $215 million for the 2000 tpd facility and slightly higher product yields of 1.1 million litres/day. A number of the participants on this paper also contributed to the earlier work by Wright et al. Shemfe [8] looked at a 72 tpd plant with hydrotreating. The capital cost was estimated to be £6.6 million (~€5.5 million) for the pyrolysis portion and £10 million for the hydrotreating portion. The plant produced 7.9 million litres of gasoline equivalent after hydrotreating.

We have assumed that the first of kind plants have capital costs of €1.0 per annual litre of production and that the nth plants will be able to reduce that to €0.30 per annual litre of production.

Operating and Maintenance Costs

Wright reported fixed costs of $11.5 million per year and variable operating costs ex feedstock of $3.6 million per year. At 350 million litres of pyrolysis oil per year those costs amount to $0.03 and $0.01 per litre for Fixed and Variable O&M costs respectively.

Start-up Costs

The start-up costs are included in the operating cost estimates.

Technology Specific Data

The properties of fast pyrolysis oil are shown in the following table [11] and compared to typical values for heavy fuel oil [12]. The bio-oil has some oxygen which reduces the energy content and increases the density.

Table 7-3	Fast Pyrolysis Bio-Oil Properties

| Parameter | Fast Pyrolysis Oil | Heavy Fuel Oil |
| --- | --- | --- |
| Energy Content, MJ/kg (LHV) | 16 | 39 |
| Water Content, wt. % | 25 | 0.1 |
| Density, kg/m3 | 1.20 | 0.98 |
| Oxygen Content, wt. % | 47 | 0 |
| Pour Point, C | -36 | 15 |
| Flash Point, C | 50 | 100 |
| Kinematic viscosity at 40 °C, mm2/s | 13 |  |
| Kinematic viscosity at 50 °C, mm2/s |  | 200-600 |

Summary

The information on pyrolysis oil production is summarized in the following table.

Table 7-4	Fast Pyrolysis Bio-oil Datasheet

| Technology | Fast Pyrolysis Bio Oil |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2030 | 2050 | Uncertainty (2020) |  | Uncertainty (2050) |  | Note | Ref |
|  |  |  |  |  | Lower | Upper | Lower | Upper |  |  |
| Energy/technical data |  |  |  |  |  |  |  |  |  |  |
| Typical total plant size 1,000 t Bio Oil/year | 20 | 80 | 200 | 1000 | 50% | 125% | 75% | 125% | A, B | 1, 6 |
| Typical total plant size. MW | 15 | 60 | 150 | 750 | 50% | 125% | 75% | 125% | A, B | 1, 6 |
| Inputs |  |  |  |  |  |  |  |  |  |  |
| Feedstock Consumption, MWh/MWh Total Input | 1.0 | 1.0 | 1.0 | 1.0 | 90% | 150% | 90% | 125% | E | 4 |
| Outputs |  |  |  |  |  |  |  |  |  |  |
| Bio Oil Output, MWh/MWh Total Input | 0.60 | 0.60 | 0.62 | 0.65 | 90% | 110% | 90% | 110% | E | 4 |
| Co-products Output MWh/MWh Total output | 0.25. | 0.0.25 | 0.23 | 0.20 | 90% | 110% | 90% | 110% | E | 4 |
| Forced outage (%) | n.a. | 0 | 0 | 0 |  |  |  |  |  |  |
| Planned outage (weeks per year) | 4 |  |  |  |  |  |  |  | C | 8 |
| Technical lifetime (years) | 20+ |  |  |  |  |  |  |  |  |  |
| Construction time (years) | 2.0 |  |  |  |  |  |  |  |  |  |
| Financial data |  |  |  |  |  |  |  |  |  |  |
| Specific investment (€ /l Bio Oil) | 1.0 | 0.80 | 0.50 | 0.30 | 75% | 125% | 75% | 125% | D | 6, 8 |
| - equipment (%) | 75 | 75 | 75 | 75 |  |  |  |  |  |  |
| -installation (%) | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| Fixed O&M (€ /l Bio Oil) | 0.03 | 0.03 | 0.03 | 0.03 | 75% | 125% | 75% | 125% | F | 6 |
| Variable O&M (€ /l Bio Oil) | 0.01 | 0.01 | 0.01 | 0.01 | 75% | 125% | 75% | 125% | F | 6 |
| Start up (€ /l Bio Oil) | n.a. | 0 | 0 | 0 |  |  |  |  |  |  |
| Technology specific data |  |  |  |  |  |  |  |  |  |  |

Notes:

- The plant size range is likely limited by the feedstock availability.

- The plant size range is typical of the range found in the literature. It is highly uncertain given that the commercial demonstration plants in operation are much smaller.

- This is twice the time normally found in process operations.

- Capital costs for nth plant are used for 2050 and earlier costs are estimates.

- These are based on the commercial demonstration plant results.

- Nth plant estimates.

References

- Muggen, G. 2015. Looking back at the first half year of commercial scale pyrolysis oil production at Empyro. tcbiomass Chicago November 4th 2015. http://www.gastechnology.org/tcbiomass/tcb2015/Muggen_Gerhard-Presentation-tcbiomass2015.pdf

- BTG Bioliquids. Empyro project. https://www.btg-btl.com/en/company/projects/empyro

- Mahajan. M. 2018. Biomass-A Massive opportunity in disguise. https://ec.europa.eu/energy/sites/ener/files/documents/32_dharmesh_mahajan-honeywell.pdf

- Bridgwater, A., Meier, D. Radlein, D. 1999. An overview of fast pyrolysis of biomass. Organic Geochemistry, Volume 30, Issue 12, Pages 1479-1493, https://doi.org/10.1016/S0146-6380(99)00120-5.

- BTG. November 2017 Newsletter. https://www.btg-btl.com/nieuwsbrieven/2017/november/en

- Mark M. Wright, Justinus A. Satrio, and Robert C. Brown 2010. Techno-Economic Analysis of Biomass Fast Pyrolysis to Transportation Fuels https://www.nrel.gov/docs/fy11osti/46586.pdf

- Hu, W., Dang, Q., Rover, M., Brown, R., Wright, M. 2015. Comparative techno-economic analysis of advanced biofuels, biochemicals, and hydrocarbon chemicals via the fast pyrolysis platform. Biofuels, 7:1, 57-67. http://dx.doi.org/10.1080/17597269.2015.1118780

- Shemfe, M.,Sai Gu, S., Ranganathan, P. 2015. Techno-economic performance analysis of biofuel production and miniature electric power generation from biomass fast pyrolysis and bio-oil upgrading, Fuel, Volume 143, 2015, Pages 361-372, https://doi.org/10.1016/j.fuel.2014.11.078.

- BE-Sustainable. 2014. Empyro BV breaks ground of its biomass to liquid pyrolysis plant. http://www.besustainablemagazine.com/cms2/empyro-bv-breaks-ground-of-its-biomass-to-liquid-pyrolysis-plant/

- Ensyn. 2016. Scale-up & Optimization BETO Workshop. https://www.energy.gov/sites/prod/files/2016/10/f33/Graham_0.pdf

- BTG. Pyrolysis oil properties. https://www.btg-btl.com/en/applications/oilproperties

- Environment Canada. 1999. Bunker C Fuel Oil. www.etc-cte.ec.gc.ca/databases/oilproperties/pdf/web_bunker_c_fuel_oil.pdf

Cellulosic Ethanol

Qualitative Description

Cellulose is an important structural component of the primary cell wall of green plants. It is an organic compound with the formula (C6H10O5)n, a polysaccharide consisting of a linear chain of several hundred to many thousands of β(1→4) linked D-glucose units. The basic chemical formula is the same as for starch with the difference being that the “n” is much larger for cellulose.

If the cellulose can be broken down into the glucose molecules then those can be fermented into ethanol and the rest of the process is similar to the 1st generation starch ethanol process.

Contact Information

Don O’Connor – (S&T)2 Consultants Inc.

Brief Technology Description

The cellulosic feedstock, which could be straw, corn stover, bagasse, or wood residue is generally subjected to a pretreatment stage to separate the cellulosic portion of the material from the lignin and make the feedstock amenable to hydrolysis. The pretreatment stage may use acid and or steam to accomplish the goal.

The hydrolysis stage breaks down the cellulose molecules into smaller polysaccharides or completely into glucose units; this is a hydrolysis reaction. Hydrolysis can be carried out with cellulase enzymes or with acid. Most process developers are using enzymes.

Following the hydrolysis stage a yeast is added to convert the sugars into ethanol and carbon dioxide. The “beer” is then distilled to remove ethanol and water from the unfermented solids. A second distillation column to reach 95% ethanol and 5% water is followed by a molecular sieve to produce anhydrous ethanol by most process developers.

The unfermented solids have little value as animal feed and process developers generally convert the material to energy (thermal and electricity) to be used in the process with any excess being exported to the grid.

A typical process flow diagram is shown in the following figure.

Figure 8-1	Cellulosic Ethanol Process

Input

The primary input is the cellulosic feedstock, along with water and the chemicals used in the process. The quantity and type of feedstock vary between process developers.

Output

The output from the system is ethanol and in most cases some electricity. With most developers the lignin and unfermented solids are burned and/or anaerobically digested to produce energy for the process. Some developers try to sell the stillage (unfermented solids) as a fertilizer for field application. This will return the nitrogen, phosphorus and phosphates in the straw to the fields and replace the synthetic fertilizers that would be used to replace these nutrients.

Energy Balance

There are a number of technology developers working on cellulosic ethanol processes. There are different approaches to the basic system that are being employed. As this is a new technology, developers do not release too much information to the public. The most detailed techno-economic information available if from the US National Renewable Energy Laboratory [1]. This information is now dated and likely doesn’t represent the current state of the art but it is still the most complete analysis available. A Sankey diagram based on published data from the US National Renewable Energy Laboratory is shown in the following figure.

Figure 8-2	Cellulosic Ethanol Energy Balance

This process also consumes a significant quantity of chemicals to assist with the process and waste treatment. The Sankey diagram when these chemicals are included is shown in the following figure. The chemical energy is the energy required to produce and transport the chemicals.

Figure 8-3	Cellulosic Ethanol Energy Balance with Chemicals

Since the process generates its own energy for use in the process, the internal energy consumption is high. The process also consumes significantly more energy than the 1st generation ethanol processes.

For comparison the Sankey diagram for the Inbicon technology is shown in the following figure [2].

Figure 8-4	Inbicon Cellulosic Ethanol Energy Balance

This technology has much lower ethanol production but also produced animal feed and lignin for energy applications after some of the lignin was used to supply the heat and power for the facilities. The Inbicon plants were designed to be integrated with power plants.

Typical Capacities

There have been about a half dozen commercial demonstration plants built in Europe, Brazil and the United States over the past five years. The plants have been sized in the 60 to 115 million litre per year range (45 to 80 MW), however few of the plants have been able to operate at capacity for significant periods of time. The plants consume 50,000 to 100,000 tonnes of feedstock per year when operated at capacity.

Regulation Ability

Since most of the plants have not been able to operate continuously for extended periods of time it has been difficult to assess their regulation capabilities. In theory, the regulation capabilities should be similar to those of the 1st generation plants but the added complexity of the process and the fact that the process supplies the fuel for the process my limit the regulation capacity.

Space Requirements

The 50 million litre per year Beta Renewables Plant in Italy covers an area of 14 ha including some feedstock storage. The 110 million litre per year DuPont facility in Nevada Iowa has an area of 18.5 ha, excluding the storage area. The storage area appears to be twice the size of the plant. The 80 million litre per year POET DSM covers an area of 120 ha including feedstock storage.

The space requirements for the plant do vary significantly, some of which is due to the need to store a year or more of feedstock on the site. Space requirements of 0.5 ha/million litres (7000 m2/MW) would appear to be a reasonable value which allows room for feedstock storage.

Advantages/Disadvantages

The cellulosic ethanol process utilizes a feedstock that is largely underutilized in most agricultural regions of the world, thus there is no perceived competition for feed and food crops. The GHG emissions are similar to those of sugarcane ethanol, where crop residues provide the energy for the process.

The technology is complex and capital costs will be higher than for 1st generation ethanol plants. The technology complexity has also negatively impacted the time required to commercialize the process. The POET DSM plant has only recently announced that they have been able to solve the pre-treatment challenges and have been able to run the plant at 80% of capacity [3]. This is three years after the plant opened.

Other large scale first of kind plants have also had commercial and technical challenges, for example the DuPont plant is now for sale, the Beta Renewables plant in Italy has been idled and the Abengoa plant in Kansas was sold in a bankruptcy sale. The exact reasons for these decisions are not public and in some cases the decisions may have been due to other challenges that the companies may have faced.

POET DSM [4] reported that in addition to resolving the technical challenges of pretreatment that they have decided to build an on-site enzyme production facility. Presumably this is a commercial decision; on-site production would not need to concentrate the enzyme to reduce transportation costs, saving capital and operating costs.

Van der Meij [5] recently presented a venture capitalist’s view of the technology and stated that he could not see the business case for the technology citing feedstock logistics costs, scale, high pretreatment costs and margin uncertainty as issues.  He also stated that there was still significant technology risk present.

Environment

In some regions of the world there is concern that removing too much straw from the field will have negative impacts on soil health. Reduced soil carbon and increased erosion potential are potential outcomes if the crop residue harvest is not undertaken in a sustainable manner.

Research and Development Perspective

Cellulosic ethanol produced from straw is a category 2 technology, a pioneer phase technology with limited applications to date. The technology has been proven to work through demonstration facilities or semi-commercial plants. However, due to the limited application, the price and performance is still attached with high uncertainty, since development and customization is still needed. The technology still has a significant development potential.

This technology has probably had more research undertaken on it than all of the other technologies in this report. As the technology has been scaled to near commercial scale many of the process developers have experienced significant challenges with materials handling through the front end of the process.

The challenges of operating the first of a kind cellulosic ethanol facility were acknowledged by Merritt [6]. He stated that three years after start-up the plant is operating at 80% of capacity, converting 95% of the fermentable sugars to ethanol at the design solids loading. The engineering focus is now on how to simplify the process and reduce capital costs, exactly the activities that create the learning curve. He also highlights advances in enzyme effectiveness in reducing future operating costs.

Examples of Market Standard Technology

The cellulosic ethanol technology has recently suffered some setbacks with respect to the commercialization of the technology.

One of the early movers on the commercialization of cellulosic ethanol technology was the Danish company Inbicon. However they have had difficulty in developing a commercial project without government guarantees.

The Italian company, Gruppo Mossi Ghisolfi, which developed the Beta Renewables plant in Crescentino Italy and supplied the technology for the GranBio plant in Brazil, has recently been granted protection from creditors [7].

DuPont has recently put their Nevada Iowa plant up for sale [8].

The one company that has recently moved ahead with new projects is Clariant. They have announced a license agreement with Enviral [9] to add a 50 million litre per year plant to an existing facility in Slovakia and a similar sized plant to be built in Romania [10].

Clariant Produkte Deutschland GmbH

Biotech & Renewables Center

Staffelseestrasse 6

D-81477 Munich, Germany

Predication of Performance and Cost

This technology is at the early stages of commercialization. There are some reported capital costs for the first commercial scale plants (pioneer plants) but there is no published, independently verified information on the actual performance of the plants.

It is expected that capital costs and operating costs will improve significantly once the existing plants achieve consistent operations and new plants are built based on the learnings from operating the first commercial plants.

Uncertainty

There remains significant uncertainty with respect to the commercialization of the technology. The plants built 3 to 5 years ago have experience some unforeseen technical and economic challenges which has slowed the development progress.

Additional Remarks

The significant cellulosic plants that have been built are shown in the following table [11].

Table 8-1	Cellulosic Ethanol Plants

| Name | Annual Capacity | Feedstock | Comment |
| --- | --- | --- | --- |
| Abengoa Bioenergy Biomass of Kansas | 90 million litres | corn stover, wheat straw, switch grass | Idle |
| Beta Renewables | 50 million litres | wheat straw, rice straw, arundo donax, poplar | Idle |
| DuPont | 110 million litres | Corn Stover | Idle |
| GranBio | 80 million litres | Sugarcane bagasse and straw | Operational |
| POET-DSM | 80 million litres | Corn Stover and corn cobs | Operational |
| Raizen Energia | 40 million litres | Bagasse | Operational |

Quantitative Description

The available quantitative data that is available on the technology is mostly from third parties and not from the technology providers or plant operators. Actual plant data is considered confidential by the process developers.

Energy/Technical Data

The process itself is essentially identical to the starch ethanol process with the primary difference being the length of glucose polymer chain.

The process involves two chemical reactions. In the first, cellulose is hydrolyzed to glucose as shown below. One of the technical challenges with the process is making the cellulose component of the lignocellulosic feedstock accessible for the hydrolysis process.

(C6H10O5)n + nH2O → nC6H12O6

Cellulose      Water      Glucose

The glucose is then fermented with yeast to ethanol and carbon dioxide.

C6H12O6        →     2C2H5OH    +      2CO2

Glucose     Ethanol     Carbon Dioxide

Typical Plant Size

The plants that have been built have been in the 50 to 100 million litres per year range. Some of the techno economic studies that have been done use plant sizes of 200 million litre per year (140 MW) [1]. As with the 1st generation plants it is likely that plant sizes will increase as more plants are built and experience is gained with the technology.

Input and Output

The primary input is the feedstock. Most technology developers (but not all) utilize the non fermentables to provide the thermal energy and electric power for the plant and some have excess power available for sale.

The plants also consume enzymes, yeast, and a number of process chemicals. Enzymes can be purchased or produced on site using a portion of the sugar produced by hydrolysis. Enzyme consumption is much higher than it is in a 1st generation ethanol plant. In the 2012 state of the art case [1] reported the consumption of 0.38 kg of chemicals for every kilogram of ethanol produced. It is likely that this has been decreased with additional development work but NREL have not published an updated state of the art report.

The plants produce ethanol and in some cases electricity. Some developers propose to sell a fertilizer product instead of producing electricity. The data used for the Sankey diagram is shown in the following table [1].

Table 8-2	Cellulosic Ethanol Inputs and Outputs

| Parameter | Input | Output |
| --- | --- | --- |
| Feedstock | 3.38 kg |  |
| Chemicals | 0.38 kg |  |
| Sulfuric acid | 0.072 |  |
| Sodium hydroxide | 0.082 |  |
| Ammonia | 0.042 |  |
| Corn steep liquor | 0.048 |  |
| Diammonium phosphate | 0.005 |  |
| Sorbitol | 0.002 |  |
| Glucose | 0.103 |  |
| Host nutrients | 0.002 |  |
| Sulfur dioxide | 0.001 |  |
| Ethanol |  | 1 litre |
| Power |  | 0.70 kWh |

The current performance is not likely at this level. It has been assumed that the current yield is 80% of the value shown, the value can be achieved in 2030 and the 2050 value is 10% better.

Forced and Planned Outage

The existing operating plants have struggled with forced outages. The following figure shows the production rate versus time for three generations of Iogen’s cellulosic technology [12]. The figure shows the difficulty in achieving design rates but also the progress that can be made with experience.

Figure 8-5	Cellulosic Ethanol Production Rates

Both Iogen and Poet have stated that the initial operational challenges have been in the pre-treatment stage of the process. Severe fouling and flow restrictions have been reported in this section. Eventually it is expected that plants will be able to operate for 330 to 350 days per year with no forced downtime.

Technical Lifetime

Plant lifetimes should exceed 25 years, as the US and EU starch ethanol industries have demonstrated.

Construction Time

Due to the added complexity of the process construction times are expected to be 24 to 30 months.

Financial Data

There is very limited financial data available on the process. The available information is a combination of techno-economic studies and the published capital costs of the commercial plants.

Investment Costs

Lynd et al (2017) reported the size and capital expenditures for the pioneering plants [13]. The information is shown in the following table.

Table 8-3	Capital Costs

| Company | Location | Feedstock | Capacity, million litres | Capital Expenditure, Million US$ | Capital, $/ litre |
| --- | --- | --- | --- | --- | --- |
| Abengoa | Kansas, USA | Straw | 95 | 444.6 | 4.7 |
| Beta Renewables | Crescentino, Italy | Straw | 51 | 171 | 3.4 |
| DuPont | Iowa, USA | Corn stover | 114 | 500 | 4.4 |
| Granbio | Alagoas, Brazil | Bagasse, straw | 82 | 265 | 3.2 |
| POET/DSM | Iowa, USA | Corn stover | 76 | 275 | 3.6 |
| Raizen | Piracicaba, Brazil | Bagasse, straw | 40 | 102 | 2.5 |
| Average |  |  | 76 | 293 | 3.9 |

The capital costs for the pioneering plants have been about six to seven times higher than 1st generation plants. While de Jong [14] did not report a capital growth factor for this technology, the factors for other biomass feedstock technologies was in the range of 0.4 suggesting that the nth plants might have capital costs of $1.60/litre.

The NREL capital cost for the nth plant [1] was $381 million for a 207 million litre plant ($1.84/litre), in line with the capital growth factor adjusted costs of the pioneering facilities.

Irena [15] reported capital costs on the range of 2,000 to 3,500 US$/kW ethanol (1.5 to 2.6 US $/annual litre). They see this being reduced to 1,200 to 2,300 US $/kW ethanol by 2045.

The EU Sub Group on Advanced Biofuels [16] reported capital costs 3,300 €/kW but noted that the first plants ranged from 2,380 to over 6,700 €/kW.

The capital costs are 3 to 5 times higher than 1st generation plants and vary widely between plants. The current estimated range is €1.5 to €2.5/litre (€3,300 to 5,600/KW).

Operating and Maintenance Costs

The NREL operating and maintenance costs are summarized in the following table [1].

Table 8-4	Operating and Maintenance Costs

| Parameter | Euro/litre |
| --- | --- |
| Variable operating costs, ex feedstock | 0.083 |
| Fixed costs | 0.04 |

These are nth plant estimates which are unlikely to be achieved for the pioneering plants although there is no published information on the performance of the pioneering plants.

Start-up Costs

Start-up costs are included in the operating costs.

Technology Specific Data

The ethanol properties of cellulosic ethanol are identical to the 1st generation ethanol properties.

Summary

The quantitative data for the cellulosic ethanol process are summarized in the following table.

Table 8-5	Cellulosic Ethanol Datasheet

| Technology | Cellulosic Ethanol |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2030 | 2050 | Uncertainty (2020) |  | Uncertainty (2050) |  | Note | Ref |
|  |  |  |  |  | Lower | Upper | Lower | Upper |  |  |
| Energy/technical data |  |  |  |  |  |  |  |  |  |  |
| Typical total plant size 1,000 tonnes/year | 60 | 60 | 150 | 200 | 50% | 150% | 50% | 150% | A, B | 9 |
| Typical total plant size, MW | 55 | 55 | 130 | 180 | 50% | 150% | 50% | 150% | A, B | 9 |
| Inputs |  |  |  |  |  |  |  |  |  |  |
| Feedstock Consumption, MWh/MWh Total Input | 1.0 | 1.0 | 1.0 | 1.5 | 90% | 150% | 90% | 150% | C | 1 |
| Outputs |  |  |  |  |  |  |  |  |  |  |
| Ethanol Output, MWh/MWh Total Input | 0.29 | 0.29 | 0.37 | 0.40 | 80% | 120% | 80% | 120% | C | 1 |
| Co-products Output, MWh/MWh Total Input | 0.04 | 0.04 | 0.05 | 0.06 | 80% | 120% | 80% | 120% | C | 1 |
| Forced outage (%) | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Planned outage (weeks per year) | 2 |  |  |  |  |  |  |  |  |  |
| Technical lifetime (years) | 25 |  |  |  |  |  |  |  |  |  |
| Construction time (years) | 2.0 |  |  |  |  |  |  |  |  |  |
| Financial data |  |  |  |  |  |  |  |  |  |  |
| Specific investment (€ /l ethanol) | 4.0 | 4.0 | 1.8 | 1.5 | 75% | 150% | 75% | 150% | D | 1, 10, 11 |
| - equipment (%) | 75% | 75% | 75% | 75% |  |  |  |  |  |  |
| -installation (%) | 25% | 25% | 25% | 25% |  |  |  |  |  |  |
| Fixed O&M (€ /l ethanol) | 0.08 | 0.08 | 0.04 | 0.04 | 95% | 200% | 95% | 200% | F | 1 |
| Variable O&M (€ /l ethanol) | 0.16 | 0.16 | 0.08 | 0.08 | 95% | 200% | 95% | 200% | F | 1 |
| Start up (€ /l ethanol) | 0 | 0 | 0 | 0 |  |  |  |  | E |  |
| Technology specific data |  |  |  |  |  |  |  |  |  |  |

Notes:

- The plant size is assumed based on the plants sizes of existing ethanol plants.

- The plant size uncertainty represents the current range in the market.

- Not all process developers are attempting to maximize ethanol output; this can result in higher feedstock consumption but lower chemical use. Co-product production is higher if the ethanol yield is low.

- There is a wide range of reported capital costs for the existing plants and a wide range in the cost of future plants.

- Start-up costs are included in the operating costs.

- O&M costs are based on the nth plant. There are no public estimates for the costs from current plants. It has been estimated that they are twice the values for the nth plants. This may be optimistic unless the on line performance can be improved.

References

- Tao, L., Schell, D., Davis, R., Tan, E., Elander, R. and Bratis, A. 2014. NREL 2012 Achievement of Ethanol Cost Targets: Biochemical Ethanol Fermentation via Dilute-Acid Pretreatment and Enzymatic Hydrolysis of Corn Stover. NREL/TP-5100-61563. https://www.nrel.gov/docs/fy14osti/61563.pdf

- Kacelle. 2014. Bringing cellulosic ethanol to industrial production at Kalundborg, Denmark. https://biorefiningalliance.com/wp-content/uploads/2014/12/KACELLE-PUBLISHABLE-SUMMARY-web.pdf

- POET-DSM achieves cellulosic biofuel breakthrough. Nov 2017. http://poetdsm.com/pr/poet-dsm-achieves-cellulosic-biofuel-breakthrough

- Thakrar, A. 2017. PROJECT LIBERTY: Lessons learned in commercializing cellulosic biofuels. Presented at 6TH International Conference on Lignocellulosic Ethanol. Brussels, Sept 2017.

- Van der Meij, R. 2017. A venture capitalist’s view on investment in lingo-cellulosic ethanol. Presented at 6TH International Conference on Lignocellulosic Ethanol. Brussels, Sept 2017.

- Merritt, M. 2017. Real Progress in Cellulosic Biofuel. Biofuels, Bioprod. Bioref., 11: 943-944. https://doi.org/10.1002/bbb.1837

- Beta Renewables. Oct 2017. The Alessandria court admitted the Gruppo Mossi Ghisolfi companies to the “concordato preventivo”. http://www.betarenewables.com/en/media-relations/news-detail/32

- DuPont to sell cellulosic ethanol plant in blow to biofuel. Nov 2017. https://www.reuters.com/article/us-dowdupont-ethanol/dupont-to-sell-cellulosic-ethanol-plant-in-blow-to-biofuel-idUSKBN1D22T5

- Clariant. 2017. Clariant And Enviral Announce First License Agreement on Sunliquid® Cellulosic Ethanol Technology. https://www.clariant.com/en/Corporate/News/2017/09/Clariant-and-Enviral-announce-first-license-agreement-on-sunliquid-cellulosic-ethanol-technology

- Clariant. 2017. Clariant to Build Flagship Sunliquid® Cellulosic Ethanol Plant in Romania. https://www.clariant.com/en/Corporate/News/2017/10/Clariant-to-build-flagship-sunliquid-cellulosic-ethanol-plant-in-Romania

- IEA Bioenergy Task 39. Cellulosic Ethanol Plants. http://demoplants.bioenergy2020.eu/

- Brian Foody. 2016. Scaling Up Cellulosic Biofuels, Ideas & Experience. Presented at Scaling Up, Ottawa, Ontario, Nov 15, 2016.

- Lee R Lynd, Xiaoyu Liang, Mary J Biddy, Andrew Allee, Hao Cai, Thomas Foust, Michael E Himmel, Mark S Laser, Michael Wang, Charles E Wyman. 2017. Cellulosic ethanol: status and innovation, In Current Opinion in Biotechnology, Volume 45, 2017, Pages 202-211, ISSN 0958-1669, https://doi.org/10.1016/j.copbio.2017.03.008

- de Jong, S., Hoefnagels, R., Faaij, A., Slade, R., Mawhood, R. and Junginger, M. 2015. The feasibility of short-term production strategies for renewable jet fuels – a comprehensive techno-economic comparison. Biofuels, Bioprod. Bioref., 9: 778–800. https://www.doi.org/10.1002/bbb.1613

- Irena. 2016. Innovation Outlook. Advanced Liquid Biofuels. http://www.irena.org/DocumentDownloads/Publications/IRENA_Innovation_Outlook_Advanced_Liquid_Biofuels_2016.pdf

- Sub Group on Advanced Biofuels. 2017. Cost of Biofuel. http://platformduurzamebiobrandstoffen.nl/wp-content/uploads/2017/07/2017_SGAB_Cost-of-Biofuels.pdf

Biomass Gasification for Diesel and Jet Fuel

Qualitative Description

The production of diesel or jet fuel from biomass is a two-step process, in the first step the solid biomass is converted to the gas phase and in the second step the gas is converted to liquid fuels.

Gasification is a process that converts organic or fossil-based carbonaceous materials at high temperatures (>700°C), without combustion, with a controlled amount of oxygen and/or steam into carbon monoxide, hydrogen, and carbon dioxide (syngas). The carbon monoxide then reacts with water to form carbon dioxide and more hydrogen via a water-gas shift reaction.

The Fischer–Tropsch process (or Fischer–Tropsch Synthesis or F-T) is a set of chemical reactions that changes a mixture of carbon monoxide gas and hydrogen gas into liquid hydrocarbons. These reactions occur in the presence of certain metal catalysts, typically at temperatures of 150–300 °C and pressures of one to several tens of atmospheres.

Contact Information

Don O’Connor – (S&T)2 Consultants Inc.

Brief Technology Description

The biomass could be agricultural or forestry residues. There is a wide range in the design of gasifiers used for biomass. Different technological solutions can be implemented in order to obtain different plant configurations; in particular, the mode of contact of the biomass with the gasification agent may be in counter-current, or co-current, or crossflow, and the heat can be transferred from the outside or directly in the reactor using a combustion agent; the residence time can be in the order of hours (static gasifiers, rotary kiln) or minutes (fluidized bed gasifiers). Different gasifier designs are better suited to different feedstocks and gas needs.

The Fisher Tropsch reactions are practised commercial on syngas produced from coal (Sasol) and on natural gas (Shell, Chevron, Sasol, and others).

The overall process is shown in the following simplified process flow diagram.

Figure 9-1	Biomass to Diesel and Jet Process

Input

The primary input for most process is just the biomass. The reactions are exothermic and generate enough heat for the process and to produce the power required for the system.

Output

The FT synthesis process produces a range of hydrocarbon from light ends to heavy waxes. It is difficult to control the selectivity of the process to produce just diesel fuel or jet fuel. In some commercial facilities the light ends and the heavy wax materials can be recycled through the process to improve the selectivity but usually at the expense of overall efficiency. Some systems will produce excess power for sale from the system.

Energy Balance

The energy balance for a system is shown in the following figure [1]. It is based on NREL’s work on biomass to liquids. The carbon efficiency of the biomass gasifier to raw syngas is 71.8% and the carbon efficiency of the syngas to fuels is 46.2% for an overall carbon efficiency of biomass to fuels of 33.2%. The energy out per unit of energy in is 38.5%.

Figure 9-2	Biomass to Diesel and Jet Energy Balance

The overall energy efficiency of the process is relatively low. There are two potential means to recover some of the waste heat. The plants use some of the process heat to produce electricity for the plant use and potentially a small amount to be exported. Steam from the exit of the final steam turbine would be available for other uses. This could have a temperature between 150 and 185C depending on the design. There may also be some opportunity to recover some lower grade heat as the syngas is conditioned prior to synthesis. Details of the potential for energy recovery are not reported in most of the recent techno-economic studies published.

Other computer simulations of biomass to FT systems have reported higher efficiencies. Kreutz et al [2] reported 49 to 50.5% energy efficiency on a LHV basis. They had similar carbon efficiency of the feedstock to the fuels.

Baliban et al [3] modelled several optimized hardwood to FT liquid process configurations. The energy efficiency ranged from 56 to 61% and the carbon conversion efficiency ranges from 54 to 60%.

Sikarwar [4] identified feedstock characteristics that influence the performance of biomass gasification systems. These include moisture content, ash content, chlorine and sulphur, and the cellulose, hemicellulose and lignin fractions as the three components degrade at different temperatures during gasification. He reports that in general, the higher the cellulose and hemicellulose content, the greater the volume of gaseous products formed. Therefore, softwood, hardwood, wheat straw and bagasse with much higher cumulative percentages of cellulose and hemicellulose are preferred over sunflower seed hull, coconut shell, almond shell, larch plant or poultry litter, when attempting to obtain gas as the final product.

Typical Capacities

There are no commercial scale systems in operation. The NREL techno-economic analysis was based on processing 2,000 tonnes of dry wood per day. The plant would produce 175 million litres per year of fuel.

In a review of the state of the art biomass gasification [5], Molino et al reports on the European biomass gasification plants. They identify 22 gasification facilities but only five that has more than 7,000 h/year of operation time. The capacities are in the 10 to 40 MW range with one larger facility that co-fires biomass and fossil fuels. The facilities are identified in the following table.

Table 9-1	European Biomass Gasification Plants

| Site | Thermal Output, GJ/year | Running h/year |
| --- | --- | --- |
| Harboore (Denmark) | 576,000 | 8,000 |
| Gussing (Austria) | 230,400 | 7,000–8,000 |
| Skive (Denmark) | 576,000 | 7,500 |
| Lahti (Finland) | 1,152,000 | 7,000 |
| Buggenum (the Netherlands) | 17,280,000 | 7,500 |

The commercial FT plants using fossil energy as the input are all much larger than the biomass gasification plants shown in the table above. The largest fossil plant is the 260,000 bbls/day (500 million GJ/year) Shell plant in Qatar. Shell’s original GTL plant in Malaysia has a capacity of less than 15,000 bbls/day (30 million GJ/year).

There is work ongoing on small FT distillate reactors. Velocys claims that the commercially optimal size for their biomass to FT liquids system is 1,900 bbl/day (72 million litres/year) [6]. Their reference plant processes landfill gas and produced 200 bbls/day of finished products (375,000 GJ/year).

Regulation Ability

Biomass gasifiers can be operated down to about 35% of the rated capacity depending on the configuration, feedstock moisture contents and the acceptable efficiency loss [7]. However, the gas composition will change over this range and when the gasifier is coupled to a synthesis reactor there is a need for relatively constant feed compositions. The high temperature and pressure of the FT reactor will limit the regulation ability of the overall system.

Space Requirements

The biomass gasification district heating plant in Harboore is situated on a less than one hectare. The GoBiGas facility in Sweden, which is a biomass gasifier and an SNG facility is on a two hectare site. The original design capacity was 100 MW (3 million GJ/year), although only the first phase of 20 MW was built.

The space requirements will be less than 1,000 m2/MW. This may be reduced if the size of the units are larger.

Advantages/Disadvantages

Biofuels that can be produced from non-feed or food feedstocks and can be used in heavy duty transport applications, which can’t be easily electrified, are an attractive option for decarbonizing the transport sector. Drop-in biofuels, such as the FT fuels made by this technology can be used in the existing fuel infrastructure and are attractive to the existing fuel providers.

This technology combines gasification systems that have only been operated at small scale and FT synthesis systems that are commercialized at very large scale. Determining the combined size that will work, technically and economically, for both technologies is a challenge. The techno-economic analyses that have been done on this technology have considered plants in the range of 1,000 to 2,000 dry tonnes of wood per day (350,000 to 700,000 dry tonnes of wood per year.

The Danish Energy Agency reports the following production of woody biomass in 2015 [8].

Table 9-2	Woody Biomass Production and Consumption for Energy

| Type | TJ | Tonnes (dry) |
| --- | --- | --- |
| Wood Chips | 13,335 | 701,842 |
| Firewood | 21,943 | 1,154,895 |
| Wood pellets | 2,641 | 139,000 |
| Wood waste | 8,837 | 465,105 |
| Total | 46,756 | 2,460,842 |

A single woody biomass to diesel and jet fuel plant would require a 15 to 30% increase in the current production and consumption of woody biomass in Denmark.

Environment

The sustainability of the feedstock production is a potential issue with all biomass systems. The overall energy out per unit of energy in the feedstock is relatively low for this technology. Biomass gasification systems will produce some ash that must be disposed of. The wood ash can be used to adjust the pH of soils but the availability of the nutrients in the ash may not always be fully bioavailable. Wood species and gasification type appear to have some influence on the properties [9].

The fuels produced have no sulphur, are low in aromatics and are considered clean burning. Their volumetric energy content is about 10% lower than diesel fuel due to the lower density.

Research and Development Perspective

Biomass gasification for diesel and jet fuel produced from wood or straw is a category 2 technology, a pioneer phase technology with limited applications to date. The technology has been proven to work through demonstration facilities or semi-commercial plants. However, due to the limited application, the price and performance is still attached with high uncertainty, since development and customization is still needed. The technology still has a significant development potential.

Both the gasification and the FT distillate technology have been known and practiced for almost 100 years. They are commercial technologies for other feedstocks.

There is work underway on integrating the two technologies, improving the gas clean-up system performance and addressing the issue of scale for the fuel synthesis stage.

Examples of Market Standard Technology

The technology has not yet been commercialized. In Europe, Repotec, an Austrian company, have been involved with the Gussing gasifier, the GoBiGas SNG project in Sweden, and the Senden wood gasifier to power facility in Germany.

In Denmark, B&W Volund built the wood gasifier at Harboore but no other references for the technology were identified.

The UK-American company, Velocys is working on small scale FT plants. They are developing smaller scale microchannel FT technology that was originally developed by the Pacific Northwest National Laboratory in Washington State, USA. Their first project is using landfill gas but they are working with ThermoChem Recovery International of gasification systems for woody biomass that would be coupled with the Velocys FT technology [10]. The system would produce 1,400 bbl/day of FT products. This would require 1,000 tonnes of wood per day.

Velocys is working on a USDA Phase 2 application for a loan guarantee for a wood to FT liquid project [11]. Phase 2 applications include the environmental report, technical report, financial model, and the lender’s credit evaluation. The plant is to be built in Natchez, Mississippi on a 40 ha site. The plan is to start construction in late 2018 or early 2019. Velocys are also working on a waste to jet fuels project in the UK with British Airways and other partners but this project is not as well defined as the wood project in the USA.

Velocys plc

Harwell Innovation Centre
173 Curie Avenue, Harwell
OX11 0QG, United Kingdom

Predication of Performance and Cost

The prediction of performance and cost is based on published techno-economic papers rather than on actual plant performance. The NREL paper is based on a plant twice the size of the proposed Velocys project.

Uncertainty

There is a high level of uncertainty for the technology given the state of development and the fact that there are no operating plants in the world at this time.

Additional Remarks

One of the challenges for small scale FT plants has been that a range of products is produced, from gasoline boiling range products to waxes. Markets for all products are required for commercial success and finding markets for small volumes of gasoline blending components and the wax can be an issue. In some projects the revenue from the wax has been a significant portion of the total revenue.

FT synthesis produces a range of products between C1 and waxes. The actual ranges will vary with process type, catalysts, and syngas quality but there is always a range of products. Tijm [12] reports on the product distribution for two different process severities as shown in the following table. Unfortunately the paper does not provide the accompanying yield data for the two operating conditions but there is more gasoline produced in the kerosene mode than the diesel mode.

Table 9-3	Product Distributions – Shell SMDS

|  | Gas Oil Mode | Kerosene Mode |
| --- | --- | --- |
|  | % wt |  |
| Tops/naphtha | 15 | 25 |
| Kerosene | 25 | 50 |
| Gas Oil | 50 | 25 |

In his 1999 thesis, van der Lann [13] showed that the quantity of each group of products did vary with operation conditions. This is shown in the following figure where the two right hand bars represent the liquid products and the two left hand bars represent the gaseous products. The sum of the two liquid products (and thus the yield) as well as the ratio of heavy to light liquid products does vary with the pretreatment conditions.

Figure 9-3	Selectivity vs. Yield

Quantitative Description

The available quantitative data that is available on the technology is mostly from third parties and not from the technology providers or plant operators. Actual plant data is considered confidential by the process developers.

Energy/Technical Data

There are three basic reactions that occur in the process. The first reaction breaks the biomass down to a combination of hydrogen, carbon monoxide and carbon dioxide. A simplified reaction is shown below. Actual biomass has highly variable composition and complexity with cellulose as one major component.

C6H12O6 + O2 + H2O → CO + CO2 + H2 + CH4 + other species

Note: The above reaction uses glucose as a surrogate for cellulose.

Stoichiometry for methanol production of syngas requires the ratio of H2 / CO to equal 3. The product gases are then subjected to the water-gas shift reaction to increase the quantity of hydrogen. The equilibrium for this reaction is temperature dependent which controls the CO to CO2 ratio.

CO + H2O ↔ CO2 + H2

This is then followed by the synthesis reaction as shown below. In this reaction the carbons are added sequentially making it difficult to control the chain lengths of the final products.

(2n + 1) H2 + n CO → CnH2n+2 + n H2O

Generally, the Fischer–Tropsch process is operated in the temperature range of 150–300 °C. Higher temperatures lead to faster reactions and higher conversion rates but also tend to favor methane production. For this reason, the temperature is usually maintained at the low to middle part of the range. Increasing the pressure leads to higher conversion rates and also favors formation of long-chained alkanes, both of which are desirable. Typical pressures are up to 30 bar.

Typical Plant Size

The proposed Velocys plant will process 1,000 tpd of wood and produce 79 million litres of product. The NREL techno-economic work assumed a plant size of 2,000 tpd. This is about the wood required for an average pulp mill. New pulp mills are being built larger and can consume up to 10,000 tpd. Plant size will likely be determined by the feedstock availability.

Input and Output

The primary input and output for a wood to FT plant is summarized in the following table [1]. There are some chemicals and catalysts required by the quantities are very small.

Table 9-4	Inputs and Outputs

| Parameter | Input | Output |
| --- | --- | --- |
| Wood | 4.06 kg |  |
| Wood | 76 MJ |  |
| Naphtha |  | 0.36 litre |
| Jet |  | 0.38 litre |
| Diesel |  | 0.26 litre |
| Power |  | 0.26 kWh |

Forced and Planned Outage

The plants are expected to operate for 350 days per year. Wood gasifiers are capable of operating at these rates as shown earlier and fossil FT plants are also capable of operating at these rates. Forced outages are expected to be minimal.

Technical Lifetime

Plant lifetimes should exceed 25 years.

Construction Time

Construction time for the technology is expected to be about 24 months.

Financial Data

The financial data is only available from the literature. There is a preliminary cost estimate of $300 million for the 72 million litre Velocys plant in Mississippi (80 MW) [14]. This would be €4.25/litre for a European plant.

Investment Costs

Tan et al project that the total capital investment for the plant is $650 million for the 180 million litre plant. The cost basis is 2011. Converting this to 2015 Euros the cost would be 716 million Euro, or €4.0/litre of product.

Irena [15] report current capital costs for this pathway as $3,000 to $5,000/kW (€2.8 to €4.6/litre). The EU Sub Group on Advanced Biofuels [16] report the capital costs as €3,000/kW (€3.35/litre).

Considering the capital growth factor information from de Jong [17] the capital cost information for the nth plant from Tan and the Velocys pioneering plant is too close together. It is likely that the pioneering cost estimate is too low, we have increased it to €5.00/litre, which may still be too optimistic

Operating and Maintenance Costs

The Tan et al estimates of fixed and variable operating costs, excluding feedstock are shown in the following table. These costs are much less than the feedstock costs and the capital related costs in the analysis.

Table 9-5	Operating and Maintenance Costs

| Parameter | Euro/litre |
| --- | --- |
| Variable operating costs, ex feedstock | 0.01 |
| Fixed costs | 0.12 |

Start-up Costs

The start-up costs are included in the costs shown above.

Technology Specific Data

The typical properties of FT diesel are compared to petroleum diesel in the following table. The fuel has a higher cetane than petroleum diesel but a lower volumetric energy content.

Table 9-6	Typical Fuel Properties

|  | Petroleum Diesel | FT Diesel |
| --- | --- | --- |
| Density, kg/litre | 0.84 | 0.77 |
| Energy content, MJ/litre | 36.0 | 33.9 |
| Energy content, MJ/kg | 42.8 | 44.0 |
| Cetane | 48 | ~70 |

Summary

The quantitative data for the biomass to diesel and jet process are summarized in the following table.

Table 9-7	Biomass Gasification Datasheet

| Technology | Biomass to Diesel and Jet |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2030 | 2050 | Uncertainty (2020) |  | Uncertainty (2050) |  | Note | Ref |
|  |  |  |  |  | Lower | Upper | Lower | Upper |  |  |
| Energy/technical data |  |  |  |  |  |  |  |  |  |  |
| Typical total plant size 1,000 t FT Liquids/year | 50 | 50 | 100 | 150 | 100% | 150% | 50% | 150% | A, B | 1, 6 |
| Typical total plant size, MW | 75 | 75 | 150 | 225 | 100% | 150% | 50% | 150% | A, B | 1, 6 |
| Inputs |  |  |  |  |  |  |  |  |  |  |
| Feedstock Consumption, , MWh/MWh Total Input | 0.33 | 0.33 | 0.38 | 0.38 | 90% | 110% | 90% | 110% | C | 1 |
| Outputs |  |  |  |  |  |  |  |  |  |  |
| FT Liquids Output, , MWh/MWh Total Output | 0.38 | 0.38 | 0.40 | 0.45 | 90% | 110% | 90% | 110% | D | 1 |
| Co-products Output, , MWh/MWh Total Input | 0.01 | 0.01 | 0.02 | 0.02 | 90% | 110% | 90% | 110% | D | 1 |
| Forced outage (%) | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Planned outage (weeks per year) | 2 |  |  |  |  |  |  |  |  |  |
| Technical lifetime (years) | 25 |  |  |  |  |  |  |  |  |  |
| Construction time (years) | 2.0 |  |  |  |  |  |  |  |  |  |
| Financial data |  |  |  |  |  |  |  |  |  |  |
| Specific investment (€ /l FT Liquids) | 5.00 | 5.00 | 4.50 | 4.00 | 75% | 120% | 75% | 120% | E | 1, 6, 7, 8, 9 |
| - equipment (%) | 75 | 75 | 75 | 75 |  |  |  |  |  |  |
| -installation (%) | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| Fixed O&M (€ / l FT Liquids) | 0.12 | 0.12 | 0.12 | 0.12 |  |  |  |  |  | 1 |
| Variable O&M (€ / l FT Liquids) | 0.01 | 0.01 | 0.01 | 0.01 |  |  |  |  |  | 1 |
| Start up (€ / l FT Liquids) | 0 | 0 | 0 | 0 |  |  |  |  | F |  |
| Technology specific data |  |  |  |  |  |  |  |  |  |  |

Notes:

- The plant size is assumed based on the proposed Velocys plant and the NREL nth plant.

- Feedstock availability is likely to determine the maximum plant size.

- The feedstock requirements could vary with efforts to improve the desired product selectivity.

- Over time the power available for export may increase due to improve thermal management in the plant.

- There is a wide range of reported capital costs for the existing plants and a wide range in the cost of future plants.

- Start-up costs are included in the operating costs.

References

- Tan, E. C. D., Snowden-Swan, L. J., Talmadge, M., Dutta, A., Jones, S., Ramasamy, K. K., Gray, M., Dagle, R., Padmaperuma, A., Gerber, M., Sahir, A. H., Tao, L. and Zhang, Y. 2017. Comparative techno-economic analysis and process design for indirect liquefaction pathways to distillate-range fuels via biomass-derived oxygenated intermediates upgrading. Biofuels, Bioprod. Bioref., 11: 41–66. https://www.doi.org/10.1002/bbb.1710

- Kreutz, T., Larson, E., Liu, G., Williams, R. 2008. Fischer-Tropsch Fuels from Coal and Biomass. 25th Annual International Pittsburgh Coal Conference. http://acee.princeton.edu/wp-content/uploads/2016/10/Kreutz-et-al-PCC-2008-10-7-08.pdf

- Baliban, R., Elia, J., Floudas, C., Gurau, B., Weingarten, M., and Klotz, S. 2013. Hardwood Biomass to Gasoline, Diesel, and Jet Fuel: 1. Process Synthesis and Global Optimization of a Thermochemical Refinery. Energy & Fuels 2013 27 (8), 4302-4324. https://www.doi.org/10.1021/ef302003f

- Sikarwar, V., Zhao, M., Clough, P., Yao, J., Zhong, X., Memon, Shah, M., Anthony, E., and Fennell, P. 2016. An overview of advances in biomass gasification. Energy Environ. Sci. 2016, 9, 2939. https://doi.org/10.1039/C6EE00935B

- Molino, A., Chianese, S., Musmarra, D. 2016. Biomass gasification technology: The state of the art overview, In Journal of Energy Chemistry, Volume 25, Issue 1, 2016, Pages 10-25, ISSN 2095-4956, https://doi.org/10.1016/j.jechem.2015.11.005

- Velocys. 2017. The biomass-to-liquids process. http://www.velocys.com/our-biorefineries/

- Chandra, P.K. & Payne, Fred. 1986. Turndown Ratio of a Gasifier-Combustor Predicted by a Simulation Model. Transactions of the ASAE. 29. 1748-1753. https://doi.org/10.13031/2013.30383.

- Danish Energy Agency. 2017. Energy Statistics 2015. https://ens.dk/sites/ens.dk/files/Statistik/energy_statistics_2015.pdf

- Li X, Rubæk GH, Müller-Stöver DS, Thomsen TP, Ahrenfeldt J and Sørensen P. 2017. Plant Availability of Phosphorus in Five Gasification Biochars. Front. Sustain. Food Syst. 1:2. https://www.doi.org/10.3389/fsufs.2017.00002

- Velocys. 2017. Establishment of a strategic alliance with TRI. http://www.velocys.com/establishment-of-a-strategic-alliance-with-tri/

- Velocys. 2017. Velocys plans for the construction of a commercial BTL plant move forward. https://biorrefineria.blogspot.ca/2017/06/velocys-plans-for-construction-of-commercial-BTL-plant-move-forward.html

- Tijm, P. 1994. Shell Middle Distillate Synthesis: The Process, The Plant, The Products. http://web.anl.gov/PCS/acsfuel/preprint%20archive/Files/39_4_WASHINGTON%20DC_08-94_1146.pdf

- van der Laan, P. 1999. Kinetics, Selectivity and Scale Up of the Fischer-Tropsch Synthesis. http://www.adktroutguide.com/files/1999_Phd_Thesis_-_Kinetics_Selectivity_and_Scaleup_of_FT_Synthesis.pdf

- Industrial Equipment News. 2017. Mississippi Lands Biofuel Refinery. https://www.ien.com/operations/news/20980331/mississippi-lands-biofuel-refinery

- Irena. 2016. Innovation Outlook. Advanced Liquid Biofuels. http://www.irena.org/DocumentDownloads/Publications/IRENA_Innovation_Outlook_Advanced_Liquid_Biofuels_2016.pdf

- Sub Group on Advanced Biofuels. 2017. Cost of Biofuel. http://platformduurzamebiobrandstoffen.nl/wp-content/uploads/2017/07/2017_SGAB_Cost-of-Biofuels.pdf

- de Jong, S., Hoefnagels, R., Faaij, A., Slade, R., Mawhood, R. and Junginger, M. 2015. The feasibility of short-term production strategies for renewable jet fuels – a comprehensive techno-economic comparison. Biofuels, Bioprod. Bioref., 9: 778–800. https://www.doi.org/10.1002/bbb.1613

Methanol from Biomass Gasification

Qualitative Description

The front end of this process is identical to the gasification process in the previous description. The production of methanol from biomass is a two-step process, in the first step the solid biomass is converted to the gas phase and in the second step the gas is converted to methanol.

Gasification is a process that converts organic or fossil-based carbonaceous materials at high temperatures (>700°C), without combustion, with a controlled amount of oxygen and/or steam into carbon monoxide, hydrogen, and carbon dioxide (syngas). Stoichiometry for methanol production of syngas requires the ratio of H2/CO to equal 2. The H2/CO ratio can be lowered to some extent by the reverse water-gas shift reaction

Depending on the catalyst supplier, the methanol synthesis reaction is normally carried out at about 40 to 120 bar and 200 to 300°C.

Methanol is not the only product that could be produced by this route. Dimethyl Ether (DME) could also be produced instead of methanol or in an additional process step. The methanol could also be further processed into gasoline.

Contact Information

Don O’Connor – (S&T)2 Consultants Inc.

Brief Technology Description

The biomass could be agricultural or forestry residues. There is a wide range in the design of gasifiers used for biomass. Different technological solutions can be implemented in order to obtain different plant configurations; in particular, the mode of contact of the biomass with the gasification agent may be in counter-current, or co-current, or crossflow, and the heat can be transferred from the outside or directly in the reactor using a combustion agent; the residence time can be in the order of hours (static gasifiers, rotary kiln) or minutes (fluidized bed gasifiers). Different gasifier designs are better suited to different feedstocks and gas needs.

The syngas to methanol reactions are practised commercially mostly using natural gas to produce the syngas but there are a few plants that gasify coal to produce the syngas. While the scale of commercial plants is large there have been some small scale methanol plants built where large natural gas reserves are not available.

The overall process is shown in the following simplified process flow diagram.

Figure 10-1	Biomass to Methanol Process

Input

The primary input for most process is just the biomass. The reactions are exothermic and generate enough heat for the process and in some cases to produce the power required for the system. In other examples power is purchased for the process.

Output

The plants produce methanol and in some cases could produce some excess power and or steam for sale.

Energy Balance

The energy balance for a biomass to methanol system is shown in the following figure [1].

Figure 10-2	Bio Methanol Energy Balance

There are two potential means to recover some of the waste heat. The plants use some of the process heat to produce electricity for the plant use and potentially a small amount to be exported. Steam from the exit of the final steam turbine would be available for other uses. This could have a temperature between 150 and 185C depending on the design. There may also be some opportunity to recover some lower grade heat as the syngas is conditioned prior to synthesis. Details of the potential for energy recovery are not reported in most of the recent techno-economic studies published.

Other biomass to methanol systems have been proposed that offer higher efficiencies [2] [3]. The GreenSynFuels project provided the energy balance for both a traditional biomass to methanol plant and one integrated with a solid oxide electrolyzer to produce hydrogen to provide a better CO to H2 ratio for the methanol synthesis stage. Clausen provided information for a highly optimized biomass to methanol process. The energy balances for these systems are shown in the following figures.

Figure 10-3	GreenSynFuels Traditional Methanol Plant

This plant produces electricity instead of consuming it and the methanol production rate is slightly higher per unit of wood consumed. The following figure shows the highly optimized system described by Clausen. The methanol production rate is 8% higher per unit of feedstock.

Figure 10-4	Optimized Biomass to Methanol Plant

This final energy balance system considers the supplementation of hydrogen to alter the carbon to hydrogen ration of the syngas to better match the methanol synthesis requirements. It produces more methanol per unit of energy in and has a much better carbon efficiency.

Figure 10-5	Hybrid Biomass to Methanol Plant

Typical Capacities

There are currently no commercial biomass to methanol plants in operation. In the past OCI operated a former natural gas to methanol plant on crude glycerine from biodiesel plants as the feedstock in the Netherlands but that operation is now processing natural gas again. There was also a bioDME pilot plant operated in Sweden for a number of years where methanol production was an intermediate product. It gasified black liquor from a pulp mill rather than biomass.

Commercial plants would likely be similar in size to the biomass to diesel and jet technology that was discussed in the previous section, with and early commercial plant consuming 500 to 1000 tpd of biomass and producing 125 to 250 million litres/year.

Eventually plants could be built larger with feedstock availability being the limiting factor.

Regulation Ability

While biomass gasifiers can operate down to about 35% of rated capacity, commercial methanol plants usually operate at steady state conditions close to the design capacity. Commercial methanol plants can take 2-3 days to reach full production so starting and stopping the plants is generally not an option for regulating capacity. Smaller scale systems with different catalysts may have better regulation capabilities than the large scale plants.

Space Requirements

Space requirements will be similar to the space for the biomass to diesel and jet pathway, on an area per feedstock basis. The area per volume of fuel produced will be lower due to the lower energy density of methanol compared to diesel and jet fuel.

Based on the Velocys commercial FT liquids plant the area requirements for biomass to methanol are about 0.16 ha/million litres of methanol.

Advantages/Disadvantages

Methanol is not widely used as a transportation fuel today but there are several potential emerging applications that are generating some interest. One is the use of methanol as a hydrogen carrier for fuel cell vehicles such as those developed by Serenergy in Denmark. There is also some interest in methanol as a marine fuel to meet the new IMO sulphur limitations. In China there is some methanol gasoline blending with 10 and 15% methanol. Low level methanol blends (3%) with a co-solvent have been used in the UK in recent years. Methanol has also been used in blends with ethanol and gasoline in performance vehicles.

Methanol from biomass would offer GHG emission benefits for many applications that are not available for methanol produced by natural gas.

Much of the world’s methanol is produced from stranded natural gas and is very low cost. It will be difficult for biomass to methanol to complete against these projects on only an economic basis.

Environment

Biomass to methanol should have a very low GHG emission profile, especially when they are designed to be self-sufficient in electric power.

Methanol as a fuel is a biodegradable product.

Research and Development Perspective

Biomass gasification for methanol production from wood or straw is a category 2 technology, a pioneer phase technology with limited applications to date. The technology has been proven to work through demonstration facilities or semi-commercial plants. However, due to the limited application, the price and performance is still attached with high uncertainty, since development and customization is still needed. The technology still has a significant development potential.

This technology pathway is the combination of two commercial systems. There has been considerable development work on biomass gasification in Europe over the past several decades but there has not been a commercial break through yet.

The production of a synfuel from a biomass gasification system is a more demanding application than the use of the gas in an engine or in an external combustion system. It is reported that the Chemrec BioDME plant operated for more than 11,000 hours between 2011 and 2016 [4]. Production during that time was reported to be 1000 tonnes of DME. The capacity of the plant was 165 kg/hour which works out to 6,000 hours of operation.

More work is required on the integration of the two main systems.

Examples of Market Standard Technology

There are no market standard technologies. There is a biomass gasification to methanol proposal for a plant in Sweden, Värmland Methanol [5]. The plant is cost estimated at approximately 350 million Euro and will produce 375,000 liters of methanol per day (130 million litres/year). As a "byproduct" 15 MW of district heating is obtained.

An EPC contract with ThyssenKrupp Industrial Solutions of Germany has been signed. The project was proposed in 2009 but has been unable to raise financing for the project. ThyssenKrupp Industrial Solutions do have experience and expertise in gasification and methanol production technologies.

Enerkem, a Canadian company has operated a municipal solid waste (MSW) gasification to methanol production system in Edmonton Alberta for the past two years. The company is focussed on MSW as a feedstock due to the favourable economics. The Edmonton plant is in the process of being converted to produce ethanol rather than methanol from the syngas.

Predication of Performance and Cost

The prediction of performance and cost is based on published techno-economic papers rather than on actual plant performance.

Uncertainty

There is a high level of uncertainty for the technology given the state of development and the fact that there are no operating plants in the world at this time.

Additional Remarks

There is a Danish Methanol Association promoting bio methanol but the syngas is produced from biogas and not from biomass. These systems will be smaller due to the resource availability but with less technology risk since there is no biomass gasification involved.

Quantitative Description

The available quantitative data that is available on the technology is mostly from third parties and not from the technology providers or plant operators. No actual plant data is available.

Energy/Technical Data

There are three basic reactions that occur in the process. The first reaction breaks the biomass down to a combination of hydrogen, carbon monoxide and carbon dioxide. A simplified reaction is shown below. Actual biomass has highly variable composition and complexity with cellulose as one major component.

C6H12O6 + O2 + H2O → CO + CO2 + H2 + CH4 + other species

Note: The above reaction uses glucose as a surrogate for cellulose.

Stoichiometry for methanol production of syngas requires the ratio of H2/CO to equal 2. The product gases are then subjected to the water-gas shift reaction to increase the quantity of hydrogen. The equilibrium for this reaction is temperature dependent which controls the CO to CO2 ratio.

CO + H2O ↔ CO2 + H2

Carbon monoxide and hydrogen react over a catalyst to produce methanol. Today, the most widely used catalyst is a mixture of copper and zinc oxides, supported on alumina. At 50–100 bar and 250 °C, the reaction is characterized by high selectivity (>99.8%):

CO + 2 H2 → CH3OH

Compared to the production of diesel and jet fuel from the gasification of biomass, this pathway requires a lower H2/CO ratio and operates at lower temperatures but higher pressures.

Typical Plant Size

As with the biomass to diesel and jet process, the plant size will be determined by the feedstock availability. The proposed plant in Sweden would produce 130 million litres of methanol per year (65 MW) from 1,100 tpd of wood [6]. It is not stated but this is likely on a wet basis (660 dry tpd).

NREL undertook a techno-economic analysis of a wood to methanol plant [7]. They based the plant on 2000 tpd of feedstock producing 380 million litres per year (200 MW).

Input and Output

The input and output of a typical system are shown in the following table [2]. These will be nth plant values. Pioneering plants may have a lower efficiency.

Table 10-1	Inputs and Outputs

| Parameter | Input | Output |
| --- | --- | --- |
| Wood, dry | 100 MJ |  |
| Power |  | 1.8 MJ |
| Methanol |  | 58.2 MJ |

Forced and Planned Outage

The plants are expected to operate for 350 days per year. Wood gasifiers are capable of operating at these rates as shown earlier and fossil methanol plants are also capable of operating at these rates. Forced outages are expected to be minimal.

      - Technical Lifetime

The plants should be constructed for more than a 25 year life expectancy.

Construction Time

Construction time for the technology is expected to be about 24 -36 months.

Financial Data

Techno-economic analyses of standalone biomass gasification to methanol systems have been published [1] [7]. These are used as the basis for the financial analysis and where possible compared to the published data for the proposed Swedish plant.

Investment Costs

The estimated capital cost of the 270,000 tonne per year methanol plant is 369 million Euros [1] (0.91 €/litre). This is the same cost as the Värmland Methanol plant but it is more than twice the capacity. This is a reasonable ratio between the nth plant and a pioneering plant.

The NREL plant [5] was $259 million in 2007 dollars, this would be €0.75/litre.

The EU Sub Group on Advanced Biofuels reported plant sizes in the range of 100 to 200 MW and capital costs of €1,850 to 3,450/kW depending on the scale (€1.00/litre  to €1.85/litre).

The capital costs range from €0.75/litre (NREL) to €2.70/litre (Värmland).

Operating and Maintenance Costs

Anderson only presented the combined operating and maintenance costs (Costs for wages, insurance for employees and chemical, water and ash disposal) of €0.10/kg (€0.08/litre). NREL did not provide a breakdown of the operating costs.

Start-up Costs

The start-up costs are included in the costs reported.

Technology Specific Data

Some the properties of methanol are shown in the following table.

Table 10-2	Methanol Properties

| Property | Value |
| --- | --- |
| Density, kg/m3 | 791 |
| LHV, MJ/kg | 19.9 |
| LHV, MJ/litre | 15.7 |
| Oxygen content | 50% |
| Blending Octane number | ~115 |
| Flash point, C | 12 |

Summary

The quantitative data for the biomass to methanol process are summarized in the following table.

Table 10-3	Bio Methanol Datasheet

| Technology | Bio Methanol |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2030 | 2050 | Uncertainty (2020) |  | Uncertainty (2050) |  | Note | Ref |
|  |  |  |  |  | Lower | Upper | Lower | Upper |  |  |
| Energy/technical data |  |  |  |  |  |  |  |  |  |  |
| Typical total plant size 1,000 t Methanol/year | 100 | 100 | 200 | 300 | 50% | 200% | 50% | 125% | A, B | 3, 5, 6 |
| Typical total plant size, MW | 65 | 65 | 130 | 195 | 50% | 200% | 50% | 125% | A, B | 3, 5, 6 |
| Inputs |  |  |  |  |  |  |  |  |  |  |
| Feedstock Consumption, MWh/MWh Total Input | 1.0 | 1.0 | 1.0 | 1.0 | 90% | 150% | 90% | 120% |  | 1 |
| Outputs |  |  |  |  |  |  |  |  |  |  |
| Methanol Output, MWh/MWh Input | 0.58 | 0.58 | 0.61 | 0.65 | 100% | 133% | 100% | 133% | D | 1 |
| Co-products Output, MWh/MWh Total Input | 0.25 | 0.25 | 0.25 | 0.25 | 0% | 100% | 0% | 100% | D | 1 |
| Forced outage (%) | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Planned outage (weeks per year) | 2 |  |  |  |  |  |  |  |  |  |
| Technical lifetime (years) | 25 |  |  |  |  |  |  |  |  |  |
| Construction time (years) | 2 - 3 |  |  |  |  |  |  |  |  |  |
| Financial data |  |  |  |  |  |  |  |  |  |  |
| Specific investment (€ /l Methanol) | 2.7 | 2.7 | 1.5 | 0.75 | 50% | 100% | 80% | 120% |  | 1, 3, 5, 6 |
| - equipment (%) | 75 | 75 | 75 | 75 |  |  |  |  |  |  |
| -installation (%) | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| Fixed O&M (€ /l Methanol) | 0.03 | 0.03 | 0.02 | 0.02 | 90% | 110% | 90% | 110% | E | 1 |
| Variable O&M (€ /l Methanol) | 0.09 | 0.09 | 0.06 | 0.06 | 90% | 110% | 90% | 110% | E | 1 |
| Start up (€ /l Methanol) | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Technology specific data |  |  |  |  |  |  |  |  |  |  |

Notes:

- The plant size range is assumed based on the proposed Värmlands plant and the NREL nth plant.

- Feedstock availability is likely to determine the maximum plant size.

- Some plants may produce their own power and have no power imports.

- Plants that produce their own power will have much lower heat available to district heat.

- Assumed a 25/75 split on fixed to variable operating costs.

References

- Andersson, J.,Lundgren, J., and Marklund, M. 2014. Methanol production via pressurized entrained flow biomass gasification – Techno-economic comparison of integrated vs. stand-alone production, In Biomass and Bioenergy, Volume 64, 2014, Pages 256-268, ISSN 0961-9534, https://doi.org/10.1016/j.biombioe.2014.03.063

- Danish Technology Institute. 2011. GreenSynFuels. EUDP project journal number: 64010-0011. http://serenergy.com/wp-content/uploads/2015/11/GreenSynFuels_report_final.pdf

- Clausen, L. 2014. Integrated torrefaction vs. external torrefaction – A thermodynamic analysis for the case of a thermochemical biorefinery. Energy, Volume 77, Pages 597-607. https://doi.org/10.1016/j.energy.2014.09.042.

- IEA Bioenergy Task 39. 2016. IEA-Bioenergy-Task-39-Newsletter-Issue-42-April-2016. http://task39.sites.olt.ubc.ca/files/2012/01/IEA-Bioenergy-Task-39-Newsletter-Issue-42-April-2016.pdf

- Värmlands Metanol AB. http://www.varmlandsmetanol.se/Om%20Projektet.htm

- Värmlands Metanol AB. 2016. Metanol från skog - ett miljövänligt drivmedel. http://www.varmlandsmetanol.se/dokument/Folder%20VM%20sept%202016.pdf

- NREL. 2011. Technoeconomic comparison of biofuels: ethanol, methanol, and gasoline from gasification of woody residues. https://www.nrel.gov/docs/fy12osti/52636.pdf

- Sub Group on Advanced Biofuels. 2017. Cost of Biofuel. http://platformduurzamebiobrandstoffen.nl/wp-content/uploads/2017/07/2017_SGAB_Cost-of-Biofuels.pdf

Methanol from Power

Qualitative Description

Methanol to power is a concept that combines carbon dioxide with hydrogen produced by electrolysis to produce methanol. There is one plant in operation in Iceland (Carbon Recycling International, CRI). The plant produces 5 million litres per year of methanol. The plant uses electricity from the grid to make hydrogen which is converted into methanol in a catalytic reaction with carbon dioxide (CO2). The CO2 is captured from flue gas released by a geothermal power plant located next to the CRI facility. The origin of the flue gas is geothermal steam emissions.

There are many possible sources of CO2 (power plants, fertilizer producers, ethanol plants) and variations of the concept have been proposed where renewable electricity sources replace the grid power used in Iceland. Smaller scale systems could use CO2 from biogas plants as an alternative to methanation.

The methanol could also be further processed into DME or gasoline with available commercial technologies.

Contact Information

Don O’Connor – (S&T)2 Consultants Inc.

Brief Technology Description

Methanol is produced commercially mostly via steam reforming natural gas. In that process the methane is reformed into hydrogen, carbon monoxide and carbon dioxide and then the gases are re-combined to produce methanol. In the power to methanol system separate sources of hydrogen and carbon dioxide are combined to produce the methanol as shown in the following figure.

Figure 11-1	Methanol to Power Process

Electricity is used to produce the hydrogen and to compress the gases prior to the methanol synthesis stage. The attractiveness of the process is that if low carbon power is used the produced methanol has a low carbon footprint.

Methanol can be used in a number of applications. Low level (3-5%) methanol has been blended with gasoline and co-solvents in Europe and North America in the past. Higher level blends (M85) has been used in flex fuel vehicles and methanol is a good source of hydrogen for fuel cell vehicles. The Danish methanol fuel cell manufacturer SerEnergy is now producing a commercial reformed methanol fuel cell vehicle with a driving range up to 800 km on a tank of methanol [1].

Input

The primary inputs to the system are electricity for hydrogen production and carbon dioxide. The carbon dioxide can come from many different sources. Alternatively the inputs could be carbon dioxide and hydrogen where the hydrogen would be produced by electrolysis.

Output

The output from the system is methanol and water.

Energy Balance

The energy balance is shown in the following figure. In this case some of the waste heat is utilized internally for the CO2 capture which is why the output totals less than 100 MJ [2] [3].

Figure 11-2	Methanol from Power Energy Balance

There are two sources of waste heat, from the electrolyzer used for hydrogen production and the methanol synthesis reaction. The electrolyzer energy loss is the larger of the two sources and is lower quality. The temperature of the waste heat will vary with the electrolyzer design and could range from 70 to 150°C for most designs. Solid oxide electrolyzers operate at much higher temperatures (700 to 800°C) and would have higher quality excess heat.

Typical Capacities

The one operating plant produces 5 million litres/year (4,000 tonnes of methanol, 3 MW) but it is as much a demonstration plant as a commercial operation. The EU funded MefCO₂ project brings together 8 partners throughout Europe together for one mission: to demonstrate the economic feasibility of valorising captured CO₂ by turning it into a versatile platform chemical and renewable fuel such as methanol using hydrogen produced from renewable energy surplus. Their business case foresees plants from 4,000 to 50,000 tonnes per year, with larger plants being possible depending on the availability of CO2 [4].

Atsonios et al [5] investigated the technical and economic aspects of methanol production through the power to methanol process. Their plant capacity was limited by the size of the electrolysis unit at 140 MWe. This would produce about 120,000 t of methanol per year. A facility this size would consume 165,000 t/year of CO2.

The 90 commercial methanol plants in the world have an average capacity of 1.2 million tonnes per year [6]. The power to methanol plants are considerable smaller than this and may be limited both by the availability of CO2 and the size of electrolyzers, although large electrolysis systems can be built with multiple electrolyzer stacks

Regulation Ability

The electrolyzers [7] have excellent load following capability and can respond to large load (100 to 25% of capacity) changes within a fraction of a second. The commercial natural gas methanol production process can take several days to reach operating capacity as both the natural gas reforming stage and the methanol synthesis stages operate at high temperature and pressure.

A power to methanol process won’t have the natural gas reforming stage but the methanol synthesis stage still operates at about 300 C and 85 bar of pressure. If this stage is not operated at a continuous rate there will be significant loss of overall efficiency for the process.

Space Requirements

A natural gas methanol plant has a space requirement of about 0.01 ha/million litres of methanol. Power to methanol plants will be smaller in overall size which will probably translate into more area per unit of production. The 4,000 tpy CRI plant in Iceland occupies 0.8 ha of land or 0.16 ha/million litres of methanol (3,000 m2/MW), although the site is not fully occupied.

Advantages/Disadvantages

With renewable power the GHG emissions for this pathway are extremely low. The technology will be limited in scale due to the supply of hydrogen and the availability of carbon dioxide.

The relatively small scale of the production systems will likely result in higher capital costs for the facilities. The methanol production process is not well suited to intermittent operation making it a poor match to wind and solar power generation without some storage capacity somewhere in the system prior to the methanol synthesis stage. Gotz [8] investigated the cost of hydrogen storage for the SNG pathway, which has a similar issue. He reported that for steady state operation of the methanation step the cost of hydrogen storage to deal with the intermittent power supply added 28% to the capital cost of the hydrogen supply electrolyzer.

Environment

The process is relatively benign, there are some catalysts used in the methanol synthesis stage but these can be recycled. The process produces water as well as the methanol.

The overall energy efficiency of the process is about 60%. The carbon dioxide input has no energy and thus all of the energy input into the process is supplied by the electricity. About 60% of the energy in the electricity ends up in the methanol, a small portion is used internally for thermal use and a third could be available as low grade heat for district heating systems.

Research and Development Perspective

Methanol production from power is a category 2 technology, a pioneer phase technology with limited applications to date. The technology has been proven to work through demonstration facilities or semi-commercial plants. However, due to the limited application, the price and performance is still attached with high uncertainty, since development and customization is still needed.

The technology is at the early stage of development although both the hydrogen production by electrolysis and the methanol synthesis are well established technologies. There is moderate development potential for the technology.

Examples of Market Standard Technology

Carbon Recycling International offers a standard Power-to-Liquids plant design with nominal 50,000 ton/yr methanol production capacity.

Carbon Recycling International

Holtasmari 1
201 Kopavogur
Iceland

Predication of Performance and Cost

Commercial scale plants will be at least an order of magnitude larger than the one operating facility and so the prediction of operating performance and costs at scale are uncertain. Some predictions of future costs have been made in the literature and theses are discussed later in this section.

Uncertainty

The one demonstration plant has been operating for several years and the process steps have been commercially practices for many years. The overall uncertainty is with the integration of the two technologies, the potential need for a third stage to produce the required CO2, and the scalability of the technologies.

Additional Remarks

The technology offers the opportunity to produce a liquid fuel (methanol) from electricity and is a potential means to address the range limitations of battery powered electric vehicles.

There are three types of electrolyzers that could be considered for the hydrogen supply. There are the traditional alkaline cells, the emerging polymer membrane systems and potentially solid oxide electrolysis cells [9]. The characteristics are of each are summarized in the following table.

Table 11-1	Electrolysis Systems

|  | Alkaline Cells | Polymer Cells | Solid Oxide Cells |
| --- | --- | --- | --- |
| Operating Temperature °C | 60-80 | 60-80 | 750-950 |
| Pressure | 32 bar | 100 bar potential | 100 bar potential |
| Electrical Efficiency (HHV) | 75-85% | 10-85% | 90% |
| Stage of Development | Commercial | Commercial | Under development |
| Products | H2 and O2 | H2 and O2 | H2 CO, and O2 |

Energy balance should improve as the more efficient polymer and solid oxide electrolyzers are developed and employed.

There are also technologies available that can produce gasoline from methanol [10] which would allow the fuel to be used in existing gasoline powered vehicles. The methanol to gasoline process is very efficient so that the overall efficiency of power to gasoline is not significantly lower than power to methanol. Capital costs are higher due to the extra process step.

Quantitative Description

The quantitative description of the technology is provided below.

Energy/Technical Data

The mass and energy balance information is discussed below along with information on the operating parameters.

Typical Plant Size

Carbon Recycling International is offering process packages for 50,000 tpy plants. This is an order of magnitude scale up from the existing facility. Other analysis has been undertaken on plants of 140,000 tpy [5] and 440,000 tpy [8].

Input and Output

The process requires carbon dioxide and hydrogen to produce methanol. The hydrogen would be produced through electrolysis so electricity becomes the process input rather than hydrogen. The CO2 could come from many different sources and depending on the source, there could be some thermal energy and electric power required to concentrate and purify the gas. For this work it has been assumed that the CO2 is the process input and is of the required quality.

Table 11-2	Mass and Energy Flows

| Parameter | Input | Output |
| --- | --- | --- |
| Electricity | 9.5 MWh |  |
| Hydrogen | (0.14 t H2) |  |
| Carbon Dioxide | 1.4 t |  |
| Methanol |  | 1 tonne |
| Water |  | 0.59 tonne |
| Oxygen |  | 1.5 tonne |

Forced and Planned Outage

The methanol synthesis stage of the process performs best when operated continuously, close to capacity and for extended periods of time. Forced outages need to be minimized and the plant should be able to operate at an average annual rate of 95% (15 days per year downtime).

Technical Lifetime

Methanol plants have long operating lifetimes. Plants should be designed to operate for at least 25 years.

Construction Time

Given the high temperature and pressure operating conditions for the methanol synthesis, the construction time could be expected to be two years.

Financial Data

The financial data is extracted from literature reports since no commercial plants have been built.

Investment Costs

The investment costs for three plant sizes from three reports are shown in the following table. The smallest plant has the highest capital costs, as one would expect. The medium and large plants have similar capital costs, reflecting either the uncertainty in estimating the costs or the lack of economies of scale once a certain plant size is arranged.

Table 11-3	Capital Costs

|  | Enea [12] | Atsonios [5] | Perez-Fortes [11] |
| --- | --- | --- | --- |
| Plant size, tpy | 8,400 | 120,000 | 440,000 |
| Total | 3,571 €/(t MeOH/yr) | 1,336 €/(t MeOH/yr) | 1,285 €/(t MeOH/yr) |

The capital costs range from €1.00/litre to €3.00/litre methanol.

Operating and Maintenance Costs

Atsonios estimates the O&M costs as 5% of fixed capital invested or €67 per tonne of methanol produced. Perez estimated the O&M costs as € 70 per tonne of methanol. It is assumed that 50% is fixed O&M and 50% id variable O&M.

Start-up Costs

Start-up costs are included in the O&M costs above, however this technology can take several days to start up and a similar time to shut down so the system performance will deteriorate significantly if the process is started up and shut down frequently. Start-up and shut down is also the most likely time for a mechanical failure resulting in forced downtime.

Technology Specific Data

The power requirements to produce 1 GJ of methanol (LHV) are 470 kWh/GJ. Carbon intensities of power of less than 190 g CO2/kWh will result in GHG emissions that are less than gasoline.

Summary

The technical information is summarized in the following table.

Table 11-4	Methanol from Power Datasheet

| Technology | Methanol from Power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2030 | 2050 | Uncertainty (2020) |  | Uncertainty (2050) |  | Note | Ref |
|  |  |  |  |  | Lower | Upper | Lower | Upper |  |  |
| Energy/technical data |  |  |  |  |  |  |  |  |  |  |
| Typical total plant size 1,000 t Methanol/year | 5 | 50 | 100 | 150 | 50% | 150% | 50% | 150% | A, B | 4, 5, 11 |
| Typical total plant size, MW | 3 | 33 | 65 | 100 | 50% | 150% | 50% | 150% | A,B | 4, 5 |
| Inputs |  |  |  |  |  |  |  |  |  |  |
| CO2 Consumption, t/t Methanol | 1.37 | 1.37 | 1.37 | 1.37 | 100% | 110% | 100% | 110% |  | 5 |
| Hydrogen Consumption, t/t Methanol | 0.192 | 0.192 | 0.192 | 0.92 | 100% | 110% | 100% | 110% |  | 5 |
| Electricity Consumption, MWh/MWh Total Inputs | 1.0 | 1.0 | 1.0 | 1.0 | 90% | 120% | 95% | 110% | C | 2, 3 |
| Outputs |  |  |  |  |  |  |  |  |  |  |
| Methanol Output, MWh/MWh Total Input | 0.59 | 0.59 | 0.61 | 0.64 | 100% | 100% | 100% | 100% | D | 2, 3 |
| Co-products Output, MWh/MWh Total Input | 0.0 | 0.0 | 0.0 | 0.0 | 100% | 100% | 100% | 100% | E | 2, 3 |
| Forced outage (%) | 0 | 0 | 0 | 0 |  |  |  |  | F |  |
| Planned outage (weeks per year) | 2 |  |  |  |  |  |  |  |  |  |
| Technical lifetime (years) | 25 |  |  |  |  |  |  |  |  |  |
| Construction time (years) | 2.0 |  |  |  |  |  |  |  |  |  |
| Financial data |  |  |  |  |  |  |  |  |  |  |
| Specific investment (€ /l Methanol) | 2.0 | 2.0 | 1.5 | 1.0 | 75% | 125% | 75% | 125% | G | 5, 8, 9 |
| - equipment (%) | 75 | 75 | 75 | 75 |  |  |  |  |  |  |
| -installation (%) | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| Fixed O&M (€ /l Methanol) | 0.03 | 0.03 | 0.03 | 0.03 | 90% | 110% | 90% | 110% |  | 5 |
| Variable O&M (€ /l Methanol) | 0.03 | 0.03 | 0.03 | 0.03 | 90% | 110% | 90% | 110% |  | 8 |
| Start up (€ /l Methanol) | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Technology specific data |  |  |  |  |  |  |  |  |  |  |

Notes:

- The plant size range is based on the CRI plants and other analysis in the literature.

- CO2 availability is likely to determine the maximum plant size.

- Power use improvement depends on improved efficiency for electrolyzers.

- Methanol is the only output.

- Waste heat is of low quality.

- The methanol synthesis step does not lend itself to intermittent operation. The concept of intermittent power to methanol is not practical without intermediate storage for hydrogen that would allow the methanol plant to operate continuously. This would have a significant capital cost impact.

- There is a wide range of capital costs in the literature.

References

- Next generation of methanol fuel cell vehicles sees the light of day. 2017. http://serenergy.com/next-generation-of-methanol-fuel-cell-vehicles-sees-the-light-of-day/

- Stefansson, B. 2015. Power and CO2 emissions to methanol. https://eu-ems.com/event_images/presentations/Benedikt%20Stefansson%20presentation.pdf

- Stefansson, B. 2017. CO2 to Renewable Methanol: experiences and perspectives for the steel industry. https://www.co2-cato.org/cato-download/4123/20170512_143353_20-11.00-Stefansson-public.pdf

- MefCO2 Project Brochure. 2017. http://www.mefco2.eu/pdf/MefCO2_Brochure_00.pdf

- Atsonios K, et al., Investigation of technical and economic aspects for methanol production through CO2 hydrogenation, International Journal of Hydrogen Energy (2016), http://dx.doi.org/10.1016/j.ijhydene.2015.12.074

- Methanol Institute. http://www.methanol.org/the-methanol-industry/

- Hovsapian, R. 2017. Role of Electrolyzers in Grid Services. US DOE. https://energy.gov/sites/prod/files/2017/06/f34/fcto_may_2017_h2_scale_wkshp_hovsapian.pdf

- Manuel Götz, Jonathan Lefebvre, Friedemann Mörs, Amy McDaniel Koch, Frank Graf, Siegfried Bajohr, Rainer Reimert, Thomas Kolb, Renewable Power-to-Gas: A technological and economic review, In Renewable Energy, Volume 85, 2016, Pages 1371-1390, ISSN 0960-1481, https://doi.org/10.1016/j.renene.2015.07.066

- Danish Technology Institute. 2011. GreenSynFuels. EUDP project journal number: 64010-0011. http://serenergy.com/wp-content/uploads/2015/11/GreenSynFuels_report_final.pdf

- Haldor Topsøe. 2017. Gasoline synthesis (TIGAS). https://www.topsoe.com/processes/gasoline-synthesis/gasoline-synthesis-tigas

- M. Pérez-Fortes and E. Tzimas; Techno-economic and environmental evaluation of carbon dioxide utilisation for fuel production. Synthesis of methanol and formic acid; EUR 27629 EN; https://dx.doi.org/10.2790/981669

- enea Consulting. 2016. The Potential of Power to Gas. Technology review and economic potential assessment. http://www.enea-consulting.com/wp-content/uploads/2016/01/ENEA-Consulting-The-potential-of-power-to-gas.pdf

- CRI. 2017. Commercial Scale Plants. http://carbonrecycling.is/comercial-scale/

SNG

Qualitative Description

Synthetic Natural Gas (SNG) is produced by the methanation of biogas. The main components in biogas are methane and CO2. The content of CO2 may vary between about 35-50 vol. % depending on the actual biogas production technology. The carbon dioxide in the biogas is reacted with hydrogen to produce additional methane.

This chapter does not consider the production of the hydrogen as there are multiple production options available.

Biogas, hydrogen production, and methanation are all commercial technologies but the combination of the three processes is a concept that is still at the research, development and deployment stage.

Contact Information

Don O’Connor – (S&T)2 Consultants Inc.

Brief Technology Description

The core unit in a methanation plant is the hydrogenation/methanation unit in which CO2 is converted to methane by reacting with hydrogen. A catalyst is usually used for this process. The methanation reaction is exothermic and will generate large amount of heat. There is also some research and development work being undertaken on biological conversion routes [1]. This section is based on the chemical route.

Figure 12-1	SNG Process

Input and Output

The systems require biogas, hydrogen and electricity to operate. They produce methane suitable for injection into the gas distribution grids, a small amount of water and some recoverable heat.

Energy Balance

The energy balance is presented as a Sankey diagram in the following figure. Note that this is based on hydrogen and the energy required to produce the hydrogen is not included. The data in the following table is based on “process concept 1” in the Swiss Federal Office of Energy report [2]. This is based on the biogas having 65% methane; gas with less methane will require more hydrogen and have a different energy balance.

Figure 12-2	SNG Energy Balance

The methanation stage is relatively efficient with some high temperature (>100 C) heat recoverable from the process.

Typical Capacities

The capacities of the systems will likely be limited by the size of the biogas plants. Gotz et al [1] reports that in 2016 the largest demonstration plant was the Audi e-gas plant in Wertle, Germany with a hydrogen production capacity of 6 MW. The plant output was 325 NM3/hr (88,000 GJ SNG/year) (3 MW) [3]. Gotz et al undertook their techno-economic analysis on a plant that produced 1000 NM3/hr (270,000 GJ/year) (9 MW).

The concept could also be applied to other sources of carbon dioxide such as biomass gasification or industrial processes that produce high quality CO2 such as ethanol production, some natural gas processing plants and processes such as ethylene oxide production [1]. Larger plants may be possible with these systems.

Regulation Ability

Without storage capacity of the biogas and the hydrogen the overall system will have very limited regulation capacity, as both inputs are required for the methanation stage.

The methanation process itself operates at elevated temperatures and load changes can induce unwanted temperature changes in the catalyst beds. Steady state operation will provide the optimum performance.

Space Requirements

The COSYMA (Container Based System for Methanation) pilot plant in Switzerland put the entire methanation process, including compressor and gas cleaning, is integrated into a standard 20 feet shipping container [2]. This was a small system but the space requirements will be less than the space requirements of the biogas facility.

Advantages/Disadvantages

The SNG concept can be considered another variation of the power to energy concept, although any source of hydrogen could be utilized in the process the most likely embodiment of the concept will produce the hydrogen from electricity. The process would essentially double the quantity of low carbon natural gas that can be produced from biogas plants and there is a ready market for the gas in the natural gas distribution systems.

The size of the plants will be limited by the capacity of biogas plants, which tend to be relatively small. Economies of scale that could reduce the production costs may be difficult to obtain. The potential to standardize and produce a number of systems that have an integrated electrolyzer and methanation unit may provide some help in reducing system costs.

Larger systems based on other sources of carbon dioxide may have more favourable economics and the potential to produce greater quantities of SNG. The GoBiGas project in Göteborg Sweden was design to produce 72 GJ/hour (576,000 GJ/year) of SNG in the first phase and eventually 2.3 to 2.9 million GJ/year in a final phase [4].

Environment

The overall impact on the GHG emissions of the produced SNG will be dependent on how the hydrogen is produced, hydrogen from electrolysis produced with renewable electricity will produce SNG with low GHG emissions. Some of the other potential systems which utilize CO2 from renewable sources may also have good emission profiles but CO2 from fossil systems will not produce renewable natural gas.

The environmental impact from the biogas production system and from the hydrogen production system will be larger than from the methanation process.

Research and Development Perspective

SNG production from power is a category 2 technology, a pioneer phase technology with limited applications to date. The technology has been proven to work through demonstration facilities or semi-commercial plants. However, due to the limited application, the price and performance is still attached with high uncertainty, since development and customization is still needed. The technology still has a significant development potential.

As noted the three sub-processes are all commercial processes although the commercial methanation processes are generally at a larger scale than would be required for biogas plants. What is required for commercialization is the system integrators who can integrate hydrogen production and methanation into a system that can be added to the biogas production unit.

Examples of Market Standard Technology

Haldor Topsøe offers methanation technology, their TREMP™ process. The applications of the technology have been in larger fossil based industries.

The applications of the technology to smaller biogas facilities has been at the demonstration plant level, so no market standard technology has developed yet. There are at least three groups that are promoting their technology demonstrations.

The Paul Scherrer Institute in Switzerland along with their partners energie 360 and erdgas biogas have the Biosweet project that has an operating system at Zurich Biogas [2]. They have published results of their system analyses and demonstration plant performance.

The Vienna University of Technology [5] is offering a system with a single stage methanation step and membrane cleanup of the gas. They claim low capital and operating costs.

HZI Etogas develops and builds Power-to-Gas Plants. They built the Audi e-gas plant in Wertle, Germany in 2013. They claim to offer a modular structure that is suited for outdoor use. The plant consists of the following operating modules:

- power electronics with rectifiers (connection to medium voltage level)

- water treatment system

- hydrogen production (alkaline pressurized electrolysis)

- CO2 feed gas treatment

- methanation reactor

- cooling system

- ancillary systems

The parent company is

Hitachi Zosen Inova.

Hardturmstrasse 127, 8005 Zurich, Switzerland.

They have recently received an order for a pilot facility in Japan that will take fossil CO2 emissions from a coal fired power plant and combine them with hydrogen to produce SNG which will then be fed into an existing gas grid [6]. HZI will supply the electrolyzer and the methanation unit.

Predication of Performance and Cost

Each of the three processes involved in the concept is a commercial process at some scale. What is not well developed is the integration of the three processes and the demonstration of the methanation at a suitable scale. With very limited commercial deployment of the concept the predictions of performance and costs is developed from published reports and peer reviewed papers.

Uncertainty

Given the lack of commercial development and the potential for variations in the methanation process configuration there is a relatively high level of uncertainty with respect to the performance and cost.

Additional Remarks

The Audi e-gas plant in Wertle Germany is shown in the following photo.

Figure 12-3	SNG Methanation Plant

As with the discussion of power to methanol there are multiple options for hydrogen production for SNG systems. Haldor Topsøe designed and constructed a pilot plant that became operational April 2016 [7]. The design capacity is 10 Nm3/h of upgraded biogas. This capacity requires approx. 50 kW solid oxide electrolyzer cell, which is provided by two Fuel Cores, each consisting of 4 SOEC stacks. Haldor Topsøe A/S also designed the biogas cleaning unit and the methanation plant which is located at the Agricultural Research Centre of Aarhus University at Foulum, Jutland.

The system has proven to be capable of withstanding power loss with having a significant negative impact on the methanation catalyst. The high efficiency of the SOEC hydrogen production system should lead to a better energy balance than show in Figure 11-1. The facility is shown below.

Figure 12-4	Haldor Topsøe SNG Pilot Plant

Quantitative Description

The theoretical chemical reaction that occurs with methanation of biogas is shown below.

CO2 + 4H2 → CH4 + H2O

The reaction is exothermic (releases heat) and the reactors are operated at temperatures below 200 °C (at 1 bar) or 300 °C (20 bar) to reach conversion rates of greater than 98%.

The catalytic methanation reaction has been known since 1902. The technology has been developed for large scale coal to gas plants. The application of the technology may require some different concepts for the smaller scale and potentially intermittent or dynamic operation.

Energy/Technical Data

Quantitative data on the biogas to SNG process is presented below.

Typical Plant Size

It is the biogas plant that will determine the typical plant size and the hydrogen production and the methanation stages can be easily scaled to match the biogas output. Centralized Danish biogas plants range in size from about 70,000 to 700,000 GJ/year [8]. The trend is towards larger plants, driven by cost reductions related to economy-of-scale effects for the biogas plant and in particular the upgrading facilities.

Biogas SNG systems have about double the gas output of the biogas or 140,000 to 1.4 million GJ per year. This range is from 2 to 20 times the size of the operating Audi e-gas plant in Germany.

Input and Output

The inputs and outputs for a small system are shown in the following table [2]. The inputs to the system are biogas, hydrogen, and a small amount of power. The system output is SNG with about 98% methane. There are a number of possible process configurations involving the number of reactor stages, the operating pressures, and gas upgrading schemes.

Table 12-1	SNG Inputs and Outputs

| Parameter | Input | Output |
| --- | --- | --- |
| Biogas, 65% CH4 | 4,777 MJ |  |
| Hydrogen | 4,108 MJ |  |
| Electricity | 19 kwh |  |
| SNG |  | 8,000 MJ |

Forced and Planned Outage

Ten to 15 days per year of downtime should be expected for the system for routine maintenance and changing catalysts.

Technical Lifetime

The technical lifetime of the systems should be greater than 25 years.

Construction Time

The construction time should be less than one year given the relatively small size and the ability modularize the hydrogen production and methanation stages of the process.

Financial Data

The most recent and comprehensive analysis of capital and operating costs for Bio SDMG systems is the work of Gotz [1].

Investment Costs

Gotz surveyed the literature and reported a wide range in the capital costs for methanation from a low of 36,000 €/GJ/hr SNG to 415,000 €/GJ/hr SNG. A small plant producing 70,000 GJ/year could have capital costs of 3.36 million € to a high of 29 million € (€48/GJ to €414/GJ). Gotz concluded that the costs at the low end of the range were the most realistic as they were recently (2014) developed by an engineering company.

The capital costs of the methanation process were less than 5% of the capital costs of the electrolyzer, compressor and hydrogen storage requirements.

Operating and Maintenance Costs

Operating costs are dominated by the cost of hydrogen, which is excluded in this analysis. There is some electricity that is required for the operation of the methanation process but it is small compared to the hydrogen costs. Little information in the literature was identified for operating and maintenance costs. The fixed O&M costs are assumed to be 4% of the capital and the variable portion as 4% of the capital costs.

Start-up Costs

The start-up costs are expected to be low in a well-designed system. Some capital investment on intermediate storage of hydrogen and or biogas will help to minimize the need for gas venting or flaring during start-up.

Technology Specific Data

The composition of the SNG will depend on the methanation design and the approach used for gas clean-up. The data from the Swiss project for the gas composition before the final clean-up are shown in the following figure.

Figure 12-5	Gas Composition Prior to Final Clean-up

With some gas clean-up methane levels of greater than 95% can be achieved. Unlike the composition of fossil natural gas there is little to no C2 to C4 components present. The heating value of the gas may be lower than the typical fossil natural gas as a result, however as it is expected that the gas will be injected into the grid and co-mingled with fossil natural gas at a low concentration, there will be no impact on the final users.

Summary

The information on the SNG production process is summarized in the following table.

Table 12-2	SNG Datasheet

| Technology | SNG from Biogas |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2030 | 2050 | Uncertainty (2020) |  | Uncertainty (2050) |  | Note | Ref |
|  |  |  |  |  | Lower | Upper | Lower | Upper |  |  |
| Energy/technical data |  |  |  |  |  |  |  |  |  |  |
| Typical total plant size 1,000 GJ SNG/year | 70 | 100 | 250 | 700 | 75% | 125% | 75% | 125% | A, B | 1, 7 |
| Typical total plant size, MW | 2.5 | 3.5 | 9 | 24 | 75% | 125% | 75% | 125% | A, B | 1, 7 |
| Inputs |  |  |  |  |  |  |  |  |  |  |
| Biogas Consumption, MWh/MWh Total Input | 0.53 | 0.60 | 0.60 | 0.60 | 80% | 120% | 80% | 120% | C, I | 2 |
| Hydrogen Consumption, MWh/MWh Total Input | 0.46 | 0.51 | 0.51 | 0.51 | 80% | 120% | 80% | 120% | C, E | 2 |
| Electricity Consumption, MWh/MWh Total Input | 0.01 | 0.009 | 0.009 | 0.009 | 50% | 150% | 50% | 150% | D | 2 |
| Outputs |  |  |  |  |  |  |  |  |  |  |
| SNG Output, MWh/MWh Total Input | 0.89 | 0.89 | 0.89 | 0.89 | 90% | 110% | 95% | 105% | G | 2 |
| Co-products Output, MWh/MWh Total Input | 0.10 | 0.10 | 0.10 | 0.10 | 50% | 150% | 50% | 150% | G | 2 |
| Forced outage (%) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | F |  |
| Planned outage (weeks per year) | 2 |  |  |  |  |  |  |  |  |  |
| Technical lifetime (years) | 25 |  |  |  |  |  |  |  |  |  |
| Construction time (years) | 1.0 |  |  |  |  |  |  |  |  |  |
| Financial data |  |  |  |  |  |  |  |  |  |  |
| Specific investment (€1,000 / annual GJ SNG) | 36 | 30 | 25 | 15 | 50% | 150% | 50% | 150% | H | 1 |
| - equipment (%) | 75 | 75 | 75 | 75 |  |  |  |  |  |  |
| -installation (%) | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| Fixed O&M (€ /GJ SNG) | 0.18 | 0.16 | 0.12 | 0.08 | 90% | 110% | 90% | 110% |  | 1 |
| Variable O&M (€ /GJ SNG) | 0.18 | 0.16 | 0.12 | 0.08 | 90% | 110% | 90% | 110% |  | 1 |
| Start up (€ /GJ SNG) | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| Technology specific data |  |  |  |  |  |  |  |  |  |  |

Notes:

- The plant size range is likely limited by the biogas availability.

- Plant size progression is based on moving from small biogas system to large systems as the technology is proven.

- There are variations in the process that will have some impact on the overall efficiency. It is not yet clear what might become the industry standard.

- This is the power for the system excluding the electrolyzer.

- The hydrogen source is not specified.

- The methanation step does not lend itself to intermittent operation. The concept of intermittent power to SNG is not practical without intermediate storage for hydrogen that would allow the methanation plant to operate continuously. This would have a significant capital cost impact.

- Some recoverable waste heat is available from the process.

- There is a significant range in the capital cost estimates.

- Assumes that the biogas is 65% methane.

References

- Manuel Götz, Jonathan Lefebvre, Friedemann Mörs, Amy McDaniel Koch, Frank Graf, Siegfried Bajohr, Rainer Reimert, Thomas Kolb, Renewable Power-to-Gas: A technological and economic review, In Renewable Energy, Volume 85, 2016, Pages 1371-1390, ISSN 0960-1481, https://doi.org/10.1016/j.renene.2015.07.066

- Swiss Federal Office of Energy SFOE. 2017. Direct Methanation of Biogas. https://www.aramis.admin.ch/Default.aspx?DocumentID=45656&Load=true

- gwf gas + energy. 2014. Das e-gas-Projekt am Biogasanlagen-standort in Werlte. https://www.di-verlag.de/media/content/gwf-GE/gwf_Gas_5_14/gwf-GE_05_2014_FB_Kurt.pdf?xaf26a=7841c984ef837209544e

- Rauch, R., Hrbek, J. and Hofbauer, H. 2014. Biomass gasification for synthesis gas production and applications of the syngas. WIREs Energy Environ, 3: 343–362. https://doi.org/10.1002/wene.97

- Combination of Power-to-Gas with Biogas Plant. Vienna University of Technology. https://www.tuwien.ac.at/fileadmin/t/tuwien/fotos/pa/download/2015/HM2015/Flyer_Power_to_Bio--Gas_EN.pdf

- Hitachi Zosen Corporation and Hitachi Zosen Inova to Build First Joint Power-To-Gas Plant. Dec 2017. http://www.hz-inova.com/cms/en/home?p=6276

- Hansen, J., Holstebroe, M., Jensen, M., Rass-Hansen, J., Heiredal-Clausen, T. 2016. SOEC Enabled Biogas Upgrading. 12th European SOFC & SOE Forum.

- Udvikling og effektivisering af biogasproduktionen I Danmark – Delrapport 1 og 2, Danish Energy Agency Biogas Taskforce 2015.

Hydrothermal Liquefaction

  - Qualitative Description

Hydrothermal liquefaction of biomass is the thermochemical conversion of biomass into liquid fuels by processing in a hot, pressurized water environment for sufficient time to break down the solid biomass structure to mainly liquid components. Typical hydrothermal processing conditions are 250–420 °C of temperature and operating pressures from 40 to 350 bar of pressure.

Alkali catalysts are often used to facilitate the degradation of macromolecules by hydrolysis, decarboxylation, and depolymerisation type of reactions, as well as inhibit formation of tar, char, and coke.

The process can handle most types of biomass and unlike some other thermochemical process it does not require dry biomass.

Contact Information

Don O’Connor – (S&T)2 Consultants Inc.

Brief Technology Description

A biomass slurry is fed into reactors at high pressure and high temperature. The slurry contains water, biomass and the alkali catalysts. The reactors provide sufficient residence time for the solid biomass to breakdown. After leaving the reactor the product stream is degassed and the liquid is separated into an aqueous phase and an oil phase. The aqueous phase is recirculated through the reactors and oil is available for market or further processing. The gas is used internally in the process to provide the thermal energy. The basic process flow is shown in the following figure.

Figure 13-1	Hydrothermal Liquefaction Process Flow

Input

The process inputs are biomass, water, and an alkali catalyst. Some electricity is required to operate the process.

Output

The primary output from the system is the bio-oil and water that is recycled through the process. There is some gas produced in the reactors but this is consumed in the process to provide the process heat to maintain the reactor operating temperature.

Energy Balance

The energy balance of the process is shown in the following figure [4]. This energy balance is based on laboratory scale operations and could change with feedstock and operating conditions. A similar energy balance but without the electrical input was reported by Jensen [5].

Figure 13-2	Hydrothermal Liquefaction Energy Balance

Typical Capacities

Steeper Energy, one of the companies developing the technology is proposing that its first commercial plants will produce 2,000 bbls or oil per day (140 MW) and consume 240,000 odt of feedstock per year [6].

Zhu [7] undertook a techno-economic analysis of an HTL plant and upgrader that processed 2,000 odt per day and produced 3,780 to 4,900 bbls of oil per day. The low end of the production range represents the state of the art according to Zhu and the high end of the range, the ultimate design goal.

Regulation Ability

There is little published on the performance of continuously operated plants. Given the high pressure and temperatures required in the reactors and the required reactor residence time it is likely that the performance will be altered when the process is operated at rates below the design capacity.

Space Requirements

The space requirements are likely to be dominated by the area required for feedstock storage. None of the techno economic analyses found in the literature report the area required for the plants. There have been no large scale, similar technology plants built. The best estimate is that the area required would be similar to pulp mills that have the same feedstock inputs.

Advantages/Disadvantages

The primary attractiveness of the technology is that biomass can be converted into a liquid fuel. The HTL process produces a liquid with a low oxygen content compared to pyrolysis processes and may be suitable for use as a fuel that can substitute for heavy fuel oil in applications such as the marine sector. The low oxygen content will also make the fuel easier to hydro treat to remove the oxygen and upgrade the fuel so that it is suitable for more demanding applications such as the use in transport diesel applications. The product could also be used as a bio crude oil for use in existing petroleum refineries to produce bio-gasoline and diesel blending components.

Environment

The GHG emission performance of the product is expected to be very good due to the limited fossil fuel inputs into the system and the high oil yield. The high oil yield is also attractive where biomass availability is limited. The low oxygen content of the product will also mean that if it is upgraded through hydrotreating, the hydrogen requirements will be modest and the GHG emission profile will still be good.

Research and Development Perspective

Hydrothermal liquefaction of biomass to produce a bio-oil is a category 1 technology, a technology in the research, development and deployment stage. There is significant uncertainty with respect to the performance and costs of the technology.

There is potential to improve yields and reduce costs as more experience with the technology is gained from a demonstration facility and then the technology is scaled to commercial plants.

Examples of Market Standard Technology

Since this is a Category 1 technology there are no market standard technologies. One of the leading development companies with this technology is the Danish-Canadian company Steeper Energy. They have recently announced plans [1] for an industrial scale demonstration plant at a former pulp mill located in Tofte, Norway with their partner Silva Green Fuel, a Norwegian-Swedish joint venture.

Steeper Energy

Sandbjergvej 11
DK 2970 Hørsholm
Denmark

http://steeperenergy.com/

The other major technology supplier is the Australian company, Licella. They have form a joint venture with the Canadian pulp and paper company Canfor and announced plans to build a 400,000 bbl/day facility in Prince George, BC, Canada [8].

Licella Pty Ltd

Level 7, 140 Arthur Street
North Sydney NSW 2060 Australia

http://www.licella.com.au/contact/

Predication of Performance and Cost

Given the early stage of development predictions of performance and cost are uncertain. The announcement of an industrial scale demonstration facility is a significant step in the development of the technology.

There is some discussion in the literature [7] of the current state of the art with respect to performance and capital and operating costs. These estimates have been developed by independent third parties and not by process developers.

Uncertainty

There is significant uncertainty with respect to performance and economics due to the stage of the development of the technology.

Additional Remarks

There is significant interest in this pathway from petroleum refiners. Due to the low oxygen content of the product the refiners there is the potential for the product to be co-processed in existing refineries with minimal modifications.

HTL oil is very viscous which will limit the potential applications of the oil to a replacement for other heavy viscous oil in applications like boilers. The physical properties of HTL can be improved by hydrotreating the oil.

Jensen et al [9] have reported that about 2% wt. hydrogen is consumed when the oil is hydrotreated. Hydrotreating yields were about 80% on a mass basis, and over 90% on an energy basis. Hydrotreating also produced 9% water and 5-6% gas. The properties of the HTL and the hydrotreated HTL are shown in the following table. The hydrotreated HTL properties will vary with the severity of the hydrotreating.

Table 13-1 	Hydrotreated HTL Properties

|  | HTL | Hydrotreated HTL |
| --- | --- | --- |
| Density (kg/m3) | 1103 | 989 |
| Viscosity at 20 °C (cP) | 80,432 | 297 |
| HHV (MJ/kg) | 37.2 | 42.1 |
| Carbon (wt. %) | 80.6 | 88.1 |
| Hydrogen (wt. %) | 9.1 | 11.9 |
| Oxygen (wt. %) | 10.1 | 0.0 |
| Nitrogen (ppm) | 1500 | 1175 |
| Sulphur (ppm) | 309 | 389 |
| TAN (mg/g) | 55.7 | 0.0 |

Quantitative Description

The reaction chemistry of hydrothermal liquefaction is complex, and many different chemical reactions may proceed depending of the specific operating conditions. Jensen et al [2] have proposed a number of the major chemical reactions involved in the process and these are shown in the following figure.

Figure 13-3	Major Chemical Reactions in the Steeper Energy Process

Energy/Technical Data

Some quantitative information on the process is presented below.

Typical Plant Size

The technology has not yet moved beyond the pilot plant stage. Steeper and Licella are planning commercial plants in the range of 1,000 to 2,000 bbls per day. Techno-economic assessments [7] have considered plants of 5,000 bbls/day as long term design objective. These different plant sizes are shown in the following table with different approximate equivalent metrics.

Table 13-2	Typical Plant Sized

| Volume Based, bbl/day | Input Mass Based, tpy | Energy Output Based, GJ/day |
| --- | --- | --- |
| 1,000 | 120,000 | 6,100 |
| 2,000 | 240,000 | 12,200 |
| 5,000 | 600,000 | 30,500 |

Input and Output

The input and output for a 1,000 bbl/day plant has been extrapolated from Steeper published information [4] and summarized in the following table. In addition to the parameters shown in the table there would 420 GJ of produced gas that is utilized in the process.

Table 13-3	Typical Input and Output

|  | Input | Output |
| --- | --- | --- |
| Wood, tonnes/day | 350 |  |
| Power, kWh | 93,325 |  |
| Bio-oil, bbls/day |  | 1,000 Bbls |

Forced and Planned Outage

Harris Group prepared a report on HTL reactor design for the National Renewable Energy Laboratory (NREL) and used a 90% on stream design factor. That allows 36 days per year for system maintenance.

The combination of a solid feedstock and the high pressures involved have been problematic for systems like pressurized biomass gasification systems. Harris reported that primary challenges associated with the reactor section design were (1) maximizing heat integration, (2) managing the potential for poor heat transfer from the reactor effluent to the reactor feed due to the potential for high viscosities in the feed streams, and (3) minimizing cost associated with the reactor system itself, given the very high required pressures. It is possible that the first large scale demonstration plants will experience some forced outages as these challenges are addressed.

Technical Lifetime

Plants of this type would normally be designed for at least a 20 year lifetime.

Construction Time

Construction periods of 2 years are likely due in part to the fabrication time of the specialty high pressure reactors involved in the process.

Financial Data

Several sources of detailed information [7] [10] [11] [12] on capital costs and operating costs are in the public domain. These have mostly been generated by independent third parties and not by the leading process developers. However, Pedersen et [12] are involved with Steeper Energy so the information from this paper has been used in the data sheet. The information from of the reports is discussed below.

Investment Costs

Zhu [7] and Pedersen [12] provided information for both the production of HTL and for upgrading the HTL to a pure hydrocarbon... The capital costs are compared in the following table. The Harris Group [10] investigated five different designs with widely varying capital costs due to trying to address the design challenges identified above.

Table 13-4	Capital Cost Comparison

| Parameters | Zhu | Harris | Pedersen |
| --- | --- | --- | --- |
| Base Year for Costs | 2007 | 2011 | 2016 |
| Feedstock, tpd | 2000 | 2000 | 500 |
| Total Capital Investment | $512 Million | $222 to $1,646 million | $225 |
| Best case |  | $364 million |  |

Zhu’s cost estimate was prepared using Aspen Process Economic Analyzer includes an allowance for potential missing equipment in the design. The Harris estimate is an engineering estimate based on vendor quotes for the equipment. Both use factors for the installation costs and indirect construction costs.

Using the Pedersen estimate and an output of 1150 bbl/day the capital costs are $3.40/annual litre (€3.5/annual litre for a plant in Europe).

Operating and Maintenance Costs

The operating and maintenance costs were estimated in the studies. The results are compared in the following table. The Zhu report does not separate the operating costs for the upgrading from the HTL production.

Table 13-5	Operating and Maintenance Costs

| Parameters | Zhu | Harris |
| --- | --- | --- |
| Feedstock | $46 million | Not reported |
| Waste disposal | $25 million | Not reported |
| Utilities | $8 million | $22 million |
| Fixed costs | $24 million | $10 million |

The operating and maintenance costs from the two reports have significantly different profiles. Pedersen provided a more detailed breakdown of the variable operating costs. The data was presented per litre of gasoline equivalent (LGE). We have converted that to a per litre of HTL basis by assuming that 1 LGE is equal to 0.90 litres of HTL. The information is shown in the following table.

Table 13-6	Operating and Maintenance Costs

| Parameter | US$ per Litre HTL |
| --- | --- |
| Feedstock | 0.13 |
| Hydrogen | 0.11 |
| Thermal energy | 0.24 |
| Power | 0.05 |
| Wood Grinding | 0.058 |
| Water disposal | 0.014 |
| Fixed O&M | 0.10 |

The variable O&M costs less feedstock, power and thermal energy is 0.18 US$/litre (0.15 €/litre) and the fixed O&M is 0.10 US$/litre (0.08 €/litre).

Start-up Costs

The start-up costs are included in the operating cost estimates.

Technology Specific Data

The properties of the Hydrofaction™ oil produced by the Steeper Energy process are shown in the following table [2] and compared to typical values for heavy fuel oil [3]. The bio-oil has some oxygen which reduces the energy content and increases the density but the properties are broadly similar. The bio-oil viscosity is very sensitive to the temperature and is more viscous that the petroleum HFO at the same temperature.

Table 13-7	HTL Bio-Oil Properties

| Parameter | Hydrofaction™ Oil | Heavy Fuel Oil |
| --- | --- | --- |
| Energy Content, MJ/kg (LHV) | 36.7 | 39 |
| Water Content, wt. % | 0.8 | 0.1 |
| Density, kg/m3 | 1.05 | 0.98 |
| Oxygen Content, wt. % | 9.8 | 0 |
| Pour Point, C | 24 | 15 |
| Flash Point, C | 29 | 100 |
| Kinematic viscosity at 40 °C, mm2/s | 17,360 |  |
| Kinematic viscosity at 50 °C, mm2/s |  | 200-600 |
| Kinematic viscosity at 60 °C, mm2/s | 1,545 |  |

Summary

The information on HTL production is summarized in the following table. Since there are no operating plants in 2015, the data is presented for the years 2020 to 2050.

Table 13-8	Hydrothermal Liquefaction Datasheet

| Technology | Bio Oil |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2020 | 2030 | 2050 | Uncertainty (2020) |  | Uncertainty (2050) |  | Note | Ref |
|  |  |  |  |  | Lower | Upper | Lower | Upper |  |  |
| Energy/technical data |  |  |  |  |  |  |  |  |  |  |
| Typical total plant size 1,000 t Bio Oil/year | n.a. | 120 | 180 | 300 | 50% | 125% | 75% | 125% | A, B | 1, 6 |
| Typical total plant size. MW | n.a. | 145 | 220 | 360 | 50% | 125% | 75% | 125% | A, B | 1, 6 |
| Inputs |  |  |  |  |  |  |  |  |  |  |
| Feedstock Consumption, MWh/MWh Total Input | n.a. | 0.95 | 0.95 | 0.95 | 90% | 150% | 90% | 125% | E | 4 |
| Electricity Consumption, MWh/MWh Total Input | n.a. | 0.05 | 0.05 | 0.05 | 75% | 125% | 75% | 125% | E | 4 |
| Outputs |  |  |  |  |  |  |  |  |  |  |
| Bio Oil Output, MWh/MWh Total Input | n.a. | 0.82 | 0.82 | 0.82 | 90% | 110% | 90% | 110% | E | 4 |
| Co-products Output MWh/MWh Total Input | n.a. | 00.0 | 00.0 | 0.0 | 90% | 110% | 90% | 110% | E | 4 |
| Forced outage (%) | n.a. | 0 | 0 | 0 |  |  |  |  |  |  |
| Planned outage (weeks per year) | 4 |  |  |  |  |  |  |  | C | 8 |
| Technical lifetime (years) | 20+ |  |  |  |  |  |  |  |  |  |
| Construction time (years) | 2.0 |  |  |  |  |  |  |  |  |  |
| Financial data |  |  |  |  |  |  |  |  |  |  |
| Specific investment (€ /l Bio Oil) | n.a. | 2.80 | 2.10 | 1.40 | 75% | 125% | 75% | 125% | D | 6, 8 |
| - equipment (%) | n.a. | 75 | 75 | 75 |  |  |  |  |  |  |
| -installation (%) | n.a. n.a. | 25 | 25 | 25 |  |  |  |  |  |  |
| Fixed O&M (€ /l Bio Oil) | n.a. | 0.08 | 0.08 | 0.08 | 75% | 125% | 75% | 125% | F | 6 |
| Variable O&M (€ /l Bio Oil) | n.a. | 0.15 | 0.15 | 0.15 | 75% | 125% | 75% | 125% | F | 6 |
| Start up (€ /l Bio Oil) | n.a. | 0 | 0 | 0 |  |  |  |  |  |  |
| Technology specific data |  |  |  |  |  |  |  |  |  |  |

Notes:

- The plant size range is likely limited by the feedstock availability.

- The plant size range is typical of the range found in the literature. It is highly uncertain given that there is no commercial demonstration plant in operation yet.

- This is twice the time normally found in process operations.

- Capital costs for nth plant are used for 2050 and earlier costs are estimates.

- These are based on extrapolated pilot plant results.

- Nth plant estimates

References

- Steeper Energy Announces Eur 50.6 M (Dkk 377 M) Advanced Biofuel Project With Norwegian-Swedish Joint Venture Silva Green Fuel In Licensing Deal. 2017. http://steeperenergy.com/2017/12/15/steeper-energy-announces-eur-50-6-m-dkk-377-m-advanced-biofuel-project-with-norwegian-swedish-joint-venture-silva-green-fuel-in-licensing-deal/

- Jensen, C.U., Rodriguez Guerrero, J.K., Karatzos, S. et al. 2017. Fundamentals of Hydrofaction™: Renewable crude oil from woody biomass. Biomass Conv. Bioref. (2017) 7: 495. https://doi.org/10.1007/s13399-017-0248-8

- Environment Canada. 1999. Bunker C Fuel Oil. www.etc-cte.ec.gc.ca/databases/oilproperties/pdf/web_bunker_c_fuel_oil.pdf

- Fracking Biomass. Biofuels Digest. Feb 2, 2017. http://www.biofuelsdigest.com/bdigest/2017/02/02/fracking-biomass-steeper-energy-and-the-pursuit-of-renewable-hydrocarbons/

- Jensen, C.U., Rodriguez Guerrero, J.K., Karatzos, S. et al. Biomass Conv. Bioref. (2017) 7: 495. https://doi.org/10.1007/s13399-017-0248-8

- Toms, P. 2017. Hydrofaction™ Oil: A Solution for Decarbonizing Long-haul Transport. https://www.bio.org/sites/default/files/0830AM-Perry%20Toms.pdf

- Zhu, Y., Biddy, M., Jones, S., Elliott, D., Schmidt, A. 2014. Techno-economic analysis of liquid fuel production from woody biomass via hydrothermal liquefaction (HTL) and upgrading, In Applied Energy, Volume 129, 2014, Pages 384-394, ISSN 0306-2619, https://doi.org/10.1016/j.apenergy.2014.03.053

- Licella. 2016. Licella Canfor Joint Venture at the forefront of global bioenergy. http://www.licella.com.au/news/licella-canfor-joint-venture-at-the-forefront-of-global-bioenergy/

- C.U. Jensen, J.K.R. Guerrero, S. Karatzos, G. Olofsson and S.B. Iversen, 10 - Hydrofaction™ of forestry residues to drop-in renewable transportation fuels, In Direct Thermochemical Liquefaction for Energy Applications, edited by Lasse Rosendahl,, Woodhead Publishing, 2018, Pages 319-345, ISBN 9780081010297, https://doi.org/10.1016/B978-0-08-101029-7.00009-6.

- Harris Group. 2013. Production of Advanced Biofuels via Liquefaction Hydrothermal Liquefaction Reactor Design. NREL/SR-5100-60642. http://www.ourenergypolicy.org/wp-content/uploads/2015/08/60462.pdf

- Magdeldin, M., Kohl, T., Järvinen, M. 2018. Techno-economic Assessment of Integrated Hydrothermal Liquefaction and Combined Heat and Power Production from Lignocellulose Residues, J. sustain. dev. energy water environ. syst., 6(1), pp 89-113, 2018. https://doi.org/10.13044/j.sdewes.d5.0177

- Pedersen, T. H., Hansen, N. H., Pérez, O. M., Cabezas, D. E. and Rosendahl, L. A. 2018.  Renewable hydrocarbon fuels from hydrothermal liquefaction: A techno‐economic analysis. Biofuels, Bioprod. Bioref., 12: 213-223. doi:10.1002/bbb.1831

Lower cloud points sacrifice some product yield.